| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0083_r1 |
| --- | --- |

| Title: | RCE2: Experimental results for Test D.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi Joel Sole Marta Karczewicz Matteo Naccari Marta Mrak | Tel: Email: Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com +44 (0)303 0409640 matteo.naccari@bbc.co.uk |
| Source: | Qualcomm Inc and BBC – R&D |  |  |

_____________________________

# Abstract

This document reports the experimental results for Test D.1 of HEVC Range Extensions core experiment 2. This is a combination of methods described in Tests C.1 and C.2 from document JCTVC-M1122. These two tests propose to extend residual DPCM along the horizontal and vertical directions for lossy coding over transform units for which transform is skipped. Test C.1 applies RDPCM for inter coded transform units with explicit signalling of the direction (including no residual DPCM). Test C.2 applies residual DPCM to intra coded transform units with horizontal or vertical intra prediction mode. It is reported that in lossy configuration for screen content test conditions, the combination shows BD-rate gains of up to 6.0%, 4.3% and 4.7% for all-Intra, random access and low delay configurations, respectively, for RGB 4:4:4 screen content sequences.

# Introduction

At the 13th JCTVC meeting in Incheon the Sample Adaptive Prediction (SAP) along the Horizontal and Vertical directions (SAP H/V) proposed in JCTVC-M0056 has been adopted for lossless coding of intra predicted Transform Units (TUs). SAP H/V is conceptually identical to RDPCM proposed in JCTVC-M0056 always for intra TUs. Moreover, document JCTVC-M0442 extended RDPCM for inter blocks in lossless coding.

This document reports the experimental results for Test D.1 of range extensions core experiment 2 (RCE2). This is a combination of methods described in Tests C.1 and C.2 from document JCTVC-M1122. These two tests propose to extend residual DPCM (RDPCM) along the horizontal and vertical directions for lossy coding over transform units (TUs) for which transform is skipped. Test C.1 (JCTVC-N0074) is an extension of inter RDPCM proposed in JCTVC-M0442 to lossy coding. Inter RDPCM is applied to inter coded TUs with explicit signalling of the direction (including no RDPCM). Test C.2 (JCTVC-N0052) applies RDPCM to lossy intra coded transform-skip TUs with horizontal or vertical intra prediction mode. This was first proposed in JCTVC-M0351

# Lossy residual DPCM

When lossy coding is used, RDPCM is applied at TU level and only for those TUs which are transform skipped (i.e. 4×4 transform units). For intra TUs, RDPCM is always applied for intra coded transform-skip TUs with horizontal or vertical intra prediction mode, so there is no additional signalling. For inter TUs, the sum of absolute difference (SAD) is computed over the after performing residual DPCM. The SADs for horizontal and vertical RDPCM quantized residuals are compared with the SAD of the residual quantised without applying any RDPCM. The mode which minimises the SAD is selected as the RDPCM mode and signalled to the decoder. The signalling is done using two binary flags: one to signal whether RDPCM is applied and one to signal the direction in which RDPCM is applied.

# Experimental results

The combination of Tests C.1 and C.2 has been implemented in the HM10.1-rext-3.0 software and tested over the test material and coding conditions specified for RCE2 (document JCTVC-M1122). The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. This section reports the obtained BD-rates.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.3% | -2.8% | -2.9% | -2.4% | -2.9% | -3.0% | -2.4% | -2.9% | -3.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -5.9% | -5.8% | -5.8% | -6.0% | -6.0% | -5.8% | -5.5% | -5.4% | -5.1% |
| SC YUV 444 | -4.3% | -4.9% | -4.8% | -4.6% | -5.3% | -5.4% | -4.5% | -5.1% | -5.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 102% |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  | 93% |  |  |

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -1.4% | -1.7% | -1.8% | -1.3% | -1.6% | -1.7% |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| SC RGB 444 | -4.3% | -4.4% | -4.4% | -4.3% | -4.4% | -4.4% |
| SC YUV 444 | -2.9% | -3.5% | -3.5% | -2.9% | -3.7% | -3.8% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  |

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -1.0% | -0.8% | -1.0% | -0.7% | -0.9% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
| SC RGB 444 | -4.7% | -4.8% | -4.9% | -4.4% | -4.5% | -4.3% |
| SC YUV 444 | -2.5% | -2.8% | -2.8% | -2.5% | -2.9% | -3.0% |
| RangeExt | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 106% |  |  | 107% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

## Combination with subtest C.4

Table 2 shows the BD-rate results when RDPCM is combined with the replacement DC intra prediction mode as tested in subtest C.4 (JCTVC-N0053) for lossy configuration. As seen from the table additional gains are obtained when lossy RDPCM (intra and inter) is combined with subtest C.4 (replacement of DC intra prediction mode).

# Text changes

The required text changes with respect to the current text specification for HEVC-RExt (JCTVC-M1005-v2) are included in a separate document attached to this contribution.

# Complexity analysis

Complexity analysis of the proposed tools is provided separately in the corresponding documents, i.e. JCTVC-N0074 for Test C.1 and JCTVC-N0052 for Test C.2

# Patent rights declaration(s)

Qualcomm Inc. and the British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).