| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0084 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0056 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On various cross-layer alignments |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Adarsh K. Ramasubramonian Jianle Chen Hendry 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8345 yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com cjianle@qti.qualcomm.com fhendry@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes to require cross-layer alignment of leading pictures, TSA/STSA pictures, IRAP picture types, and "GOP structures".

# On cross-layer alignment of leading pictures

Each leading picture (either RASL picture or RADL picture) is associated with an IRAP picture. All leading pictures are discardable without affecting the decoding of trailing pictures, and RASL pictures must be discarded when random accessing from the associated IRAP picture.

As mix of leading pictures and other types of pictures within an AU is not useful but increases at least costs for bitstream conformance tests, we propose to add the following restriction that requires leading pictures for all IRAP pictures in the same AU are to be cross-layer-aligned:

For any two IRAP pictures picA and picB in an AU, let layerA and layerB be the two layers containing picA and picB, respectively, when there exists a picture picC that is in layerA and is a leading picture of picA and there exists a picD that is in layerB and is in the same AU as picC, picD shall be a leading picture of picB.

# On cross-layer alignment of TSA and STSA pictures

TSA and STSA pictures enable temporal sub-layer up switching. Similarly for temporal sub-layer up switching capability to be valid across layers, we propose to require cross-layer alignment of TSA and STSA pictures, as follows:

- When one picture in an access unit has nal_unit_type equal to TSA_N or TSA_R, any other picture in the same access unit shall have nal_unit_type equal to TSA_N or TSA_R.

- When one picture in an access unit has nal_unit_type equal to STSA_N or STSA_R, any other picture in the same access unit shall have nal_unit_type equal to STSA_N or STSA_R.

One may ask why not to more tightly restrict the NAL unit types as follows:

- When one picture in an access unit has nal_unit_type nutA equal to TSA_N, TSA_R, STSA_N, or STSA_R, any other picture in the same access unit shall have nal_unit_type equal to nutA.

This is because the tighter constraint would disallow efficient use of sub-layer non-reference pictures at lower layers when higher layers have higher picture rates.

# On cross-layer alignment of IRAP picture types

We propose the following constraints:

- When one IRAP picture in an access unit has nal_unit_type equal to IDR_N_LP, any other IRAP picture in the same access unit shall have nal_unit_type equal to IDR_N_LP.

- When one IRAP picture in an access unit has nal_unit_type equal to IDR_W_RADL any other IRAP picture in the same access unit shall have nal_unit_type equal to IDR_W_RADL.

- When one IRAP picture in an access unit has nal_unit_type equal to BLA_N_LP and has nuh_layer_id equal to layerId, any other IRAP picture in the same access unit that has nuh_layer_id less than layerId shall have nal_unit_type equal to BLA_N_LP or CRA_NUT.

- When one IRAP picture in an access unit has nal_unit_type equal to BLA_W_LP or BLA_W_RADL and has nuh_layer_id equal to layerId, any other IRAP picture in the same access unit that has nuh_layer_id less than layerId shall have nal_unit_type equal to BLA_W_LP, BLA_W_RADL, or CRA_NUT.

Note: When one IRAP picture in an access unit has nal_unit_type equal to CRA_NUT, any other IRAP picture in the same access unit must have nal_unit_type equal to CRA_NUT, BLA_W_LP, BLA_W_RADL, or BLA_N_LP.

Some reasons for the above proposed constraints:

- RADL and RASL may be non-cross-aligned, and thus NAL unit types of the same type of IRAP pictures in an AU should be allowed to be different according to leading pictures associated with them.

- There seems to be no reason to mix different types of IRAP pictures in the same AU. For example, in a two layer bitstream, there is no point to allow a base layer picture to be an IDR picture and the enhancement layer picture in the same AU to be a CRA picture while it at least increases costs for decoder conformance tests.

# On cross-layer alignment of "GOP structures"

A group of pictures (GOP) structure may be used to refer to temporal prediction structures, e.g. hierarchical B coding structure. Herein each GOP consists of one key picture and a number of associated non-key pictures. The non-key pictures follow the key picture in decoding order but precede the key picture in output order, similarly as an IRAP picture and its associated leading pictures. Actually, an IRAP picture and its associated leading pictures are one example of a GOP consisting of a key picture and its associated non-key pictures.

The currently AU definition implicitly requires cross-layer alignment of key pictures and non-key pictures if each AU includes a picture for each layer, but not otherwise. For example, the AU definition does not guarantee cross-layer key picture alignment when different layers have different picture rates. For example, the following cases as shown in Figures 1 and 2 are allowed by the AU definition but would be disallowed if cross-layer key picture alignment is required.

Figure 1: Arrows connecting pictures depicts the decoding order of the pictures within each layer.

Figure 2: Example of bitstream when key pictures are not aligned.

In our understanding, such reordering of pictures only adds delay and conformance testing costs and does not bring any benefit.

Therefore, we propose to require cross-layer alignment of key pictures, as follows:

- When a picture of one layer in an AU is a key picture, all pictures of other layers in the same AU shall be key pictures.

We also propose adding the following definitions of key picture and non-key picture:

- key picture: a picture for which there is no other picture in the same layer that precedes the picture in decoding order and follows the picture in output order.

- non-key picture: a picture that follows another picture in the same layer in decoding order and precedes the another picture in output order.

Note that a key picture cannot use any other picture later in output order for inter prediction reference, and the relative output order between any two key pictures of a layer is the same as the relative decoding order. Cross-layer alignment of key pictures implies cross-layer alignment of non-key pictures.

With the above, an AU containing key pictures may be referred to as a key AU, and an AU not containing key pictures may be referred to as non-key AU. IRAP pictures are by definition all key pictures.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).