| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0088 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0094 |
| --- | --- |

| Title: | Extension of (temporal) motion constrained tile sets SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | karsten.suehring@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de yago.sanchez@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a modification to the motion constrained tile sets SEI message to keep it aligned with the inter-layer constrained tile sets SEI message that is proposed in JCTVC-N0087. We also suggest adding the term "temporal" to the name of the SEI message for better distinction.

# Introduction

Although the adopted syntax of the SEI message allows signalling that each tile can be decoded independently of other tiles of previously decode pictures by describing an own set per each tile, we think that this special case requires an unnecessary high number of syntax elements. Especially for inter-layer independence we think that this might even be the most common case. Thus we added a shortcut for signalling independence of all tiles to the newly proposed inter-layer constrained tiles sets SEI message. For keeping the syntax of both SEI messages aligned, we propose to add the shortcut also for the (temporal) motion constrained tile sets SEI message. If the inter-layer constrained tiles sets SEI message is adopted, we propose to add the term "temporal" to the SEI name to allow better distinction.

# Proposed changes

| temporal_motion _constrained_tile_sets ( payloadSize ) { | Descriptor |
| --- | --- |
| all_tiles_independence_flag | u(1) |
| if (all_tiles_independend_flag) |  |
| exact_sample_value_match_flag | u(1) |
| else { |  |
| num_sets_in_message_minus1 | ue(v) |
| for( i = 0; i <= num_sets_in_message_minus1; i++) { |  |
| mcts_id[ i ] | ue(v) |
| num_tile_rects_in_set_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| top_left_tile_index[ i ][ j ] | ue(v) |
| bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| exact_sample_value_match_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

all_tiles_independence_flag equal to 1 indicates that each tile forms an own motion-constrained tile set. If all_tiles_independence_flag is equal to 1, the value of exact_sample_value_flag applies to all motion constrained tile sets.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).