| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0089 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0083 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On signaling of scaled reference offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen, Adarsh Krishnan Ramasubramonian Ye-Kui Wang Ying Chen Xiang Li 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com aramasub@qti.qualcomm.com yekuiw@qti.qualcomm.com cheny@qti.qualcomm.com lxiang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes moving the scaled reference offsets syntax from SPS extension to VPS extension. In addition, similar to SVC, the scaled reference offsets can be further signaled in the slice header to have picture-level adaptation.

# Signaling of scaled reference offset in SHVC

In JCTVC-NN0092/JCT3V-E0060, it is proposed to put the spatial resolution, bit depth, and color format, which are currently signalled in the SPS, in the VPS. The VPS is envisioned to be used for session negotiation for a multi-layer video communication session, and hence the VPS is expected to contain all the necessary information for all the layers required for negotiation. Information, such as spatial resolution, is important to enable the client to determine which operating point can be supported at the decoder, and therefore is desirable to be put in the VPS.

The scaled reference offsets, which are currently signaled in the SPS, are meaningful together with the spatial resolutions of both a current layer and a reference layer. When the VPS contains the information of the spatial resolutions of the enhancement layers, it is straightforward to have the scaled reference offsets in the VPS too, as the scaled reference offsets are cross-layer information. One clear benefit is that when a new SPS instance does not need to be created for an enhancement layer (with a different resolution as the base layer) due to the reason that the only different characteristics of these two layers is spatial resolution (therefore the SPS for the base layer can be used by the enhancement layer), no SPS needs to be created just because of the requirement of signaling the scaled reference offsets in the SPS. In other words, sharing of SPS becomes more likely when the scaled reference offsets are signalled in the VPS. In addition, the scaled reference offsets are used to calculate scaling factor between layers which might also be useful for session negotiation when they are signalled in VPS.

In some applications, it is desirable to update the spatial resolution without having to signal a new VPS. Therefore, in JCTVC-NN0092/JCT3V-E0060, it was proposed to allow the spatial resolution formation for specific layers to be updated in SPS. When the spatial resolution of a layer is changed, the scaled reference offsets of this layer and the higher layers that refer to this layer may also need update. To handle this case, this contribution proposes to allow update of scaled reference offset information in slice header, as done in SVC. Signaling the information in slice header also provides the functionality of picture-level adaptation of the reference region in the reference layer, which is useful for the user case that the position of region of interest (coded in base layer or enhancement layer) is changeable.

# Proposal

This contribution proposes to move the scaled reference offsets syntax from SPS extension to VPS extension provided that the spatial resolution is also present in VPS e.g. as proposed in JCTVC-NN0092/JCT3V-E0060.

Slice-level signaling of these offsets is also proposed, similar as in SVC.

The proposed changes to the SHVC specicification are highlighted in the following section.

## Video parameter set syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| vps_ref_layer_offsets_present_flag | u(1) |
| slice_ref_layer_offsets_enable_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) { |  |
| direct_dependency_type[ i ][ j ] | u(1) |
| if( vps_ref_layer_offsets_present_flag ) { |  |
| vps_scaled_ref_layer_left_offset[ i ][ j ] | se(v) |
| vps_scaled_ref_layer_top_offset[ i ][ j ] | se(v) |
| vps_scaled_ref_layer_right_offset[ i ][ j ] | se(v) |
| vps_scaled_ref_layer_bottom_offset[ i ][ j ] | se(v) |
| } |  |
| vps_shvc_reserved_zero_flag | u(1) |
| } |  |

## Video parameter set semantics

The following semantics are added in Video parameter set semantics

vps_ref_layer_offsets_present_flag equal to 1 specifies that scaled reference layer offset parameters are present in VPS. vps_ref_layer_offsets_present_flag equal to 0 specifies scaled reference layer offset parameters are not present in VPS

slice_ref_layer_offsets_enable_flag equal to 1 specifies that slice_ref_layer_offsets_present_flag is present in the slice headers of non-base layer pictures in the CVS. slice_ref_layer_offsets_enable_flag equal to 0 specifies that slice_ref_layer_offsets_present_flag is not present in slice headers in the CVS.

vps_scaled_ref_layer_left_offset[ i ][ j ] specifies the horizontal offset between the upper-left luma sample of the scaled picture of the direct reference layer with index j and the upper-left luma sample of the picture with layer index i in units of two luma samples. When not present, the value of vps_scaled_ref_layer_left_offset[ i ][ j ] is inferred to be equal to 0.

vps_scaled_ref_layer_top_offset[ i ][ j ] specifies the vertical offset between the upper-left luma sample of the scaled picture of the direct reference layer with index j and the upper-left luma sample of the picture with layer index i in units of two luma samples. When not present, the value of vps_scaled_ref_layer_top_offset[ i ][ j ] is inferred to be equal to 0.

vps_scaled_ref_layer_right_offset[ i ][ j ] specifies the horizontal offset between the bottom-right luma sample of the scaled picture of the direct reference layer with index j and the bottom-right luma sample of the picture with layer index i in units of two luma samples. When not present, the value of vps_scaled_ref_layer_right_offset[ i ][ j ] is inferred to be equal to 0.

vps_scaled_ref_layer_bottom_offset[ i ][ j ] specifies the vertical offset between the bottom-right luma sample of the scaled picture of the direct reference layer with index j and the bottom-right luma sample of the picture with layer index i in units of two luma samples. When not present, the value of vps_scaled_ref_layer_bottom_offset[ i ][ j ] is inferred to be equal to 0.

## Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_extension_vui_parameters( ) |  |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_left_offset[ i ] | se(v) |
| scaled_ref_layer_top_offset[ i ] | se(v) |
| scaled_ref_layer_right_offset[ i ] | se(v) |
| scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| } |  |

The related semantics are also removed from in sequence parameter set extension semantics

## General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( NumActiveRefLayerPics > 0 && slice_ref_layer_offsets_enable_flag ) { |  |
| slice_ref_layer_offsets_present_flag | u(1) |
| if( slice_ref_layer_offsets_present_flag ) { |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| slice_scaled_ref_layer_left_offset [ i ] | se(v) |
| slice_scaled_ref_layer_top_offset[ i ] | se(v) |
| slice_scaled_ref_layer_right_offset[ i ] | se(v) |
| slice_scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| } |  |
| ... |  |

## General slice segment header semantics

The following semantics are added in slice segment header set semantics

slice_ref_layer_offsets_present_flag equal to 1 specifies that scaled reference layer offset parameters are present in the slice segment header. slice_ref_layer_offsets_present_flag equal to 0 specifies scaled reference layer offset parameters are not present in slice segment header. When not present, the value of slice_ref_layer_offsets_present_flag is inferred to be equal to 0.

slice_scaled_ref_layer_left_offset[ i ] specifies the horizontal offset between the upper-left luma sample of the scaled i-th direct reference layer picture used for inter-layer prediction and the upper-left luma sample of the current picture in units of two luma samples. When not present, the value of slice_scaled_ref_layer_left_offset[ i ] is inferred to be equal to vps_scaled_ref_layer_left_offset[ LayerIdxInVps[ currLayerId ] ][ inter_layer_pred_layer_idc[ i ] ], where currLayerId is the nuh_layer_id value of the current picture.

slice_scaled_ref_layer_top_offset[ i ] specifies the vertical offset between the upper-left luma sample of the scaled i-th direct reference layer picture used for inter-layer prediction and the upper-left luma sample of the current picture in units of two luma samples. When not present, the value of slice_scaled_ref_layer_top_offset[ i ] is inferred to be equal to vps_scaled_ref_layer_top_offset[ LayerIdxInVps[ currLayerId ] ][ inter_layer_pred_layer_idc[ i ] ] where currLayerId is the nuh_layer_id value of the current picture.

slice_scaled_ref_layer_right_offset[ i ] specifies the horizontal offset between the bottom-right luma sample of the scaled i-th direct reference layer picture used for inter-layer prediction and the bottom-right luma sample of the current picture in units of two luma samples. When not present, the value of slice_scaled_ref_layer_right_offset[ i ] is inferred to be equal to vps_scaled_ref_layer_right_offset[ LayerIdxInVps[ currLayerId ] ][ inter_layer_pred_layer_idc[ i ] ] where currLayerId is the nuh_layer_id value of the current picture.

slice_scaled_ref_layer_bottom_offset[ i ] specifies the vertical offset between the bottom-right luma sample of the scaled i-th direct reference layer picture used for inter-layer prediction and the bottom-right luma sample of the current picture in units of two luma samples. When not present, the value of slice_scaled_ref_layer_bottom_offset[ i ] is inferred to be equal to vps_scaled_ref_layer_bottom_offset[ LayerIdxInVps[ currLayerId ] ][ inter_layer_pred_layer_idc[ i ] ] where currLayerId is the nuh_layer_id value of the current picture.

## Changes to the decoding process

The only change to the decoding process occurs in sub-clause G.8.1.4 in the derivation of variables related to scaled reference offsets, and the in the decoding process of the inter-layer reference picture set, where the resampling process is invoked. The changes are highlighted below:

      - Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to RefPicLayerId[ i ] ) {  
		an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] index i given as inputs
		RefPicSetInterLayer[ i ] = rsPic 
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer.

If the current picture is a RADL picture, there shall be no entry in the RefPicSetInterLayer that is a RASL picture.

NOTE –  An access unit may contain both RASL and RADL pictures.

- G.8.1.4	Resampling process for inter layer reference pictures

Input to this process is:

- a decoded reference layer picture rlPic

- a direct reference layer index dRlIdx .

Output of this process is	the resampled reference layer picture rsPic.

The variables PicWidthInSamplesL and PicHeightInSamplesL are set equal to pic_width_in_luma_samples and pic_height_in_luma_samples, respectively. The variable rsPicSample is defined as a group of sample arrays specifying the resampled sample values of rsPic of the luma and chroma components. The variable rsPicMotion is defined as a group of variable arrays specifying the resampled motion field of rsPic.

The variables RefLayerPicWidthInSamplesL and RefLayerPicHeightInSamplesL are set equal to the width and height of the decoded reference layer picture rlPic in units of luma samples, respectively. The variable rlPicSample is defined as a group of sample arrays specifying the sample values of rlPic of the luma and chroma components. The variable rlPicMotion is defined as a group of variable arrays specifying the compressed motion field of rlPic.

The variables PicWidthInSamplesC, PicHeightInSamplesC, RefLayerPicWidthInSamplesC, and RefLayerPicHeightInSamplesC are derived as follows:

PicWidthInSamplesC  = PicWidthInSamplesL / subWidthC				(G-1)
PicHeightInSamplesC = PicHeightInSamplesL / subHeightC				(G-2)
RefLayerPicWidthInSamplesC  = RefLayerPicWidthInSamplesL / subWidthC	(G-3)
RefLayerPicHeightInSamplesC = RefLayerPicHeightInSamplesL / subHeightC	(G-4)

The variables ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are derived as follows:

ScaledRefLayerLeftOffset = slice_scaled_ref_layer_left_offset[ dRlIdx ] << 1	(G-5)
ScaledRefLayerTopOffset = slice_scaled_ref_layer_top_offset[ dRlIdx]  << 1	(G-6)
ScaledRefLayerRightOffset = slice_scaled_ref_layer_right_offset[ dRlIdx ]  << 1	(G-7)
ScaledRefLayerBottomOffset = slice_scaled_ref_layer_bottom_offset[ dRlIdx ]  << 1	(G-8)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).