| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0094r1 |
| --- | --- |

| Title: | HEVC version 1 (corrigendum): Derivation of CPB removal time |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports that the derivation of nominal CPB removal times of access units in the specification of HEVC version 1 has a bug, more specifically related to the derivation of the variable AuCpbRemovalDelayVal, and proposes a method to fix the issue.

In revision 1 of this document, a few typos have been corrected that are marked.

# Introduction

The HRD conformance of HEVC decoders are performed with the help of HRD parameters that may be signalled along with the bitstream or provided through external means. In order to claim output timing conformance, the decoder would have to satisfy all the conditions as specified in subclause C.5 of the specification.

It is reported that the derivation of nominal CPB removal time in the specification of HEVC version 1 has a bug. The nominal CPB removal time derived for many access units, as a result of this bug, would be incorrect. For convenience, the derivation of nominal CPB removal time and the semantics of the syntax element au_cpb_removal_delay_minus1 are reproduced below.

  - Nominal removal time

The time at which the AU would be removed from the CPB is specified by the CPB removal time, which is derived based on the nominal CPB removal time. The nominal CPB removal time is calculated with respect to the time of initialization of the HRD, and is monotonically increasing with each access unit in decoding order.

The calculation of the nominal removal time is done with respect to the previous AU, in decoding order, that has a buffering period SEI message, and adding an offset that is specified by the variable AuCpbRemovalDelayVal. The derivation of AuCpbRemovalDelayVal is specified in the following section (Semantics of au_cpb_removal_delay_minus1).

The spec text is as follows:

The nominal removal time of the access unit n from the CPB is specified as follows:

–	If access unit n is the access unit with n equal to 0 (the access unit that initializes the HRD), the nominal removal time of the access unit from the CPB is specified by:

AuNominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ SchedSelIdx ]  90000	(C-1)

–	Otherwise, the following applies:

–	When access unit n is the first access unit of a buffering period that does not initialize the HRD, the following applies:

The nominal removal time of the access unit n from the CPB is specified by:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime = AuNominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay =
		Max( ( auCpbRemovalDelayDeltaMinus1 + 1 ),	(C-2)
			Ceil( ( InitCpbRemovalDelay[ SchedSelIdx ]  90000 + 
				AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] )  ClockTick )
}
AuNominalRemovalTime( n ) = baseTime + ClockTick * ( tmpCpbRemovalDelay − CpbDelayOffset )

where AuNominalRemovalTime[ firstPicInPrevBuffPeriod ] is the nominal removal time of the first access unit of the previous buffering period, AuNominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding picture in decoding order with TemporalId equal to 0 that is not a RASL, RADL or sub-layer non-reference picture, AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in subclause Error: Reference source not found, associated with access unit n, and concatenationFlag and auCpbRemovalDelayDeltaMinus1 are the values of the syntax elements concatenation_flag and au_cpb_removal_delay_delta_minus1, respectively, in the buffering period SEI message, selected as specified in subclause Error: Reference source not found, associated with access unit n.

After the derivation of the nominal CPB removal time and before the derivation of the DPB output time of access unit n, the values of CpbDelayOffset and DpbDelayOffset are updated as follows:

–	If one or more of the following conditions are true, CpbDelayOffset is set equal to the value of the buffering period SEI message syntax element cpb_delay_offset, and DpbDelayOffset is set equal to the value of the buffering period SEI message syntax element dpb_delay_offset, where the buffering period SEI message containing the syntax elements is selected as specified in subclause Error: Reference source not found:

–	Access unit n is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit n is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_LP or is a CRA access unit, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and UseAltCpbParamsFlag for access unit n is equal to 1.

–	Otherwise, CpbDelayOffset and DpbDelayOffset are both set equal to 0.

–	When access unit n is not the first access unit of a buffering period, the nominal removal time of the access unit n from the CPB is specified by:

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] +
		ClockTick * ( AuCpbRemovalDelayVal − CpbDelayOffset )	(C-3)

where AuNominalRemovalTime[ firstPicInCurrBuffPeriod ] is the nominal removal time of the first access unit of the current buffering period, and AuCpbRemovalDelayVal is the value of AuCpbRemovalDelayVal derived according to au_cpb_removal_delay_minus1 in the picture timing SEI message, selected as specified in subclause Error: Reference source not found, associated with access unit n.

  - Semantics of au_cpb_removal_delay_minus1

The variable AuCpbRemovalDelayVal is derived based on the syntax element au_cpb_removal_delay_minus1 that is signalled in the picture timing SEI message.

The semantics of au_cpb_removal_delay_minus1, which contains the derivation of AuCpbRemovalDelayVal, is as follows:

au_cpb_removal_delay_minus1 plus 1 specifies the number clock ticks between the nominal CPB removal time of the access unit associated with the picture timing SEI message and the preceding access unit in decoding order that contained a buffering period SEI message. This value is also used to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS. The syntax element is a fixed length code whose length in bits is given by au_cpb_removal_delay_length_minus1 + 1.

NOTE 1 – The value of au_cpb_removal_delay_length_minus1 that determines the length (in bits) of the syntax element au_cpb_removal_delay_minus1 is the value of au_cpb_removal_delay_length_minus1 coded in the VPS or the SPS that is active for the coded picture associated with the picture timing SEI message, although au_cpb_removal_delay_minus1 specifies a number of clock ticks relative to the removal time of the preceding access unit containing a buffering period SEI message, which may be an access unit of a different CVS.

- The variable AuCpbRemovalDelayMsb of the current picture is derived as follows:

- If the current picture is associated with a buffering period SEI message that is applicable to at least one of the operation points to which the picture timing SEI message applies, AuCpbRemovalDelayMsb is set equal to 0.

- Otherwise, the following applies:

- Let maxCpbRemovalDelay be equal to 2au_cpb_removal_delay_length_minus1 + 1.

- Let prevAuCpbRemovalDelayMinus1 and prevAuCpbRemovalDelayMsb be set equal to au_cpb_removal_delay_minus1 and AuCpbRemovalDelayMsb, respectively, of the previous picture in decoding order that has TemporalId equal to 0, that is not a RASL, RADL or sub-layer non-reference picture, and that is within the same buffering period as the current picture.

- AuCpbRemovalDelayMsb is derived as follows:

if( au_cpb_removal_delay_minus1  <=  prevAuCpbRemovalDelayMinus1 )
	AuCpbRemovalDelayMsb = prevAuCpbRemovalDelayMsb + maxCpbRemovalDelay	(D-1)
else
	AuCpbRemovalDelayMsb = prevAuCpbRemovalDelayMsb

The variable AuCpbRemovalDelayVal is derived as follows:

AuCpbRemovalDelayVal = AuCpbRemovalDelayMsb + au_cpb_removal_delay_minus1 + 1	(D-2)

The value of AuCpbRemovalDelayVal shall be in the range of 1 to 232, inclusive. Within one buffering period, the AuCpbRemovalDelayVal values for any two access units shall not be the same.

# Problem statement

The derivation of the nominal removal time for the AUs is currently broken. Consider the example illustrated in Figure 1.

Figure 1: Example with AU 0 that initializes the HRD and AU n that has the subsequent buffering period SEI message. Assume that no other AU has a buffering period SEI message.

For ease of description in the remainder of the document, the association of each syntax element/derived variable with respect to a particular AU k would be represented by the syntax element/derived variable followed by "( k )", when applicable. For example, "au_cpb_removal_delay_minus1( n )" would correspond to the syntax element au_cpb_removal_delay_minus1 corresonding to AU n.

The problem arises in the definition of AuCpbRemovalDelayVal( n + 1 ) and that for all the access units following AU n + 1. Suppose that the desired nominal removal time for each AU is known. In order that the correct nominal removal time of AU n be derived as AuNominalRemovalTime( n ) in equation C-11, it must be true that:

AuCpbRemovalDelayVal( n ) = AuNominalRemovalTime( n ) - AuNominalRemovalTime( 0 )

From the semantics of au_cpb_removal_delay_minus1 and derivation of AuCpbRemovalDelayVal, it is clear that AuCpbRemovalDelayMsb( n ) is equal to 0 and AuCpbRemovalDelayVal( n ) is equal to au_cpb_removal_delay_minus1( n ) + 1.

Since AU n + 1 does not start a buffering period, the following applies:

AuCpbRemovalDelayVal( n + 1 ) = AuNominalRemovalTime( n + 1 ) 
						- AuNominalRemovalTime( n )

Buffering period normally would last for longer periods of time unless a buffering period SEI message is signalled for each picture. Longer the buffering period, it is more likely that AuCpbRemovalDelayVal( n + 1 ) is less than AuCpbRemovalDelayVal( n ).

Consider the derivation of AuCpbRemovalDelayVal( n + 1 ).

AuCpbRemovalDelayVal( n + 1 ) = AuCpbRemovalDelayMsb( n + 1 ) + 											au_cpb_removal_delay_minus1( n + 1 ) + 1

CASE 1: Suppose au_cpb_removal_delay_minus1( n + 1 ) <= prevAuCpbRemovalDelayMinus1( n + 1 ). Then:

AuCpbRemovalDelayVal( n + 1 ) = prevAuCpbRemovalDelayMsb( n + 1 )+ maxCpbRemovalDelay 								+ 1 + au_cpb_removal_delay_minus1( n + 1 ) + 1

Since prevAuCpbRemovalDelayMsb( n + 1 ) = AuCpbRemovalDelayMsb( n ) = 0, it follows that:

AuCpbRemovalDelayVal( n + 1 ) = maxCpbRemovalDelay + 1 +											au_cpb_removal_delay_minus1( n + 1 ) + 1

This is definitely greater than au_cpb_removal_delay_minus1( n ), because maxCpbRemovalDelay + 1 is greater than the largest possible value of au_cpb_removal_delay_minus1( n ). Hence AuCpbRemovalDelayVal( n + 1 ) > au_cpb_removal_delay_minus1( n ) = AuCpbRemovalDelayVal( n ).

CASE 2: Now suppose au_cpb_removal_delay_minus1( n + 1 ) >  prevAuCpbRemovalDelayMinus1( n + 1 ). Then

AuCpbRemovalDelayVal( n + 1 ) = prevAuCpbRemovalDelayMsb( n + 1 ) + 											au_cpb_removal_delay_minus1( n + 1 ) + 1

But since that prevAuCpbRemovalDelayMsb( n + 1 ) = AuCpbRemovalDelayMsb( n ) = 0, it follows that:

AuCpbRemovalDelayVal( n + 1 ) = au_cpb_removal_delay_minus1( n + 1 ) + 1

Therefore, AuCpbRemovalDelayVal( n + 1 ) > prevAuCpbRemovalDelayMinus1( n + 1 ).= au_cpb_removal_delay_minus1( n ) = AuCpbRemovalDelayVal( n ).

In other words, the derivation of AuCpbRemovalDelayVal only works for the case where AuCpbRemovalDelayVal( n + 1 ) > AuCpbRemovalDelayVal( n ), whereas for many cases, as explained before, this would not be true. Thus, the derivation of AuCpbRemovalDelayVal is incomplete.

Consider the example in Fig. 2, where an IRAP picture is signalled every 32 frames, and the frame rate of the sequence is 30 fps. For simplicity, the sizes of all pictures with the same picture type are assumed to be the same. Let the sizes of the I, P and B frames be 200000, 50000, and 5000 bytes, respectively. Let bit_rate and cpb_size for the operation point be 3 Mbps and 2,000,000 bits (250000 bytes), respectively, and cbr_flag equal 1. Let the value of vps_time_scale be 90,000,000 (90 MHz clock) and vps_num_units_in_tick be 100,000. Therefore, the value of ClockTick will be 100,000/90,000,000 = 1/900.

The initial arrival time of each AU can be calculated based on the values, and is given in the following table:

| AU | 0 | 1 | 2 | 3 | 4 | … | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| initArrivalTime(s) | 0 | 0.5333 | 0.6666 | 0.68 | 0.6933 |  | 1.066 | 1.6 |

Figure 2: Sequence where every 32nd frame is an IRAP picture that also contains a buffering period SEI message. The other pictures do not have buffering period SEI messages.

Suppose the AuNominalRemovalTime[ 0 ] should be 0.666s (when the CPB gets full). Therefore InitCpbRemovalDelay is equal to 90,000 Hz * (0.666s) = 60,000. The nominal removal times of the subsequent AUs are dependent on how long it takes for the AUs to be received. For example, AuNominalRemovalTime[ 1 ] is calculated as 0.666s + 0.133s = 0.8s. In other words, the value of AuCpbRemovalDelayVal[ 1 ] should be 0.133s / ClockTick = 120. Similarly, we obtain AuCpbRemovalDelayVal[ k ] = 120 + (k – 1)*12, for k in the range of 2 to 31, respectively.

Now the nominal CPB removal time of AU 32 is computed with respect to AU 0 (the first picture in the previous buffering period). The nominal removal time of AU 32 is 0.666s + (32 / 30)s = 1.733s and therefore, the value of AuCpbRemovalDelayVal[ 32 ] should be (1.733 – 0.666) / ClockTick which is 960. The value of AuCpbRemovalDelayVal[ 33 ] would again be the same as AuCpbRemovalDelayVal[ 1 ], which is 120.

Assume that the length of au_cpb_removal_delay_minus1 is 16 bits. Now, au_cpb_removal_delay_minus1[ 32 ] will equal 960. If au_cpb_removal_delay_minus1[ 33 ] is signalled as 120, the value of AuCpbRemovalDelayVal[ 33 ] would be derived as 120 + 216 and this would result in very high nominal removal time (in this case, approximately 70 s) for AU 33 and this would surely result in a CPB overflow.

# Proposal

To fix the bug, it is proposed to signal an additional syntax element in hrd_parameters( ) that specifies the length of au_cpb_removal_delay_minus1 for AUs that have buffering period SEI messages, and change the semantics of au_cpb_removal_delay_minus1, to fix the derivation of AuCpbRemovalDelayVal. The proposed changes are provided below, with the changes being highlighted.

    - HRD parameters syntax

| hrd_parameters( commonInfPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |  |
| --- | --- | --- |
| if( commonInfPresentFlag ) { |  |  |
| nal_hrd_parameters_present_flag | u(1) |  |
| vcl_hrd_parameters_present_flag | u(1) |  |
| if( nal_hrd_parameters_present_flag ¦ ¦ vcl_hrd_parameters_present_flag ){ |  |  |
| sub_pic_hrd_params_present_flag | u(1) |  |
| if( sub_pic_hrd_params_present_flag ) { |  |  |
| tick_divisor_minus2 | u(8) |  |
| du_cpb_removal_delay_increment_length_minus1 | u(5) |  |
| sub_pic_cpb_params_in_pic_timing_sei_flag | u(1) |  |
| dpb_output_delay_du_length_minus1 | u(5) |  |
| } |  |  |
| bit_rate_scale | u(4) |  |
| cpb_size_scale | u(4) |  |
| if( sub_pic_hrd_params_present_flag ) |  |  |
| cpb_size_du_scale | u(4) |  |
| initial_cpb_removal_delay_length_minus1 | u(5) |  |
| au_cpb_removal_delay_length_minus1 | u(5) |  |
| au_bp_cpb_removal_delay_length_minus1 | u(5) |  |
| dpb_output_delay_length_minus1 | u(5) |  |
| } |  |  |
| } |  |  |
| for( i = 0; i <= maxNumSubLayersMinus1; i++ ) { |  |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |  |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |  |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |  |
| else |  |  |
| low_delay_hrd_flag[ i ] | u(1) |  |
| if( !low_delay_hrd_flag[ i ] ) |  |  |
| cpb_cnt_minus1[ i ] | ue(v) |  |
| if( nal_hrd_parameters_present_flag ) |  |  |
| sub_layer_hrd_parameters( i ) |  |  |
| if( vcl_hrd_parameters_present_flag ) |  |  |
| sub_layer_hrd_parameters( i ) |  |  |
| } |  |  |
| } |  |  |

    - HRD parameters semantics

au_bp_cpb_removal_delay_length_minus1 plus 1 specifies the length, in bits of the au_cpb_removal_delay_minus1 syntax element of the picture timing SEI message when the picture timing SEI message is associated with an access unit that contains a buffering period SEI message.

The semantics of the other syntax elements in the hrd_parameters( ) syntax structure remain the same.

    - Changes to the semantics of the picture timing SEI message

The variable AuCpbRemovalLength is derived as follows:

- If the picture timing SEI message is associated with an access unit that is also associated with a buffering period SEI message, the value of AuCpbRemovalLength is set equal to au_bp_cpb_removal_delay_length_minus1 + 1.

- Otherwise, AuCpbRemovalLength is set equal to au_cpb_removal_delay_length_minus1.

au_cpb_removal_delay_minus1 plus 1 specifies the number clock ticks between the nominal CPB removal time of the access unit associated with the picture timing SEI message and the preceding access unit in decoding order that contained a buffering period SEI message. This value is also used to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS. The syntax element is a fixed length code whose length in bits is given by AuCpbRemovalLengthau_cpb_removal_delay_length_minus1 + 1.

NOTE 2 – The value of au_cpb_removal_delay_length_minus1 that determines the length (in bits) of the syntax element au_cpb_removal_delay_minus1 is the value of au_cpb_removal_delay_length_minus1 coded in the VPS or the SPS that is active for the coded picture associated with the picture timing SEI message, although au_cpb_removal_delay_minus1 specifies a number of clock ticks relative to the removal time of the preceding access unit containing a buffering period SEI message, which may be an access unit of a different CVS.

- The variable AuCpbRemovalDelayMsb of the current picture is derived as follows:

- If the current picture is associated with a buffering period SEI message that is applicable to at least one of the operation points to which the picture timing SEI message applies, AuCpbRemovalDelayMsb is set equal to 0.

- Otherwise, let prevAnchorPic be the the previous picture in decoding order that has TemporalId equal to 0, that is not a RASL, RADL or sub-layer non-reference picture, and that is within the same buffering period as the current picture, and the following applies:

- If prevAnchorPic is associated with a buffering period SEI message that is applicable to at least one of the operation points to which the picture timing SEI message applies, AuCpbRemovalDelayMsb is set equal to 0.

- Otherwise, the following applies:

  - Let maxCpbRemovalDelay be equal to 2AuCpbRemovalLengthau au_cpb_removal_delay_length_minus1 + 1.

  - Let prevAuCpbRemovalDelayMinus1 and prevAuCpbRemovalDelayMsb be set equal to au_cpb_removal_delay_minus1 and AuCpbRemovalDelayMsb, respectively, of prevAnchorPicthe previous picture in decoding order that has TemporalId equal to 0, that is not a RASL, RADL or sub-layer non-reference picture, and that is within the same buffering period as the current picture.

  - AuCpbRemovalDelayMsb is derived as follows:

if( au_cpb_removal_delay_minus1  <=  prevAuCpbRemovalDelayMinus1 )
	AuCpbRemovalDelayMsb = prevAuCpbRemovalDelayMsb + maxCpbRemovalDelay	(D-1)
else
	AuCpbRemovalDelayMsb = prevAuCpbRemovalDelayMsb

The variable AuCpbRemovalDelayVal is derived as follows:

AuCpbRemovalDelayVal = AuCpbRemovalDelayMsb + au_cpb_removal_delay_minus1 + 1	(D-2)

The value of AuCpbRemovalDelayVal shall be in the range of 1 to 232, inclusive. Within one buffering period, the AuCpbRemovalDelayVal values for any two access units shall not be the same.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).