| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0095 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0062 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Inter-layer reference pictures in reference picture list initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ying Chen Li Zhang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-658-5804 aramasub@qti.qualcomm.com 1-858-845-6589 cheny@qti.qualcomm.com 1-858-651-6660 lizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

It is proposed that explicitly signaling the start position of inter-layer reference pictures in the reference picture list can be used to reduce the number of reference picture list modification commands, and hence the number of bits used in the slice header, in typical cases in MV-HEVC and SHVC.

# Introduction

In SHVC and MV-HEVC, the default position of the inter-layer reference pictures in the reference picture list initialization is after the short-term and long-term reference pictures. In many cases, the number of short-term and long-term reference pictures exceeds the size of active reference picture list, and thus the inter-layer reference pictures are not present in the default reference picture list. In such cases, the inter-layer reference pictures are placed into a desired position in the final reference picture list by invoking the reference picture list modification process. The reference picture list modification process of HEVC is less efficient that that in AVC because in HEVC, the modification process signals all the entries of the list. In AVC, the number of commands sent could be less that the size of the reference picture list. Even if only one of the pictures are re-positioned in the reference picture list, the current modification process in HEVC would signal all the entries in the reference picture list. Therefore, reference picture list construction is inefficient for typical cases in MV-HEVC and SHVC, where an inter-layer reference picture is included in the final reference picture list.

In this document, the position of the inter-layer reference pictures in the final reference picture list is signaled. In many typical cases in SHVC and MV-HEVC, this avoids signaling of reference picture list modification commands in many common cases. The following section provides details of the syntax elements and the changes to the decoding process.

# Proposal

## Slice header extension syntax and semantics

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| … |  |
| if( slice_type != I_SLICE && nuh_layer_id > 0) |  |
| inter_layer_ref_start_position_L0_plus1 | ue(v) |
| if( slice_type = = B_SLICE && nuh_layer_id > 0 ) |  |
| inter_layer_ref_start_position_L1_plus1 | ue(v) |
| … |  |
| byte_alignment( ) |  |
| } |  |

inter_layer_ref_start_position_LX_plus1 specifies the starting position of the inter-layer reference pictures in reference picture list X after reference picture list initialization. inter_layer_ref_start_position_LX_plus1 is in the range of 0 to Min( num_ref_idx_lX_active_minus1 + 1, NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr ), inclusive. When inter_layer_ref_start_position_LX_plus1 is equal to zero, the inter-layer reference pictures are present in the default position in the reference picture list. For other non-zero values, inter_layer_ref_start_position_LX_plus1 minus 1 denotes the starting position of inter-layer reference pictures in the initial reference picture list. When not present, inter_layer_ref_start_position_LX_plus1 is inferred to be the default value, which equals to NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr + 1.

The variable ILRefStartPosLX is derived as follows.

if( !inter_layer_ref_start_position_LX_plus1 )
	ILRefStartPosLX = NumPocStCurrBefore + NumPocStCurrAfter + NumPocLtCurr
else
	ILRefStartPosLX = inter_layer_ref_start_position_LX_plus1 - 1

## Decoding process

Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices as specified in subclause 8.5.3.3.2. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList0. When decoding a B slice, there is a second independent reference picture list RefPicList1 in addition to RefPicList0.

At the beginning of the decoding process for each slice, the reference picture lists RefPicList0 and, for B slices, RefPicList1 are derived as follows:

- The variable NumRpsCurrTempList0 is set equal to Max( num_ref_idx_l0_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp0 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList0 - NumActiveRefLayerPics ) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0;  i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ ) 	(Error: Reference source not found-1)
			RefPicListTemp0[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList0; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetLtCurr[ i ]
	}
	else

for( i = 0; i < NumActiveRefLayerPics; rIdx++, i++ )
			RefPicListTemp0[ rIdx ] = RefPicSetInterLayer[ i ]  
}

// Shift the pictures in the list to create space for inter-layer reference pictures, if necessary
for( cIdx = NumRpsCurrTempList0 – 1; cIdx >= ILRefStartPosL0 + NumActiveRefLayerPics; cIdx - - )
	RefPicListTemp0[ cIdx ] = RefPicListTemp0[ cIdx – NumActiveRefLayerPics ]

// Include the inter-layer reference pictures
for( i = 0; cIdx = ILRefStartPosL0; i < NumActiveRefLayerPics; cIdx+ +, i+ + )
	RefPicListTemp0[ cIdx ] = RefPicSetInterLayer[ i ]

- The list RefPicList0 is constructed as follows:

for( rIdx = 0; rIdx  <=  num_ref_idx_l0_active_minus1; rIdx++)			(Error: Reference source not found-2)
		RefPicList0[ rIdx ] = ref_pic_list_modification_flag_l0 ? RefPicListTemp0[ list_entry_l0[ rIdx ] ] :
																			RefPicListTemp0[ rIdx ]

- When the slice is a B slice, the variable NumRpsCurrTempList1 is set equal to Max( num_ref_idx_l1_active_minus1 + 1, NumPocTotalCurr ) and the list RefPicListTemp1 is constructed as follows:

rIdx = 0
while( rIdx < NumRpsCurrTempList1 - NumActiveRefLayerPics ) {
	if( InterRefEnabledInRPLFlag ) {
		for( i = 0; i < NumPocStCurrAfter  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrAfter[ i ]
		for( i = 0;  i < NumPocStCurrBefore  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ ) 	(Error: Reference source not found-3)
			RefPicListTemp1[ rIdx ] = RefPicSetStCurrBefore[ i ]
		for( i = 0; i < NumPocLtCurr  &&  rIdx < NumRpsCurrTempList1; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetLtCurr[ i ]
	}
	else
		for( i = 0; i< NumActiveRefLayerPics; rIdx++, i++ )
			RefPicListTemp1[ rIdx ] = RefPicSetInterLayer[ i ]
}

// Shift the pictures in the list to create space for inter-layer reference pictures, if necessary
for( cIdx = NumRpsCurrTempList1 - 1; cIdx >= ILRefStartPosL1 + NumActiveRefLayerPics; cIdx - - )
	RefPicListTemp1[ cIdx ] = RefPicListTemp1[ cIdx - NumActiveRefLayerPics]

// Include the inter-layer reference pictures
for( i=0, cIdx = ILRefStartPosL1; i < NumActiveRefLayerPics; cIdx++, i++ )
	RefPicListTemp1[ cIdx ] = RefPicSetInterLayer [ i ]

- When the slice is a B slice, the list RefPicList1 is constructed as follows:

for( rIdx = 0; rIdx <= num_ref_idx_l1_active_minus1; rIdx++)			(Error: Reference source not found-4)
		RefPicList1[ rIdx ] = ref_pic_list_modification_flag_l1 ? RefPicListTemp1[ list_entry_l1[ rIdx ] ] :
																			RefPicListTemp1[ rIdx ]

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).