| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0096 |
| --- | --- |

| Title: | RCE2: Test B.2: Swapping of scan order for transform skip/bypass |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Cohen Anthony Vetro Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

The coefficient scan order mapping in the current HEVC Range Extensions text specifications map Intra prediction modes 6-14 to a scanIdx of 2 (vertical) and Intra prediction modes 22-30 to a scanIdx of 1 (horizontal). The purpose of this proposal is to improve the coding performance when transform skip or transform bypass modes are used, without introducing new scan orders into the standard under development. The proposed method, which is part of Test B.2 of RCE2, swaps the horizontal and vertical scans for transform-skipped or bypassed residuals. This swapping is also tested in combination with the use of a constant context for the significance map for transform-skipped and bypassed residuals, which is proposed in Test B.1 of RCE2. For the classes of video sequences tested under lossless coding conditions, the average bit-rate savings produced by scan-swapping were up to 0.4%, and the average bit-rate savings when scan-swapping was combined with constant context were up to 1.0%. For lossy coding conditions, average BD-Rate improvements for the combined methods were up to 1.8% (2.2% for RGB sequences).

# Introduction

In High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 3 [1], different scan orders are used to scan the prediction residual coefficients of intra-coded blocks. The scan order can be horizontal, vertical, or diagonal. Currently, a vertical scan is used when the intra prediction direction is near horizontal, i.e. modes 6 to 14. A horizontal scan is used for modes 22 to 30, which are near vertical. For the remaining modes, the diagonal scan is used. This mapping is shown on the left side of Figure 1. When transform skip or transform bypass modes are used on a TU, the distribution of data in the block is no longer compacted toward the DC and low-frequency corner normally associated with a transformed block. The existing scan mapping is therefore suboptimal in this case. The purpose of this proposal, which is part of Test B.2 of RCE2, is to propose the mapping originally proposed in [2], [3] , and [4], which improved the performance on skipped blocks without introducing new scan orders into the standard. This mapping is also combined with the constant context for the significance map for transform-skipped and bypassed blocks, which is part of Test B.1 [5] of RCE2.

Figure 1. Existing (left) and proposed (right) scan mappings

# Proposed method

The modification simply swaps the horizontal and vertical scan orders for when transform skip or bypass modes are used. The modification of the scan order mapping is illustrated in the right side of Figure 1. Intra prediction modes 6-14 are mapped to scanIdx 1 (horizontal), and Intra prediction modes 22-30 are mapped to scanIdx 2 (vertical).

This swapping method is additionally tested in combination with the constant context method proposed in Test B.1 of RCE2. Here, the swapping replaces the rotation method of Test B.1.

# Experimental results

The proposed modifications were tested using the HM-10.1+RExt-3.0 reference software and the test conditions described in JCTVC-M1122 [6]. Both lossless and lossy coding experiments were performed.

## Lossless coding results

### Scan swap for all residual block sizes

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.4% | 31.7 | 31.8 | -0.2% | 49.8 | 49.9 | -0.1% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.5 | 0.0% | 381.6 | 381.8 | -0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.3% | 128.9 | 129.0 | 0.0% | 325.6 | 326.5 | -0.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 103% |  |  | 96% |  |  |

### Scan swap for 4x4 and 8x8 residual blocks only

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.4% | 31.7 | 31.8 | -0.2% | 49.8 | 49.9 | -0.1% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.5 | 0.0% | 381.6 | 381.8 | -0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.3% | 128.9 | 129.0 | 0.0% | 325.6 | 326.5 | -0.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 96% |  |  | 90% |  |  |

### Scan swap for 4x4 residual blocks only

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.4% | 31.7 | 31.8 | -0.2% | 49.8 | 49.9 | -0.1% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.4 | 0.0% | 381.6 | 382.4 | -0.2% |
| SC YUV 444 | 11.4 | 11.4 | -0.3% | 128.9 | 129.2 | -0.2% | 325.6 | 325.4 | -0.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  | 99% |  |  |

### Scan swap and constant context for all residual block sizes

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.9% | 31.7 | 31.9 | -0.6% | 49.8 | 50.0 | -0.5% |
| Class B | 2.2 | 2.2 | -0.2% | 2.6 | 2.6 | -0.1% | 2.6 | 2.6 | -0.1% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.3 | -0.1% | 381.6 | 379.8 | 0.0% |
| SC YUV 444 | 11.4 | 11.4 | -0.5% | 128.9 | 128.8 | 0.0% | 325.6 | 325.3 | -0.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | -0.1% | 2.5 | 2.5 | -0.1% |
| Enc Time[%] | 96% |  |  | 98% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  | 91% |  |  |

### Scan swap and constant context for 4x4 and 8x8 residual blocks only

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.9% | 31.7 | 31.9 | -0.6% | 49.8 | 50.0 | -0.5% |
| Class B | 2.2 | 2.2 | -0.2% | 2.6 | 2.6 | -0.1% | 2.6 | 2.6 | -0.1% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.3 | -0.1% | 381.6 | 379.8 | 0.0% |
| SC YUV 444 | 11.4 | 11.4 | -0.5% | 128.9 | 128.8 | 0.0% | 325.6 | 325.3 | -0.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | -0.1% | 2.5 | 2.5 | -0.1% |
| Enc Time[%] | 96% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 85% |  |  |

### Scan swap and constant context for 4x4 residual blocks only

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.0% | 31.7 | 31.9 | -0.6% | 49.8 | 50.0 | -0.5% |
| Class B | 2.2 | 2.2 | -0.2% | 2.6 | 2.6 | -0.1% | 2.6 | 2.6 | -0.1% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.1 | 0.0% | 381.6 | 380.4 | -0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.5% | 128.9 | 129.0 | -0.2% | 325.6 | 325.7 | -0.2% |
| RangeExt | 2.4 | 2.4 | -0.1% | 2.5 | 2.5 | -0.1% | 2.5 | 2.5 | -0.1% |
| Enc Time[%] | 97% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 87% |  |  |

## Lossy coding results

Note that for lossy coding, transform skip is only available on 4x4 residual blocks.

### Scan swap

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.9% | -0.8% | -1.0% | -0.9% | -0.9% | -1.0% | -0.9% | -0.9% | -0.9% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.2% | -1.2% | -1.0% | -1.2% | -1.0% | -0.9% | -1.0% | -0.9% | -0.8% |
| SC YUV 444 | -1.1% | -1.0% | -0.8% | -1.1% | -1.0% | -1.0% | -1.1% | -1.0% | -0.9% |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 96% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.7% | -0.6% | -0.9% | -0.7% | -0.7% | -0.8% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.1% | -1.0% | -0.8% | -1.1% | -0.9% | -0.9% |  |  |  |
| SC YUV 444 | -0.6% | -0.6% | -0.5% | -0.6% | -0.5% | -0.5% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 92% |  |  | 92% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.5% | -0.7% | -0.4% | -0.6% | -0.7% |  |  |  |
| Class B | 0.0% | 0.1% | -0.3% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -0.4% | -0.2% | -0.3% | -0.5% | -0.2% | -0.3% |  |  |  |
| SC YUV 444 | -0.3% | -0.1% | -0.1% | -0.3% | -0.3% | -0.3% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 90% |  |  | 91% |  |  |  |  |  |

### Scan swap and constant context

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.7% | -1.3% | -1.5% | -1.8% | -1.6% | -1.7% | -1.8% | -1.7% | -1.7% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% |
| SC RGB 444 | -2.2% | -1.9% | -1.7% | -2.1% | -1.8% | -1.6% | -2.0% | -1.6% | -1.5% |
| SC YUV 444 | -1.7% | -1.4% | -1.2% | -1.7% | -1.5% | -1.4% | -1.7% | -1.5% | -1.4% |
| RangeExt | -0.2% | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 96% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.2% | -1.0% | -1.0% | -1.2% | -1.0% | -1.0% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.5% | -1.2% | -1.1% | -1.6% | -1.2% | -1.1% |  |  |  |
| SC YUV 444 | -1.1% | -0.9% | -0.7% | -1.0% | -0.8% | -0.7% |  |  |  |
| RangeExt | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 93% |  |  | 93% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -0.4% | 0.0% | -0.8% | -0.7% | -0.6% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -0.8% | -0.5% | -0.6% | -1.0% | -0.7% | -0.7% |  |  |  |
| SC YUV 444 | -0.4% | -0.1% | -0.3% | -0.4% | -0.3% | -0.4% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |  |  |  |

# Conclusions

The objective of this proposal, Test B.2 in RCE2, was to improve the coding performance on transform-skipped or bypassed blocks, without introducing new scan orders or reordering data in the block. For the classes of video sequences tested under lossless coding conditions, the average bit-rate savings produced by scan-swapping were up to 0.4%, and the average bit-rate savings when scan-swapping was combined with constant context were up to 1.0%. For lossy coding conditions, average BD-Rate improvements for the combined methods were up to 1.8% (2.2% for RGB sequences). For the lossless test conditions, the best coding results were obtained when these modifications were restricted to 4x4 residuals.

# References

- D. Flynn, J. Sole, and T. Suzuke, “HEVC Range Extensions Draft 3,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M1005, Incheon, KR, April 2013.

- R. Cohen and A. Vetro, “Modified coefficient scan order mapping for transform skip mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0313, Stockholm, SE, July 2012.

- T. Sugio and T. Nishi, “Modified scan method for intra transform skipping,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0212, Stockholm, SE, July 2012.

- H. Y. Kim, S.-C. Lim, K. Y. Kim, G. H. Park, J. Lee, and J. S. Choi, “Coefficient scan for transform skip mode,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0202, Stockholm, SE, July 2012.

- J. Sole, R. Joshi, and M. Karczewicz, “RCE2 Test B.1: Residue rotation and significance map context,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0033, Vienna, AT, July 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, and M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M1122, Incheon, KR, April 2013.

# Modifications to text specifications

This section shows the corresponding modifications to HEVC Range Extensions Draft 3 [1].

## Scan swapping

### Version that does not restrict modification to 4x4 residuals

7.3.8.11  Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if(transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦¦ (cu_transquant_bypass_flag = = 1)) |  |
| scanIdx = (scanIdx = = 1) ? 2 : ((scanIdx = = 2) ? 1 : scanIdx |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |

### Version that restricts modification to 4x4 residuals

7.3.8.11  Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if(log2TrafoSize = = 2 && (transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦¦ (cu_transquant_bypass_flag = = 1))) |  |
| scanIdx = (scanIdx = = 1) ? 2 : ((scanIdx = = 2) ? 1 : scanIdx |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |

## Significance map context

The significance map context modifications are identical to those in Test B.1 of RCE2, and can therefore be found in JCTVC-N0044 [5].

# Patent rights declaration

Mitsubishi Electric Research Laboratories may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).