| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0097 |
| --- | --- |

| Title: | RCE2: Cross-verification of JCTVC-N0044, Test B.1: Residue rotation and significance map context |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A. | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-N0044, “RCE2 Test B.1: Residue rotation and significance map context.” Simulations were performed using HM-10.1+RExt-3.0 as the reference software. Test B.1 rotates the scanning of the residue coefficients for transform-skipped (lossy) and transform-bypassed (lossless) blocks. This test also applies a constant context for the significance map for such blocks. The simulation results performed for this cross-verification match those provided by the authors of JCTVC-N0044, and their software implementation is consistent with the descriptions of the proposed tools.

# Introduction

RCE2 Test B.1 [1] includes two proposed tools: Rotation of the residue coefficients for transform-skipped or transform-bypassed blocks, and the use of a constant context for the significance map for such blocks. These tools were tested separately and in combination for various block sizes. Test conditions for the core experiment of which includes this test are described in JCTVC-M1122 [2]. The purpose of this document is to report the results of a cross-check of Test B.1.

# Experimental results

## Lossless coding results

### Rotation for all block sizes

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.6% | 31.7 | 31.7 | -0.1% | 49.8 | 49.9 | -0.2% |
| Class B | 2.2 | 2.3 | -0.3% | 2.6 | 2.6 | -0.1% | 2.6 | 2.6 | -0.1% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.5 | 0.0% | 381.6 | 379.9 | 0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.4% | 128.9 | 129.0 | -0.1% | 325.6 | 327.3 | -0.4% |
| RangeExt | 2.4 | 2.4 | -0.3% | 2.5 | 2.5 | -0.1% | 2.5 | 2.5 | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 115% |  |  | 106% |  |  | 102% |  |  |

### Rotation for 4x4 and 8x8 blocks only

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.6% | 31.7 | 31.7 | -0.2% | 49.8 | 49.9 | -0.2% |
| Class B | 2.2 | 2.3 | -0.3% | 2.6 | 2.6 | -0.2% | 2.6 | 2.6 | -0.2% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.4 | 0.0% | 381.6 | 381.9 | -0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.5% | 128.9 | 129.1 | -0.1% | 325.6 | 327.6 | -0.5% |
| RangeExt | 2.4 | 2.4 | -0.3% | 2.5 | 2.5 | -0.2% | 2.5 | 2.5 | -0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 118% |  |  | 110% |  |  | 99% |  |  |

### Rotation for 4x4 blocks only

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.6% | 31.7 | 31.8 | -0.4% | 49.8 | 50.0 | -0.3% |
| Class B | 2.2 | 2.3 | -0.4% | 2.6 | 2.6 | -0.3% | 2.6 | 2.6 | -0.3% |
| SC RGB 444 | 10.1 | 10.1 | -0.3% | 100.4 | 100.5 | -0.1% | 381.6 | 381.7 | -0.2% |
| SC YUV 444 | 11.4 | 11.4 | -0.4% | 128.9 | 129.1 | -0.2% | 325.6 | 327.5 | -0.5% |
| RangeExt | 2.4 | 2.4 | -0.3% | 2.5 | 2.5 | -0.3% | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 117% |  |  | 113% |  |  | 102% |  |  |

### Constant context for all block sizes

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.5% | 31.7 | 31.8 | -0.4% | 49.8 | 49.9 | -0.3% |
| Class B | 2.2 | 2.2 | -0.2% | 2.6 | 2.6 | -0.1% | 2.6 | 2.6 | -0.1% |
| SC RGB 444 | 10.1 | 10.0 | 0.1% | 100.4 | 100.1 | 0.0% | 381.6 | 380.2 | 0.0% |
| SC YUV 444 | 11.4 | 11.4 | -0.1% | 128.9 | 128.7 | 0.0% | 325.6 | 323.9 | 0.1% |
| RangeExt | 2.4 | 2.4 | -0.1% | 2.5 | 2.5 | -0.1% | 2.5 | 2.5 | -0.1% |
| Enc Time[%] | 95% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 109% |  |  | 106% |  |  | 99% |  |  |

### Rotation and constant context for all block sizes

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.2% | 31.7 | 31.9 | -0.6% | 49.8 | 50.1 | -0.6% |
| Class B | 2.2 | 2.3 | -0.5% | 2.6 | 2.6 | -0.2% | 2.6 | 2.6 | -0.2% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.2 | 0.1% | 381.6 | 378.7 | 0.1% |
| SC YUV 444 | 11.4 | 11.4 | -0.5% | 128.9 | 129.0 | -0.2% | 325.6 | 326.4 | -0.5% |
| RangeExt | 2.4 | 2.4 | -0.4% | 2.5 | 2.5 | -0.2% | 2.5 | 2.5 | -0.2% |
| Enc Time[%] | 97% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 112% |  |  | 109% |  |  | 99% |  |  |

### Rotation and constant context for 4x4 blocks only

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.3% | 31.7 | 32.0 | -0.9% | 49.8 | 50.1 | -0.8% |
| Class B | 2.2 | 2.3 | -0.6% | 2.6 | 2.6 | -0.5% | 2.6 | 2.6 | -0.5% |
| SC RGB 444 | 10.1 | 10.1 | -0.2% | 100.4 | 100.4 | -0.3% | 381.6 | 380.3 | -0.3% |
| SC YUV 444 | 11.4 | 11.4 | -0.6% | 128.9 | 129.4 | -0.6% | 325.6 | 326.8 | -0.7% |
| RangeExt | 2.4 | 2.4 | -0.4% | 2.5 | 2.5 | -0.4% | 2.5 | 2.5 | -0.4% |
| Enc Time[%] | 97% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 113% |  |  | 110% |  |  | 101% |  |  |

## Lossy coding results

Note that for lossy coding, transform skip is only available on 4x4 blocks.

### Rotation

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.5% | -1.7% | -1.9% | -1.4% | -1.5% | -1.7% | -1.3% | -1.3% | -1.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -2.2% | -2.2% | -2.1% | -1.9% | -2.0% | -1.9% | -1.7% | -1.7% | -1.6% |
| SC YUV 444 | -2.4% | -2.4% | -2.2% | -2.2% | -2.4% | -2.2% | -2.1% | -2.2% | -2.0% |
| RangeExt | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -1.3% | -1.4% | -0.9% | -1.1% | -1.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.6% | -1.5% | -1.4% | -1.5% | -1.4% | -1.3% |  |  |  |
| SC YUV 444 | -1.5% | -1.7% | -1.4% | -1.4% | -1.6% | -1.4% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -0.9% | -0.4% | -0.6% | -0.7% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.9% | -0.8% | -0.9% | -1.0% | -0.9% | -0.9% |  |  |  |
| SC YUV 444 | -1.1% | -1.0% | -0.9% | -1.0% | -1.0% | -1.0% |  |  |  |
| RangeExt | 0.0% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |  |  |  |

### Constant context

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.1% | -0.7% | -0.8% | -1.1% | -0.9% | -1.0% | -1.2% | -1.0% | -1.1% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% | 0.2% | 0.1% |
| SC RGB 444 | -1.3% | -1.0% | -1.0% | -1.2% | -1.0% | -1.0% | -1.1% | -1.0% | -1.0% |
| SC YUV 444 | -0.8% | -0.6% | -0.5% | -0.8% | -0.6% | -0.6% | -0.8% | -0.7% | -0.6% |
| RangeExt | -0.1% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.7% | -0.4% | -0.4% | -0.7% | -0.4% | -0.5% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.8% | -0.5% | -0.5% | -0.9% | -0.7% | -0.7% |  |  |  |
| SC YUV 444 | -0.2% | 0.0% | 0.1% | -0.2% | 0.0% | 0.0% |  |  |  |
| RangeExt | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | 0.5% | 0.1% | -0.5% | 0.0% | -0.3% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.6% | -0.4% | -0.5% | -0.9% | -0.6% | -0.7% |  |  |  |
| SC YUV 444 | -0.2% | 0.1% | -0.1% | -0.2% | -0.1% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.1% | 0.2% | 0.4% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |

### Rotation and constant context

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.1% | -1.9% | -2.1% | -2.1% | -2.0% | -2.2% | -2.0% | -1.9% | -2.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| SC RGB 444 | -2.8% | -2.6% | -2.5% | -2.6% | -2.5% | -2.3% | -2.3% | -2.2% | -2.1% |
| SC YUV 444 | -2.7% | -2.6% | -2.3% | -2.6% | -2.7% | -2.4% | -2.5% | -2.6% | -2.3% |
| RangeExt | -0.2% | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.4% | -1.4% | -1.4% | -1.4% | -1.3% | -1.4% |  |  |  |
| Class B | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -2.1% | -1.8% | -1.7% | -1.9% | -1.6% | -1.6% |  |  |  |
| SC YUV 444 | -1.9% | -1.9% | -1.6% | -1.7% | -1.7% | -1.4% |  |  |  |
| RangeExt | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -0.3% | -1.0% | -1.0% | -0.6% | -1.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.6% | -1.3% | -1.3% | -1.9% | -1.7% | -1.6% |  |  |  |
| SC YUV 444 | -0.9% | -0.8% | -0.9% | -1.1% | -1.1% | -1.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% | 0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |  |  |  |

# Conclusions

The simulation results for the cross-check match those of the proponent. Encode and decode times are consistent as well, and any variations are likely due to variations in simulation platforms. The modifications to the software are consistent with the tools described in the proposal.

# References

- J. Sole, R. Joshi, and M. Karczewicz, “RCE2 Test B.1: Residue rotation and significance map context,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0044, Vienna, AT, July 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, and M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M1122, Incheon, KR, April 2013.