| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0098 |
| --- | --- |

| Title: | RCE2: Cross-verification of JCTVC-N0073, Description and report for Test D.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A. | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-N0073, “	RCE2: Description and report for Test D.3.” Simulations were performed using HM-10.1+RExt-3.0 as the reference software. Test D.3 combines Test C.3, the replacement of planar mode with template-based sample-adaptive weighted prediction, and Test C.4, the replacement of DC mode with a modified sample-based DC prediction. The simulation results performed for this cross-verification match those provided by the authors of JCTVC-N0073.

# Introduction

RCE2 Test D.3 [1] tests a combination of Tests C.3 and C.4. Test C.3 replaces planar mode with template-based sample-adaptive weighted prediction. Test C.4 replaces DC mode with a modified sample-based DC prediction. These lossless coding tests affect only intra-coded blocks. Test conditions for the core experiment of which includes this test are described in JCTVC-M1122 [2]. The purpose of this document is to report the results of a cross-check of Test D.3.

# Experimental results

Lossless coding results are summarized in the following table:

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.5 | -3.4% | 31.7 | 32.4 | -1.8% | 49.8 | 50.5 | -1.5% |
| Class B | 2.2 | 2.3 | -2.9% | 2.6 | 2.6 | -0.7% | 2.6 | 2.6 | -0.6% |
| SC RGB 444 | 10.1 | 10.8 | -6.2% | 100.4 | 107.6 | -4.5% | 381.6 | 415.2 | -4.4% |
| SC YUV 444 | 11.4 | 12.1 | -4.8% | 128.9 | 135.4 | -3.4% | 325.6 | 347.7 | -3.3% |
| RangeExt | 2.4 | 2.4 | -1.4% | 2.5 | 2.5 | -0.4% | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 102% |  |  | 103% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 95% |  |  | 99% |  |  |

# Conclusions

The simulation results for the cross-check match those of the proponent. Encode and decode times are consistent as well.

# References

- E. Wige, P. Amon, and R. Joshi, “	RCE2: Description and report for Test D.3,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0073, Vienna, AT, July 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, and M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M1122, Incheon, KR, April 2013.