| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0099 |
| --- | --- |

| Title: | RCE2: Cross-verification report for subtest B.3 (N0175) on mirroring of coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rajan Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-658-4511 rajanj@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of a cross-verification performed by Qualcomm of subtest B.3 of HEVC Range Extensions core experiment 2. The results of the subtest are documented in JCTVC-N0175 by BBC. In this subtest, mirroring of the coefficients is applied to intra-coded blocks when transform-skip or transform-bypass is selected. The mirroring depends on the selected coefficient scanning mode. The BD-rate performance results match those provided by the proponent. The implemented algorithm agrees with the core experiment description and BBC’s proposal.

# Introduction

Test B.3 of HEVC Range Extensions core experiment 2 (RCE2) [1] tested the mirroring of the coefficients applied to intra-coded blocks when transform-skip or transform-bypass is selected. The report of subtest can be found in [2]. When transform-skip or transform-bypass is selected for intra-coded blocks, the coefficients are mirrored along the anti diagonal (45°) when the coefficient scan is diagonal. This has the effect of moving high magnitude coefficients from the bottom-right corner to the top-left corner, making them more suitable for HEVC entropy coding. When the coefficient scanning direction is horizontal or vertical the coefficients are mirrored along the main diagonal (135°). This has the effect of swapping horizontal and vertical scans.

Software implementing subtest B.3 on top of HM10.1-RExt-3.0 was provided by BBC. The implemented algorithm agrees with the core experiment description and BBC’s proposal.

# Results

The proposed method was tested for both lossy and lossless configurations under the RCE2 common test conditions Error: Reference source not found. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs.The performance is compared to the anchor in terms of BD-rates.

Table 1: BD-rate performance of subtest B.3 under lossless configuration

Table 2: BD-rate performance of subtest B.3 under lossy configuration

# Conclusions

The results of subtest B.3 on mirroring of intra coefficients for transform-skip and transform-bypass blocks as reported by BBC in [2] have been verified. The implemented algorithm agrees with the core experiment description and BBC’s proposal. The BD-rates match exactly with those provided by the proponents. The encoding and decoding times are within the margin of variability.

# References

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks,” JCTVC-M1122, Incheon, KR, April 2013.

- R. Weerakkody and M. Mrak, “RCE2 Test B.3: Mirroring of Coefficients and significance map context,” JCTVC-N0175, Vienna, AT, July 2013.