| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N100 |
| --- | --- |

| Title: | Non-RCE2: Unified lossless residue coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution studies a residue coding scheme that is applied uniformly to all residues, independent of coding mode (inter/intra) or color components. In the proposed scheme, a flag is used to signal whether DPCM is used; if so, another flag is used to signal if the prediction used in DPCM is in the horizontal or vertical direction. It is asserted that, with the residue prediction scheme used for both inter and intra coding, the proposed method brings average gains of 0.6%, 1.8% and 2.9% for All-Intra, Random-Access and Low-DelayB test configurations.

# Introduction

Lossless coding is achieved in the current HEVC design by bypassing the transform and quantization stage. It has been shown that sample-based prediction can improve coding performance [1][2]. The work in [3] and [4] also showed that sample-wise prediction can improve the performance of lossless inter coding when a flag is available to allow the encoder the option of coding the residues with or without sample-wise prediction.

In this work, a residue DPCM scheme is applied to both inter and intra coding. In this way, one general residue coding scheme can be applied for the lossless coding of all prediction residues, independent of coding mode (inter/intra) or color components. In the proposed scheme, a flag is used to signal if DPCM is used for prediction residue coding; if so, another flag is used to signal if the prediction used in DPCM is in the horizontal or vertical direction. It should be noted that in the proposed scheme, the DPCM direction of intra prediction residue coding is independent of the intra prediction direction.

# Experimental results

The test sequences used are as defined in the test condition for RCE2. During encoding, the decisions on whether to use DPCM and the direction of DPCM were made during residue coding after mode decisions. The option that led to the lowest sum of absolute differences of the residues was chosen.

HM11.0 range extension software is used and the simulation results are presented below for AI-Main, RA-Main and LB-Main configurations:

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.2 | 0.1% | 31.7 | 32.9 | -3.0% | 49.8 | 53.4 | -4.7% |
| Class B | 2.2 | 2.3 | -1.2% | 2.6 | 2.6 | -0.6% | 2.6 | 2.6 | -0.6% |
| SC RGB 444 | 10.1 | 10.2 | -1.3% | 100.4 | 103.9 | -3.8% | 381.6 | 406.3 | -5.3% |
| SC YUV 444 | 11.4 | 11.3 | 0.5% | 128.9 | 130.7 | -1.5% | 325.6 | 341.9 | -3.7% |
| RangeExt | 2.4 | 2.4 | -0.9% | 2.5 | 2.5 | -0.3% | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 116% |  |  | 133% |  |  | 124% |  |  |
| Dec Time[%] | 80% |  |  | 79% |  |  | 74% |  |  |

# Conclusion

In this contribution, a lossless residue coding scheme that can be applied to both inter and intra coding is introduced. In this way, one residue coding scheme can be applied uniformly during the lossless coding of all prediction residues, independent of coding mode (inter/intra) or color components. On top of unifying residue coding for lossless coding, the proposed method also brings average coding performance improvement in all tested configurations.

# Draft Text modification

7.3.8.11		Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( cu_transquant_bypass_flag ) |  |
| { |  |
| rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| ... |  |
| } |  |

7.4.9.11		Residual coding semantics

…

rdpcm_flag[ x0 ][ y0 ][ cIdx ] indicates whether the residual prediction process is applied to the associated transform unit or not. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr. When rdpcm_flag[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

rdpcm_dir_flag[ x0 ][ y0 ][ cIdx ] indicates the direction where the residual prediction process is applied. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for luma, equal to 1 for Cb, and equal to 2 for Cr.

...

### 8.6.2 Scaling and transformation process

Inputs to this process are:

–	a luma location ( xT, yT ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable nT specifying the size of the current transform block.

Output of this process is:

–	the (nT)x(nT) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows.

–	If cIdx is equal to 0,

qP = QP’Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = QP’C	b		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = QP’C	r		(8-3)

The (nT)x(nT) array of residual samples r is derived as specified as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

- If cu_transquant_bypass_flag is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT – 1, y = 0..nT – 1 are derived as follows:

  - If rdpcm_flag[ xTbY ][ yTbY ][cIdx] is equal to 0, the (nT)x(nT) array r is set equal to the (nT)x(nT) array of transform coefficients TransCoeffLevel[ xT ][ yT ][ cIdx ].

  - Otherwise, the residual sample array values r[ x ][ y ] with x = 0..nT – 1, y = 0..nT – 1 are derived as follows:

    - The variables A and B are derived by

A = ( x = = 0 ) ? 0 : r[ x-1 ][ y ]

B = ( y = = 0 ) ? 0 : r[ x ][ y-1 ]

    - The residual sample r[ x ][ y ] is derived by

- r[ x ][ y ] = rdpcm_dir_flag[ xTbY ][ yTbY ][cIdx]?A:B + TransCoeffLevel[ x ][ y ][ cIdx ]

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, the (nT)x(nT) array of transform coefficients, the chroma component variable cIdx and the quantization parameter qP as the inputs and the output is a (nT)x(nT) array of scaled transform coefficients d.

- The (nT)x(nT) array of residual samples r is derived as follows.

- If transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are derived as:

r[ x ][ y ] = ( d[ x ][ y ] << 7 )		(8-4)

- Otherwise (transform_skip_flag[ xT ][ yT ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xT, yT ), the size of the transform block nT, and the (nT)x(nT) array of scaled transform coefficients d as the inputs and the output is a (nT)x(nT) array of residual samples r.

- The variable shift is derived as:

shift = (cIdx = = 0) ? 20 – BitDepthY : 20 – BitDepthC	(8-5)

- The residual sample values r[ x ][ y ] with x = 0..nT − 1, y = 0..nT − 1 are modified as follows.

r[ x ][ y ] = ( r[ x ][ y ] + (1 << ( shift − 1) ) ) >> shift	(8-6)

Table 9-1 – Syntax elements and associated binarizations

...

| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | - |
| --- | --- | --- | --- |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
|  | rdpcm_flag[ ][ ][ ] | FL | cMax = 1 |
|  | rdpcm_dir_flag[ ][ ][ ] | FL | cMax = 1 |

…

Table 9-2 – Assignment of ctxInc to syntax elements with context coded bins

…

| rdpcm_flag[ ][ ][ ] | bypass | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| rdpcm_dir_flag[ ][ ][ ] | bypass | na | na | na | na | na |

…

# References

- Y.-L. Lee, K.-H. Han, and G. J. Sullivan, “Improved lossless intra coding for H.264/MPEG-4 AVC,” IEEE Trans. Image Processing, pp. 2610-2615, Sep 2006.

- M. Zhou, “AHG22: Sample-based angular prediction (SAP) for HEVC lossless coding,” JCTVC-G093, Geneva, Nov 2011.

- Y. H. Tan, C. Yeo, “Non-RCE2: TU-level flag for DPCM,” JCTVC-M0068, Incheon, April 2013.

- M. Naccari et al, “Inter-Prediction Residual DPCM,” JCTVC-M0442, Incheon, April 2013.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).