| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0101 |
| --- | --- |

| Title: | RCE2 Test A.3: Simplified update of the coefficient level Rice parameter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents an update process of the Rice parameter of the syntax element coeff_abs_level_remaining to reportedly improve coding efficiency for lossless coding and lossy screen content coding. The proposed modification removes one condition check. Three variants of the method are presented. For the lossy case, results are also provided when the proposed update is only applied to transform-skip blocks. Results under RCE2 common test conditions show gains for lossless and lossy conditions. For the third variant and max Rice parameter equal to 4, the gains for lossless are

| Method 3 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.0% | 0.0% | -4.8% | -3.4% | -1.1% |
| RA | -0.5% | 0.0% | -4.0% | -3.0% | -0.6% |
| LB | -0.3% | 0.0% | -3.7% | -3.1% | -0.6% |

and the BD-rates for screen content for lossy are

|  | AI RExt Main-tier |  |  | AI RExt High-tier |  |  | AI RExt Super-High |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -0.2% | -0.4% | -0.3% | -0.5% | -0.6% | -0.5% | -0.8% | -0.9% | -0.8% |  |  |  |
| SC RGB444 | -1.3% | -1.2% | -1.2% | -2.5% | -2.5% | -2.5% | -4.0% | -4.3% | -4.2% |  |  |  |
| SC YUV444 | -0.2% | -0.3% | -0.2% | -0.7% | -0.7% | -0.6% | -1.4% | -1.5% | -1.4% |  |  |  |
|  | RA RExt Main-tier |  |  | RA RExt High-tier |  |  | LB RExt Main-tier |  |  | LB RExt High-tier |  |  |
| Class F | -0.1% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% | -0.2% | 0.1% | 0.4% | -0.2% | -0.1% | -0.1% |
| SC RGB444 | -1.1% | -1.1% | -1.0% | -2.1% | -2.1% | -2.1% | -0.5% | -0.4% | -0.4% | -1.9% | -1.7% | -1.8% |
| SC YUV444 | 0.0% | 0.0% | 0.1% | -0.3% | -0.3% | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% | -0.3% |

# Technical Description

The proposed method changes the update process of the Rice parameter used to binarize the syntax element coeff_abs_level_remaining. In HEVC, the parameter is conditionally updated depending on its previous value and the absolute coefficient level.

This contribution proposes an update process that improves coding efficiency while reducing the complexity of the HEVC method by removing the condition check related to the previous value of the Rice parameter and absolute level [1].

Results are presented for the two methods in RCE2 (method 1 & 2) and a third variant of the approach (method 3). The three methods are very similar.

For the lossy case, the update can be applied to only transform-skip blocks. Thus, it is used much more often for screen content than for natural content. The methods are also tested when applied to all the blocks (transform and transform-skip).

Finally, the different variants are also tested with the maximum Rice parameter set equal to 5 instead of 4 as in HEVC.

## Proposed updates

The modifications are applied in section  9.3.3.9 Binarization process for coeff_abs_level_remaining.

Original:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-1)

Method 1:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  <<  cLastRiceParam ) ), 4 )	(9-2)

Method 2:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 1 + ( 1  <<  cLastRiceParam ) ) ), 4 )	(9-3)

Method 3:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  +  cLastRiceParam ) ), 4 )	(9-4)

For the case of maximum  Rice parameter equal to 5, the value of 4 in equations 9-13 has to be replaced by 5.

## Proposed updates applied only to non-transformed coefficients

Since in HEVC lossless coding transform is skipped for all the blocks, the proposed update is used for all the blocks.

For the lossy case, there are two variants. In the first variant, as described above, the method is applied to transform as well as transform-skip blocks. In the second variant, the proposed Rice parameter update is applied only to transform-skip blocks. For the non-skip blocks HEVC version 1 update is used.

The working draft (WD) text for the second variant for method 1 with maximum Rice parameter equal to 4 is shown below. For the case of maximum Rice paramter equal to 5, the value 4 in equation 9-13 has to be replaced by 5. For methods 2 and 3, equation 9-13 below is replaced by the appropriate equation corresponding to the method.

Method 1 with proposed updated applied to only non-transformed coefficients:

- If transform_skip_flag is equal to 1 or cu_transquant_bypass_flag is equal to 1, the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  <<  cLastRiceParam ) ), 4 )	(9-5)

- Otherwise,  the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-6)

# Results

The proposed methods are implemented in HM10.1_Rext3.0 and simulated under the RCE2 common test conditions Error: Reference source not found. The performance is compared to the anchor in terms of BD-rate savings.

## Results for the lossless case

Table 1. Lossless results for the three methods with max Rice parameter equal to 4

| Method 1 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -0.8% | 0.0% | -4.5% | -3.0% | -0.2% |
| RA | -0.4% | -0.1% | -3.6% | -2.7% | 0.3% |
| LB | -0.2% | -0.1% | -3.3% | -2.9% | 0.3% |

| Method 2 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -0.9% | -0.1% | -4.7% | -3.3% | -0.9% |
| RA | -0.5% | 0.0% | -3.8% | -2.9% | -0.5% |
| LB | -0.3% | 0.0% | -3.7% | -3.0% | -0.4% |

| Method 3 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.0% | 0.0% | -4.8% | -3.4% | -1.1% |
| RA | -0.5% | 0.0% | -4.0% | -3.0% | -0.6% |
| LB | -0.3% | 0.0% | -3.7% | -3.1% | -0.6% |

Table 2. Lossless results for the three methods with max Rice parameter equal to 5

| Method 1 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -0.9% | 0.1% | -6.9% | -3.7% | 0.5% |
| RA | -0.4% | -0.1% | -5.6% | -3.4% | 1.3% |
| LB | -0.1% | -0.1% | -5.2% | -3.6% | 1.3% |

| Method 2 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.1% | 0.0% | -7.1% | -4.1% | -0.7% |
| RA | -0.6% | 0.0% | -5.7% | -3.7% | 0.1% |
| LB | -0.3% | 0.0% | -5.4% | -3.9% | 0.1% |

| Method 3 | Class F | Class B | SC RGB444 | SC YUV444 | RangeExt |
| --- | --- | --- | --- | --- | --- |
| AI | -1.2% | 0.0% | -7.3% | -4.2% | -0.9% |
| RA | -0.6% | 0.0% | -6.0% | -3.6% | 0.0% |
| LB | -0.3% | 0.0% | -5.4% | -3.9% | 0.0% |

## Results for the lossy case (update applied to all the blocks)

Table 3. Lossy results for method 2 with max Rice parameter equal to 4

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.3% | -0.5% | -0.5% | -0.5% | -0.7% | -0.7% | -0.7% |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| SC RGB 444 | -1.2% | -1.1% | -1.1% | -2.3% | -2.3% | -2.2% | -3.7% | -3.8% | -3.8% |
| SC YUV 444 | -0.2% | -0.3% | -0.2% | -0.6% | -0.6% | -0.5% | -1.2% | -1.3% | -1.3% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% |  |  |  |
| Class B | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -1.1% | -1.1% | -1.1% | -2.0% | -2.0% | -2.0% |  |  |  |
| SC YUV 444 | 0.1% | 0.0% | 0.2% | -0.2% | -0.2% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.6% | -0.2% | -0.2% | -0.4% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.2% | -1.1% | -1.1% | -2.2% | -2.0% | -2.1% |  |  |  |
| SC YUV 444 | -0.2% | 0.0% | -0.3% | -0.5% | -0.3% | -0.5% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |  |  |  |

Table 4. Lossy results for method 3 with max Rice parameter equal to 4

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.4% | -0.3% | -0.5% | -0.6% | -0.5% | -0.8% | -0.9% | -0.8% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.3% | -1.2% | -1.2% | -2.5% | -2.5% | -2.5% | -4.0% | -4.3% | -4.2% |
| SC YUV 444 | -0.2% | -0.3% | -0.2% | -0.7% | -0.7% | -0.6% | -1.4% | -1.5% | -1.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.1% | -1.1% | -1.0% | -2.1% | -2.1% | -2.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.1% | -0.3% | -0.3% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 94% |  |  | 94% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | 0.1% | 0.4% | -0.2% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.5% | -0.4% | -0.4% | -1.9% | -1.7% | -1.8% |  |  |  |
| SC YUV 444 | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% | -0.3% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |

Table 5. Lossy results for method 2 with max Rice parameter equal to 5

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.2% | -0.4% | -0.4% | -0.3% | -0.5% | -0.6% | -0.5% |
| Class B | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| SC RGB 444 | -1.1% | -1.1% | -1.1% | -2.1% | -2.1% | -2.1% | -3.7% | -4.0% | -3.9% |
| SC YUV 444 | -0.2% | -0.3% | -0.2% | -0.5% | -0.5% | -0.4% | -1.1% | -1.2% | -1.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -1.2% | -1.2% | -1.1% | -2.1% | -2.1% | -2.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.1% | -0.3% | -0.3% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.5% | -0.4% | -0.2% | -0.4% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.7% | -0.7% | -0.8% | -1.7% | -1.6% | -1.7% |  |  |  |
| SC YUV 444 | -0.2% | -0.2% | -0.2% | -0.5% | -0.5% | -0.5% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 104% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |  |  |  |

Table 6. Lossy results for method 3 with max Rice parameter equal to 5

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.3% | -0.4% | -0.4% | -0.4% | -0.7% | -0.7% | -0.7% |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.2% | -1.2% | -1.1% | -2.3% | -2.3% | -2.3% | -4.1% | -4.5% | -4.4% |
| SC YUV 444 | -0.2% | -0.3% | -0.2% | -0.6% | -0.6% | -0.5% | -1.3% | -1.3% | -1.3% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 94% |  |  | 95% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.3% | -0.2% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -1.1% | -1.1% | -1.0% | -2.2% | -2.1% | -2.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.3% | -0.3% | -0.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 101% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.4% | 0.1% | -0.2% | -0.3% | -0.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.9% | -0.8% | -0.8% | -1.8% | -1.7% | -1.8% |  |  |  |
| SC YUV 444 | -0.3% | -0.2% | -0.2% | -0.4% | -0.4% | -0.4% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |  |  |  |

## Results for the lossy case (proposed update applied only to transform-skip blocks)

Table 7. Lossy results for method 1 applied to transform-skip blocks with max Rice parameter equal to 4

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.4% | -0.6% | -0.6% | -0.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.0% | -1.0% | -1.0% | -2.0% | -2.1% | -2.0% | -3.4% | -3.6% | -3.6% |
| SC YUV 444 | -0.2% | -0.2% | -0.2% | -0.5% | -0.5% | -0.5% | -1.1% | -1.2% | -1.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.0% | -1.0% | -1.0% | -2.0% | -2.0% | -2.0% |  |  |  |
| SC YUV 444 | -0.1% | 0.0% | 0.0% | -0.3% | -0.3% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 101% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.2% | -0.3% | -0.2% | -0.4% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.7% | -0.7% | -0.8% | -2.1% | -2.1% | -2.2% |  |  |  |
| SC YUV 444 | -0.1% | 0.1% | -0.1% | -0.3% | -0.2% | -0.4% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |

Table 8. Lossy results for method 2 applied to transform-skip blocks with max Rice parameter equal to 4

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.3% | -0.5% | -0.5% | -0.5% | -0.7% | -0.8% | -0.8% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.2% | -1.2% | -1.2% | -2.3% | -2.3% | -2.3% | -3.7% | -3.9% | -3.8% |
| SC YUV 444 | -0.3% | -0.3% | -0.2% | -0.6% | -0.7% | -0.6% | -1.3% | -1.4% | -1.3% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.1% | -0.2% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.9% | -1.0% | -0.9% | -1.9% | -1.9% | -1.9% |  |  |  |
| SC YUV 444 | -0.1% | -0.1% | 0.0% | -0.3% | -0.3% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 91% |  |  | 92% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.3% | -0.7% | -0.2% | -0.5% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.6% | -0.5% | -0.6% | -2.1% | -2.0% | -2.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.1% | 0.0% | -0.3% | -0.3% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 93% |  |  | 93% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

Table 9. Lossy results for method 3 applied to transform-skip blocks with max Rice parameter equal to 4

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.3% | -0.5% | -0.5% | -0.5% | -0.8% | -0.8% | -0.8% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.2% | -1.2% | -1.2% | -2.4% | -2.4% | -2.4% | -3.8% | -4.1% | -4.0% |
| SC YUV 444 | -0.3% | -0.3% | -0.2% | -0.7% | -0.7% | -0.6% | -1.4% | -1.4% | -1.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 95% |  |  | 95% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 102% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.2% | -0.1% | -0.3% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.1% | -1.1% | -1.0% | -2.1% | -2.1% | -2.1% |  |  |  |
| SC YUV 444 | -0.1% | -0.1% | -0.1% | -0.4% | -0.4% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 96% |  |  | 97% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.5% | -0.5% | -0.3% | -0.5% | -0.5% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.3% | -0.2% | -0.2% | -1.8% | -1.7% | -1.8% |  |  |  |
| SC YUV 444 | 0.1% | 0.3% | 0.1% | -0.2% | -0.1% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |  |  |  |

Table 10. Lossy results for method 1 applied to transform-skip blocks with max Rice parameter equal to 5

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.2% | -0.1% | -0.2% | -0.2% | -0.2% | -0.4% | -0.5% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.8% | -0.8% | -0.8% | -1.6% | -1.6% | -1.6% | -3.3% | -3.6% | -3.5% |
| SC YUV 444 | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.3% | -0.8% | -0.9% | -0.9% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.7% | -0.8% | -0.7% | -1.5% | -1.5% | -1.6% |  |  |  |
| SC YUV 444 | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 95% |  |  | 95% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.1% | -0.5% | -0.1% | -0.1% | -0.5% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.6% | -0.6% | -0.7% | -1.7% | -1.7% | -1.8% |  |  |  |
| SC YUV 444 | 0.0% | 0.1% | 0.0% | -0.2% | -0.1% | -0.2% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |  |  |  |

Table 11. Lossy results for method 2 applied to transform-skip blocks with max Rice parameter equal to 5

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.2% | -0.4% | -0.4% | -0.4% | -0.6% | -0.6% | -0.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.1% | -1.1% | -1.1% | -2.1% | -2.1% | -2.1% | -3.7% | -4.0% | -4.0% |
| SC YUV 444 | -0.3% | -0.3% | -0.2% | -0.6% | -0.6% | -0.5% | -1.1% | -1.2% | -1.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  | 105% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | -0.2% | -0.3% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.9% | -0.9% | -0.9% | -1.7% | -1.8% | -1.8% |  |  |  |
| SC YUV 444 | 0.0% | -0.1% | 0.0% | -0.2% | -0.2% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 94% |  |  | 95% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.2% | -0.6% | -0.2% | -0.2% | -0.4% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.6% | -0.5% | -0.5% | -2.0% | -1.9% | -2.0% |  |  |  |
| SC YUV 444 | 0.0% | 0.1% | 0.0% | -0.3% | -0.2% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 94% |  |  | 93% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |  |  |  |

Table 12. Lossy results for method 3 applied to transform-skip blocks with max Rice parameter equal to 5

|  | All Intra RExt Main-tier |  |  | All Intra RExt High-tier |  |  | All Intra RExt Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.3% | -0.2% | -0.4% | -0.4% | -0.4% | -0.7% | -0.7% | -0.7% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.1% | -1.1% | -1.1% | -2.2% | -2.2% | -2.2% | -3.9% | -4.2% | -4.1% |
| SC YUV 444 | -0.3% | -0.3% | -0.2% | -0.6% | -0.6% | -0.5% | -1.2% | -1.3% | -1.3% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 97% |  |  | 96% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 102% |  |  |
|  | Random Access RExt Main-tier |  |  | Random Access RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.0% | -1.0% | -1.0% | -1.9% | -1.9% | -2.0% |  |  |  |
| SC YUV 444 | -0.2% | -0.2% | -0.1% | -0.4% | -0.4% | -0.3% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |
|  | Low delay B RExt Main-tier |  |  | Low delay B RExt High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.5% | -0.5% | -0.3% | -0.5% | -0.5% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.2% | -0.1% | -0.2% | -1.6% | -1.5% | -1.6% |  |  |  |
| SC YUV 444 | 0.1% | 0.3% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |  |  |  |

# Conclusions

Three Rice parameter update methods are presented that remove a condition in the coefficient coding loop while improving the coding performance for lossless and screen content coding.

# References

- J. Sole, M. Karczewicz, “Simplified update of the coefficient level Rice parameter”, JCTVC-M0366, Incheon, KR, April 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, Incheon, KR, April 2013.

# Working Draft Text

Changes are in section, 9.3.3.9 Binarization process for coeff_abs_level_remaining.

Original:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-7)

## For the case the methods are always applied:

The text is for maximum cRiceParam equals to 4. For the case of maximum cRiceParam equals to 5, the value of 4 in equations 9-13 has to be replaced by 5.

### Text for method 1:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  <<  cLastRiceParam ) ), 4 )	(9-8)

### Text for method 2:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 1 + ( 1  <<  cLastRiceParam ) ) ), 4 )	(9-9)

### Text for method 3:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  +  cLastRiceParam ) ), 4 )	(9-10)

## For the case the methods are applied only to non-transformed residuals:

The text is for maximum cRiceParam equals to 4. For the case of maximum cRiceParam equals to 5, the value of 4 in equations 9-13 has to be replaced by 5.

### Text for method 1:

- If transform_skip_flag is equal to 1 or cu_transquant_bypass_flag is equal to 1, the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  <<  cLastRiceParam ) ), 4 )	(9-11)

- Otherwise,  the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-12)

### Text for method 2:

- If transform_skip_flag is equal to 1 or cu_transquant_bypass_flag is equal to 1, the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 1 + ( 1  <<  cLastRiceParam ) ) ), 4 )	(9-13)

- Otherwise,  the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-14)

### Text for method 3:

- If transform_skip_flag is equal to 1 or cu_transquant_bypass_flag is equal to 1, the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel >> ( 2  +  cLastRiceParam ) ), 4 )	(9-15)

- Otherwise,  the variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-16)

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).