| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0102 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0063 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On alternative collocated picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin (HiSilicon), Jinwen Zan (Huawei) No. 156, Rd. Bei-qing Hai-dian District, Beijing 100095 China | Tel: Email: | +86 10 60612372 yblin@huawei.com jinwen.zan@huawei.com |
| Source: | HiSilicon Technologies, Huawei Technologies |  |  |

_____________________________

# Abstract

This contribution discusses issue on using alternative collocated picture which was adopted in last meeting. TMVP derivation process in HEVC specification exploits not only a collocated picture but also a reference picture list indicator (i.e. collocated_from_l0_flag) which is used in collocated MV derivation. However the adopted method of using alternative collocated picture only signals the collocated picture, but not the reference picture list indicator. Therefore it seems not work well. Two solutions on the issue are proposed. The first solution proposes to avoid usage of the reference picture list indicator in collocated MV derivation by changing the corresponding condition checking. In this way the collocated MV can be derived regardless of the reference picture list indicator when alt_collocated_indication_flag is enabled. The second solution is to additionally signal the reference picture list indictor for the alternative collocated picture.

# Problem Statement

In the last meeting, method of signalling an alternative collocated picture proposed in JCTVC-M0457 was adopted [1][2]. Two syntax elements are introduced for the signalling are shown in table below.

| if( slice_temporal_mvp_enabled_flag ) { |  |
| --- | --- |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |

If alt_collocated_indication_flag is equal to 1, collocated picture is set equal to the picture in the current access unit with nuh_layer_id equal to ActiveMotionPredRefLayerId[ collocated_ref_layer_idx ].

The collocated picture is used for TMVP derivation where a collocated MV will be required for MV scaling. In case of B slice, there are two MVs of a collocated block, i.e. forward MV and backward MV. How to choose one MV from the two MVs as the collocated MV for MV scaling may depend on reference picture list indicator, i.e., collocated_from_l0_flag. For example, according to HEVC specification, if current picture is a bi-predictive picture and POC of any one reference picture is larger than POC of the current picture, collocated_from_l0_flag will be used to determine the collocated MV[3]. However, collocated_from_l0_flag is not signalled in the case of using the alternative collocated picture which is indicated by alt_collocated_indication_flag in current SHVC and MV-HEVC specifications [4][5]. Therefore the method of using alternative collocated picture seems not work well.

# Proposed solutions

The first solution

The first solution proposes to avoid the usage of the similar information as collocated_from_l0_flag which is required in collocated MV derivation process, by changing the corresponding condition checking. In this way the collocated MV can be derived regardless of collocated_from_l0_flag in case of alt_collocated_indication_flag enabled.

For example, in existing TMVP derivation process, a particular reference picture list corresponding to target temporal MV of current block is exploited to derive the collocated MV in a particular case that all POC values of reference pictures of current picture are less than the POC value of current picture. It is proposed to do the same thing as done in the particular case when alt_collocated_indication_flag is enabled. The proposed text change is as follows where only one condition is added.

##### 8.5.3.2.8 Derivation process for collocated motion vectors

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If colPb is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise, the motion vector mvCol, the reference index refIdxCol, and the reference list identifier listCol are derived as follows:

  - If predFlagL0Col[ xColPb ][ yColPb ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL1Col[ xColPb ][ yColPb ], refIdxL1Col[ xColPb ][ yColPb ], and L1, respectively.

  - Otherwise, if predFlagL0Col[ xColPb ][ yColPb ] is equal to 1 and predFlagL1Col[ xColPb ][ yColPb ] is equal to 0, mvCol, refIdxCol, and listCol are set equal to mvL0Col[ xColPb ][ yColPb ], refIdxL0Col[ xColPb ][ yColPb ], and L0, respectively.

  - Otherwise (predFlagL0Col[ xColPb ][ yColPb ] is equal to 1 and predFlagL1Col[ xColPb ][ yColPb ] is equal to 1), the following assignments are made:

      - If DiffPicOrderCnt( aPic, currPic ) is less than or equal to 0 for every picture aPic in every reference picture list of the current slice, or if alt_collocated_indication_flag is equal to 1, mvCol, refIdxCol, and listCol are set equal to mvLXCol[ xColPb ][ yColPb ], refIdxLXCol[ xColPb ][ yColPb ] and LX, respectively.

      - Otherwise, mvCol, refIdxCol, and listCol are set equal to mvLNCol[ xColPb ][ yColPb ], refIdxLNCol[ xColPb ][ yColPb ], and LN, respectively, with N being the value of collocated_from_l0_flag.

and mvLXCol and availableFlagLXCol are derived as follows:

…

The second solution

The second solution is to explicitly signal additional information similar as collocated_from_l0_flag when alt_collocated_indication_flag is enabled. The proposed syntax change is shown in following table.

| if( slice_temporal_mvp_enabled_flag ) { |  |
| --- | --- |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 ) |  |
| alt_collocated_indication_flag | u(1) |
| if( alt_collocated_indication_flag ) { |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |

# Conclusion

This contribution presents two solutions to fix the issue of lacking similar information as collocated_from_l0_flag in the method of alternative collocated picture. The first solution proposes to modify the corresponding condition checking in order to avoid the usage of the similar information as collocated_from_l0_flag which is required in collocated MV derivation process. The second solution proposes to additionally signal the similar information as collocated_from_l0_flag for the alternative collocated picture.

# References

- M. M. Hannuksela, J. Boyce, “MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications (combining aspects of JCTVC-M0205 and JCTVC-M0046)”, JCTVC-M0457, 13th Meeting: Incheon, KR, 18–26 Apr. 2013

- M. M. Hannuksela, K. Ugur, “MV-HEVC/SHVC HLS: On inter-layer sample and syntax prediction indications”, JCTVC-M0205, 13th Meeting: Incheon, KR, 18–26 Apr. 2013

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 10 (for FDIS & Consent),” JCTVC-L1003, 12th Meeting: Geneva, CH, 14–23 Jan. 2013

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Working Draft 2”, JCTVC-M1008, 13th Meeting: Incheon, KR, 18–26 Apr. 2013

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, "MV-HEVC Draft Text 4", JCT3V-D1004, 4th Meeting: Incheon, KR, 20–26 Apr. 2013

# Patent rights declaration(s)

HiSilicon Technologies and Huawei Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).