| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0105 |
| --- | --- |

| Title: | RCE2: Cross-check of Test A.2 on coefficient level Rice parameter update (JCTVC-N0228) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Sole 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of the cross-check of RCE1 test A.2 proposed by TI in JCTVC-N0228. Two methods are presented that modify the update process of the Rice parameter for the coding of the absolute value of transform coefficients. The cross-check performance results match those provided by the proponent. The implemented algorithms agree with what is described in TI’s proposal.

# Description

Software implementing JCTVC-M0316 / JCTVC-N0228 on top of the HM10.1_Rext3.0 was released by TI within the scope of the HEVC Range Extensions CE2.

For screen content and lossless, the statistics of the transform residue are different than for lossy natural content coding. This is exploited in the proposal by modifying the update process of the Rice parameter for coding the absolute level of the transformed coefficients. Two methods are used:

- Conditionally set the Rice parameter to 4 if the condition (Level > 24*(1<<RiceParam) is met

- On the top of 1, if RiceParam is equal to 4, then it is updated to 5

Code changes for these method are small. The modifications are the addition of the checks in the functions related to the entropy coding of the transform coefficients in files TComTrQuant.cpp, TDecSbac.cpp and TEncSbac.cpp.

# Results

Coding results are obtained with the range extensions common conditions defined in JCTVC-M1122 for the lossy and lossless cases. Results match those of the proponent. Results are provided in the accompanying spread-sheets files.