| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0110 |
| --- | --- |

| Title: | SHVC HLS: Earlier DPB clearing for adaptive resolution change |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Ying Chen Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com cheny@qti.qualcomm.com yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In SHVC, resolution change can be enabled by setting single_layer_for_non_irap_flag in the VPS equal to 1. This proposal discusses some related DPB issues, and in order to resolve the issues, proposes to mark all reference pictures in the DPB as "unused for reference", and to remove other decoded pictures with PicOutputFlag equal to 0 from the DPB.

# Resolution change

The semantics of single_layer_for_non_irap_flag is as follows:

single_layer_for_non_irap_flag equal to 1 indicates either that all the VCL NAL units of an access unit have the same nuh_layer_id value or that two nuh_layer_id values are used by the VCL NAL units of an access unit and the picture with the greater nuh_layer_id value is an IRAP picture. single_layer_for_non_irap_flag equal to 0 indicates that nuh_layer_id values may or may not be constrained beyond constraints specified in other parts of this Recommendation | International Standard.

This allows having adaptive resolution switching, for example from a smaller picture resolution to a higher one as shown on the figure below, wherein the picture BL3 is an IRAP picture, and other access units in the figure all contain only one picture, in either the lower layer or the higher layer.

Fig.1 Resolution change

However, with the current design, when resolution switching happens, the reference pictures of the layer from which the resolution was switched (BL in the above example) are still stored in the DPB, but the pictures in the switching-from layer are often no longer needed for inter prediction after the coding of the following access units containing only pictures of the switched-to layer. It is therefore proposed that all reference pictures, including the lower-layer picture of a switching point access unit of the switching-from layer stored in the DPB are marked as "unused for reference". Additionally, for all pictures of the switching-from layer that are not to be output or have been outputted are removed from the DPB. Thus it provides better DPB management and memory usage.

For the example on Fig. 1, when the resolution switching happened in the access unit with POC equal to 3, after decoding of BL3 (base layer picture with POC equal to 3), earlier decoded pictures BL0, BL1, BL2 that might be stored in DPB are no longer necessary for decoding of the BL pictures and can be marked as "unused for reference", and those that are not to be output or have been output, are removed from the DPB. After decoding of EL3 (enhancement layer picture with POC equal to 3), the decoded picture BL3 can be removed from the DPB. Alternatively, all these marking and removal are performed after decoding of EL3.

Alternative to marking of all pictures in the switching-from layer as "unused for reference" and potentially remove all of them from the DPB, it is further possible to keep at least one picture of the switching-from layer in the DPB if it belongs to the lower layer. Let’s name these pictures as "waiting pictures". In this case, when the next resolution switching happens, i.e., from the higher layer to the lower layer, the pictures following the switching point access units in decoding order and belonging to the lower layer may use the waiting pictures for inter prediction.

Fig. 2. Keeping a reference picture for the base layer.

For this scenario, it can be more efficient from compression point of view to keep at least one picture for temporal reference in the base layer, shown as BL3 in Fig. 2, and other base layer pictures can be marked as "unused for reference" as described above. In this case, when switching is done from high to low resolution (or just from higher layer for lower layer), this kept picture can be used for inter prediction shown with red arrow.

We propose to enable both approaches. The proposed changes are provided below.

# Proposed specification changes

Changes are marked with yellow colour.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| single_layer_for_non_irap_flag | u(1) |
| if( single_layer_for_non_irap_flag ) |  |
| keep_base_layer_picture_flag | u(1) |
| … |  |
| } |  |

keep_base_layer_picture_flag equal to 1 specifies that at least one picture from the lower layer picture is marked as "used for reference" for future reference after the switching to a higher layer. keep_base_layer_picture_flag equal to 0 specifies all pictures are marked as "unused for reference" after a layer switching. When not present, the value of keep_base_layer_picture_flag is inferred to be equal to 0.

- F.8.1 General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	If ViewScalExtLayerFlag is equal to 1, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause X.X.X is invoked.

–	Otherwise, when DependencyId[ nuh_layer_id] is greater than 0, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause G.8.1.1 is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

- When single_layer_for_non_irap_flag is equal to 1, the following applies:

  - The variable switchingFlag is set to 0.

  - When the current picture is an IRAP picture and there is a picture in the same access unit with a lower value of nuh_layer_id than the current picture, the following applies. The nuh_layer_id values of these two pictures are denoted as layerIdA and layerIdB with layerIdB greater than layerIdA, switchingFlag is set to 1, the variable layerIdSwitch is set as layerIdA, the variable keepPicFlag is set equal to keep_base_layer_picture_flag.

  - When there is only one picture within the current access unit and its nuh_layer_id value is less than the nuh_layer_id value of the picture in the previous access unit, switchingFlag is set to 1, layerIdSwitch is set to the nuh_layer_id value of the picture in the previous access unit, keepPicFlag is set equal to 0.

  - When switchingFlag is equal to 1, the following applies in the order listed:

    - When keepPicFlag is equal to 1, the picture in the same access unit as that of the current picture is marked as “used for reference”.

    - All other reference pictures with nuh_layer_id equal to layerIdSwitch in the DPB are marked as "unused for reference", and other decoded pictures with PicOutputFlag equal to 0 are removed from the DPB. Each of those pictures remaining in the DPB, except for the current picture and, when keepPicFlag is equal to 1, the lower layer picture in the same access unit as the current picture, is removed from the DPB immediately after it is outputted.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).