| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 |  |  | Document: JCTVC-N0111 |
| --- | --- | --- | --- |
| Title: | Sample position mapping with already defined scaling factor |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In SHVC, scaling factors are used together with the current and reference layer pictures sizes for the resampling process. This contribution proposes to use only scaling factors for this purpose.

# Resampling process

When reference layer picture size is different form the enhancement layer picture size, the resampling process is applied to scale the reference layer picture to match the size of the enhancement layer picture for the inter-layer prediction.

Reference layer fractional sample location derivation is one important step in the resampling process. In current SHVC, the following scaling factors calculation is applied for the reference layer sample location derivation:

ScaleFactorX = ( ( RefLayerPicWidthInSamplesL << 16 ) + ( ScaledRefLayerPicWidthInSamplesL >> 1 ) )
		 / ScaledRefLayerPicWidthInSamplesL		(1)
ScaleFactorY = ( ( RefLayerPicHeightInSamplesL << 16 ) + ( ScaledRefLayerPicHeightInSamplesL
		 >> 1 ) ) / ScaledRefLayerPicHeightInSamplesL.	(2)

Where RefLayerPicWidthInSamplesL and RefLayerPicHeightInSamplesL are the width and the height of the reference layer picture respectively, and correspondingly ScaledRefLayerPicWidthInSamplesL and ScaledRefLayerPicHeightInSamplesL are the width and height of the scaled reference layer picture, which are same to those of the enhancement layer picture when scaled reference offsets are zero.

Meanwhile, the additional rounding offset addY is applied to derive the reference layer sample location in vertical direction. The calculation of addY is as follows:

addY = ( ( ( RefLayerPicHeightInSamplesL * phaseY) << 14 ) + (ScaledRefLayerPicHeightInSamplesL 
		>> 1 ) ) / ScaledRefLayerPicHeightInSamplesL	(3)

And as can be seen, pictures sizes are also used to derive the rouding offset addY with division operation. By comparing the equation (2) and equation (3), it can be found that the calculation of addY and scaleFactorY is similar and their values can be easily derived from each other.

This contirbution proposes using scaling factors for the rounding offset calculation to avoid the additional division operation as follows:

addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(4)

# Specification change

Changes are marked with yellow colour.

G.6.2. Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(G-5)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(G-6)

The variables phaseY and addY are derived as follows:

phaseY = (cIdx = = 0) ? 0 : 1		(G-7)

addY = ( ScaleFactorY * phaseY + 2 ) >> 2 ( ( ( RefLayerPicHeightInSamplesL * phaseY) << 14 ) + (ScaledRefLayerPicHeightInSamplesL >> 1 ) ) / ScaledRefLayerPicHeightInSamplesL		(G-8)

The variables  xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP – offsetX ) * ScaleFactorX  + ( 1 << 11 ) ) >> 12 ) 	(G-6)
yRef16 = ( ( ( yP – offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) –  ( phaseY << 2 )	(G-7)

# Experimental results

Proposed method was implemented on top of SHM2.0 and experimental results based on CTC are summarised in the next table.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).