| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0112 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0065 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: High-level syntax for temporal motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroya Nakamura Motoharu Ueda Hideki Takehara Shigeru Fukushima JVC KENWOOD Corporation 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6459 nakamura.hiroya@jvckenwood.com |
| Source: | JVC KENWOOD Corporation |  |  |

_____________________________

# Abstract

This contribution proposes to add a syntax element(s) to distinguish the following conditions for temporal motion vector prediction in sequence level along with alt_collocated_indication_flag in slice level.

- no temporal motion vector prediction

- temporal motion vector prediction from reference layer or the same layer, and signalling in slice level

- temporal motion vector prediction from only reference layer

- temporal motion vector prediction from only the same layer

JCTVC-N0112 and JCT3V-E0065 contain the same proposals.

# Introduction

According to the current SHVC Working Draft 2 [1] and MV-HEVC Draft 4 [2], SHVC/MV-HEVC decoders can distinguish the following conditions for temporal motion vector prediction in sequence level with reference to sps_temporal_motion_vector_prediction.

- no temporal motion vector prediction

- temporal motion vector prediction from reference layer or the same layer, and signalling in slice level

However, the decoders cannot still distinguish the following conditions.

- temporal motion vector prediction from only reference layer

- temporal motion vector prediction from only the same layer

If condition c) and d) are clarified, some decoders may not need to prepare unused motion fields or keep them in such cases.

# Proposals

This contribution proposes to add a syntax element(s) to distinguish the following conditions for temporal motion vector prediction in sequence level along with alt_collocated_indication_flag in slice level in Proposal 1 and Propsal 2. Proposal 3 and Proposal 4 are SEI versions of Proposal 1 and Proposal 2, respectively.

- no temporal motion vector prediction

- temporal motion vector prediction from reference layer or the same layer, and signalling in slice level

- temporal motion vector prediction from only reference layer

- temporal motion vector prediction from only the same layer

## Proposal 1: Proposed syntax and semantics

Proposal 1 is to add a syntax element sps_alt_collocated_indication in sequence parameter set extension.

- G.7.3.2.2.1	Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_extension_vui_parameters( ) |  |
| if( sps_temporal_mvp_enabled_flag) { |  |
| sps_alt_collocated_indication |  |
| } |  |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_left_offset[ i ] | se(v) |
| scaled_ref_layer_top_offset[ i ] | se(v) |
| scaled_ref_layer_right_offset[ i ] | se(v) |
| scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| } |  |

- G.7.3.6.1	General slice segment header syntax

| slice_segment_headers( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| if( num_extra_slice_header_bits > 0 ) |  |
| discardable_flag | u(1) |
| for( i = 1; i < num_extra_slice_header_bits ; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 && sps_alt_collocated_indication == 2 ) |  |
| slice_alt_collocated_indication_flag | u(1) |
| if( slice_alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(v) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- G7.4.3.2.1	Sequence parameter set extension semantics

sps_alt_collocated_indication equal to 0 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_from_l0_flag, when present, and collocated_ref_idx in the CVS and slice_temporal_mvp_enabled_flag is not present in slice headers in the CVS.
sps_alt_collocated_indication equal to 1 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_ref_layer_idx in the CVS and slice_temporal_mvp_enabled_flag is not present in slice headers in the CVS. 
sps_alt_collocated_indication equal to 2 specifies that slice_alt_collocated_indication_flag is present in the slice headers of non-IDR pictures of non-base layer in the CVS. 
When sps_alt_collocated_indication is not present, it is inferred to be equal to 0.

- G.7.4.7.1	General slice segment header semantics

slice_alt_collocated_indication_flag equal to 0 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_from_l0_flag, when present, and collocated_ref_idx. slice_alt_collocated_indication_flag equal to 1 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_ref_layer_idx. When slice_alt_collocated_indication_flag is not present, it is inferred to be equal to sps_alt_collocated_indication_flag.

It is a requirement of bitstream conformance that the value of slice_alt_collocated_indication_flag shall be the same for all slices of a coded picture.

## Proposal 2: Proposed syntax and semantics

Proposal 2 is to add syntax elements sps_alt_collocated_indication_fixed_flag and sps_alt_collocated_indication_flag (instead of a syntax element sps_alt_collocated_indication in Proposal 1) in sequence parameter set extension.

- G.7.3.2.2.1	Sequence parameter set extension syntax

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_extension_vui_parameters( ) |  |
| if( sps_temporal_mvp_enabled_flag) { |  |
| sps_alt_collocated_indication_fixed_flag |  |
| if( sps_alt_collocated_indication_fixed_flag) { |  |
| sps_alt_collocated_indication_flag |  |
| } |  |
| } |  |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_left_offset[ i ] | se(v) |
| scaled_ref_layer_top_offset[ i ] | se(v) |
| scaled_ref_layer_right_offset[ i ] | se(v) |
| scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| } |  |

- G.7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| if( num_extra_slice_header_bits > 0 ) |  |
| discardable_flag | u(1) |
| for( i = 1; i < num_extra_slice_header_bits ; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPocTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( nuh_layer_id > 0 && NumActiveMotionPredRefLayers > 0 && !sps_alt_collocated_indication_fixed_flag ) |  |
| slice_alt_collocated_indication_flag | u(1) |
| if( slice_alt_collocated_indication_flag ) |  |
| if( NumActiveMotionPredRefLayers > 1 ) |  |
| collocated_ref_layer_idx | ue(1) |
| else { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- G7.4.3.2.1	Sequence parameter set extension semantics

sps_alt_collocated_indication_fix_flag equal to 1 specifies that sps_alt_collocated_indication_flag is present in the sps_extension(). 
sps_alt_collocated_indication_fixed_flag equal to 0 specifies that slice_alt_collocated_indication_flag is present in the slice headers of non-IDR pictures of non-base layer in the CVS. 
When sps_alt_collocated_indication_fix_flag is not present, it is inferred to be equal to 0.

sps_alt_collocated_indication_flag equal to 0 specifies that collocated pictures for temporal motion vector prediction is indicated by collocated_from_l0_flag, when present, and collocated_ref_idx in the CVS and slice_temporal_mvp_enabled_flag is not present in slice headers in the CVS.
slice_alt_collocated_indication_flag equal to 1 specifies that collocated pictures for temporal motion vector prediction is indicated by collocated_ref_layer_idx in the CVS and slice_temporal_mvp_enabled_flag is not present in slice headers in the CVS. 

When sps_alt_collocated_indication_flag is not present, it is inferred to be equal to 0.

- G.7.4.7.1	General slice segment header semantics

slice_alt_collocated_indication_flag equal to 0 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_from_l0_flag, when present, and collocated_ref_idx. slice_alt_collocated_indication_flag equal to 1 specifies that a collocated picture for temporal motion vector prediction is indicated by collocated_ref_layer_idx. When slice_alt_collocated_indication_flag is not present, it is inferred to be equal to sps_alt_collocated_indication_flag.

It is a requirement of bitstream conformance that the value of slice_alt_collocated_indication_flag shall be the same for all slices of a coded picture.

## Proposal 3: Proposed SEI syntax and semantics

Proposal 3 is a SEI version of Proposal 1.

- F.14.1.x	Temporal mvp collocated indication SEI message syntax

| temporal_mvp_collocated_indication( payloadSize ) { | Descriptor |
| --- | --- |
| seq_alt_collocated_indication | u(v) |
| } |  |

- F.14.2.x	Temoral mvp collocated indication SEI message semantics

The temporal mvp collocated indication SEI message provides that the constraint condition of the collocated picture for temporal motion vector prediction is indicated in the CVS.

seq_alt_collocated_indication equal to 0 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_from_l0_flag, when present, and collocated_ref_idx in the CVS.
seq_alt_collocated_indication equal to 1 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_ref_layer_idx in the CVS. 
seq_alt_collocated_indication equal to 2 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_from_l0_flag or collocated_ref_layer_idx in the CVS.

The following semantics is described without collocated_ref_layer_idx.

- F.14.2.x	Temoral mvp collocated indication SEI message semantics

Temporal mvp collocated indication SEI message provides that the constraint condition of the collocated picture for temporal motion vector prediction is indicated in the CVS.

seq_alt_collocated_indication equal to 0 specifies that collocated pictures for temporal motion vector prediction have nuh_layer_id equal to nuh_layer_id of current layer in the CVS.
seq_alt_collocated_indication equal to 1 specifies that collocated pictures for temporal motion vector prediction have nuh_layer_id not equal to nuh_layer_id of current layer in the CVS. 
seq_alt_collocated_indication equal to 2 specifies that collocated pictures for temporal motion vector prediction have nuh_layer_id which is not constrained (may have nuh_layer_id equal to nuh_layer_id of current layer and may have nuh_layer_id not equal to nuh_layer_id of current layer) in the CVS.

## Proposal 4: Proposed SEI syntax and semantics

Proposal 4 is a SEI version of Proposal 2.

- F.14.1.x	Temporal mvp collocated indication SEI message syntax

| temporal_mvp_collocated_indication ( payloadSize ) { | Descriptor |
| --- | --- |
| seq_alt_collocated_indication_fixed_flag | u(1) |
| if( seq_alt_collocated_indication_fixed_flag) |  |
| seq_alt_collocated_indication_flag | u(1) |
| } |  |

- F.14. 2.x	Temporal mvp collocated indication SEI message semantics

The temporal mvp collocated indication SEI message provides that the constraint condition of the collocated picture for temporal motion vector prediction is indicated in the CVS.

seq_alt_collocated_indication_fix_flag equal to 1 specifies that seq_alt_collocated_indication_flag is present in the temporal_mvp_collocated_indication (). 
seq_alt_collocated_indication_fixed_flag equal to 0 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_from_l0_flag or collocated_ref_layer_idx, or temporal motion vector predictors are not used in the CVS in the CVS. 
When seq_alt_collocated_indication_fix_flag is not present, it is inferred to be equal to 0.

seq_alt_collocated_indication_flag equal to 0 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_from_l0_flag, when present, and collocated_ref_idx in the CVS.
slice_alt_collocated_indication_flag equal to 1 specifies that collocated pictures for temporal motion vector prediction are indicated by collocated_ref_layer_idx in the CVS.

The following semantics is described without collocated_ref_layer_idx.

- F.14. 2.x	Temporal mvp collocated indication SEI message semantics

The temporal mvp collocated indication SEI message provides that the constraint condition of the collocated picture for temporal motion vector prediction is indicated in the CVS.

seq_alt_collocated_indication_fix_flag equal to 1 specifies that seq_alt_collocated_indication_flag is present in the temporal_mvp_collocated_indication (). 
seq_alt_collocated_indication_fixed_flag equal to 0 specifies that collocated pictures for temporal motion vector prediction have nuh_layer_id which is not constrained (may have nuh_layer_id equal to nuh_layer_id of current layer and may have nuh_layer_id not equal to nuh_layer_id of current layer) in the CVS. 
When seq_alt_collocated_indication_fix_flag is not present, it is inferred to be equal to 0.

seq_alt_collocated_indication equal to 0 specifies that collocated pictures for temporal motion vector prediction have nuh_layer_id equal to nuh_layer_id of current layer in the CVS.
seq_alt_collocated_indication equal to 1 specifies that collocated pictures for temporal motion vector prediction have nuh_layer_id not equal to nuh_layer_id of current layer in the CVS.

# Conclusions

This contribution proposes to add the syntax element to distinguish the following conditions for temporal motion vector prediction in sequence level along with alt_collocated_indication_flag in slice level.

It is recommended that this proposal be adopted into SHVC Working Draft and MV-HEVC Draft Text and SHM and MV-HEVC.

# References

[1] “SHVC Working Draft 2”,
J. Chen, J. Boyce, Y. Ye, M. Hannuksela, JCTVC-M1008, Incheon, April 2013

[2] “MV-HEVC Draft Text 4”,
G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, JCTVC-D1004, Incheon, April 2013

# Patent rights declaration(s)

JVC KENWOOD Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).