| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0116_r3 |
| --- | --- |

| Title: | AHG5/AHG8: RGB4:4:4 video coding using HEVC multi-view extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Minezawa Shun-ichi Sekiguchi Tokumichi Murakami 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In this contribution, as the base coding architecture of RGB coding to be studied in RExt, a RGB video coding scheme using architecture of HEVC multi-view extensions is proposed. The proposed scheme applies multi-view video coding architecture to encode each of RGB color planes as a view source of multi-view video. Coding performance of the proposed scheme has been evaluated using MV-HEVC software under RExt common test configurations. It is reportedly confirmed that the average BD-rate gains of the proposed scheme relative to RExt3.0 are 9%-25% and 11%-27% for PSNRGBR and PSNRGBRm based measurement, respectively. With regard to screen contents, the result demonstrates that the proposed scheme achieves up to 33% average coding gain compared with RExt3.0. It is also reported that further coding improvement of the proposed scheme relative to RExt3.0 is obtained under the condition of applying QP offset for B and R plane.

# Introduction

As some previous studies mentioned[1][2], color space conversion from RGB to YCbCr and back to RGB causes information loss in the original RGB color space domain. Considering this kind of loss, direct RGB domain coding is preferable to YCbCr domain coding for high fidelity video coding[3]. To encode RGB signal directly, it is important to consider efficient reduction of inter-color redundancy. In this contribution, we propose a new architecture for direct RGB video coding by decomposing each color plane into a monochrome view source and applying HEVC multi-view extensions known as “MV-HEVC”[4].

# Proposed scheme

The proposed coding structure in random access case is shown in Figure 1. To reduce redundancy between color signals, G plane is used as one of reference pictures of inter prediction for corresponding R and B plane. Since MV-HEVC is a high-level syntax extension of HEVC version 1, the proposed scheme requires no block-level changes from HEVC version 1 specification although current RExt specification shall require block-level changes for 4:4:4 format video coding.

Figure 1  The proposed coding structure for random access

(only representing reference pictures indicated primary refIdx of L0/L1 and inter-layer reference)

# Simulations & Results

The performance evaluation of the proposed scheme has been conducted using MV-HEVC test model based on HM10.1[5] modified to allow 4:0:0 format video coding. The coding condition is subject to the common test configurations for HEVC Range Extensions (RExt)[6]. The reference parameter set configurations using All Intra, Random Access and Low Delay B condition are shown in Table 1, respectively.

Table 1  The reference parameter set configurations of the proposed scheme

- All Intra

- Random Access

- Low Delay B

RGB test sequences in RExt common test conditions and screen contents are used in this experiment. Table 2 shows BD-rate measurement of the proposed scheme compared with RExt3.0. The following different weighted average PSNRs are used as objective distortion using BD-rate calculation:

PSNRGBR=4⋅PSNRG+PSNRB+PSNRR6

PSNRGBRm=PSNRG+PSNRB+PSNRR3.

According to the result, the proposed scheme achieves 9.5% to 25.3% and 11.2% to 26.9% average BD-rate gains for PSNRGBR and PSNRGBRm based measurement, respectively. With regard to screen contents, the proposed scheme has up to 33% average coding gain compared with RExt3.0. This result demonstrates that the proposed scheme achieves significant coding improvement on direct RGB coding relative to RExt3.0 especially for screen contents.

Table 2  BD-rate performance compared with RExt3.0

According to JCTVC-N0034[7], further coding improvement is obtained by applying the OP offset for B and R plane set to 6. Therefore, we also conduct an experiment under the condition that each QP for B and R plane is set to QP for G plane plus 6. The coding performance of the proposed scheme compared with RExt3.0 in the condition of applying QP offset is shown in Table 3. It is confirmed that the proposed scheme has up to 36% average gain and degree of the coding improvement of the proposed scheme is enlarged relative to the condition of applying no QP offset.

Table 3  BD-rate performance compared with RExt3.0 under applying the QP offset

# Conclusions

This contribution proposes a RGB domain coding architecture using HEVC multi-view extensions. The coding experiment demonstrates that the proposed scheme has remarkable coding efficiency on direct RGB coding and the coding performance of the proposed scheme achieves up to 25% and 27% BD-rate gain compared with current RExt anchor for PSNRGBR and PSNRGBRm based measurement on average, respectively. We also confirmed that further coding improvement of the proposed scheme relative to RExt3.0 is obtained in the condition of applying the QP offset for B and R plane.

Given the result and the fact that the proposed scheme simply applies MV-HEVC as it is, we believe that the proposed scheme could be considered as base coding architecture for further improvement of direct RGB coding based on HEVC. It is also noted that the proposed architecture is not only for RGB coding but could be a basis for coding of any multi-spectrum video/image signals. We recommend studying the proposed scheme in appropriate AHGs such as AHG5, AHG8 or new study group.

# References

- G. Sullivan, “Approximate theoretical analysis of RGB to YCbCr to RGB conversion error,” Joint Video Team (JVT) of ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, JVT-I017, September, 2003.

- W.-S. Kim, D.-S. Cho and H. M. Kim, “Inter-plane Prediction for RGB Coding,” Joint Video Team (JVT) of ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6, JVT-I023, September, 2003.

- T. Nguyen, A. Khairat and D. Marpe, “Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-M0230, 13th Meeting: Incheon, KR, April, 2013.

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela and J. Boyce, “MV-HEVC Draft Text 4,” Joint Collaborative Team on 3D Video Coding Extension Development of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCT3V-D1004, 4th Meeting: Incheon, KR, April, 2013.

- https://hevc.hhi.fraunhofer.de/svn/svn_3DVCSoftware/tags/HTM-DEV-0.2

- D. Flynn and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-L1006, 12th Meeting: Geneva, CH, January, 2013.

- T. Nguyen, J. Sole and J. Kim, “RCE1: Summary Report of HEVC Range Extensions Core Experiment 1 on Inter-Component Decorrelation Methods,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-N0034, 14th Meeting: Vienna, AT, July, 2013.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).