| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0118v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0084v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On Inter layer Prediction Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Ying Chen Hendry Ye-Kui Wang Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121, USA Ohji Nakagami Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +1-858-651-8208 cjianle@qti.qualcomm.com cheny@qti.qualcomm.com fhendry@qti.qualcomm.com yekuiw@qti.qualcomm.com krapaka@qti.qualcomm.com +81-50-3750-2746 ohji.nakagami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Qualcomm Incorporated and Sony Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a mechanism to avoid slice header inter-layer prediction signaling in two aspects.

The first aspect is to cover two reportedly typical encoding configurations in MV-HEVC and SHVC. The first typical is that pictures of all direct reference layers specified by the VPS are included in the inter-layer RPS subset. The second configuration is that only the picture of the highest direct reference layer specified by the VPS is included in the inter-layer RPS subset. Furthermore, it is proposed to avoid signalling of the layer IDs of pictures to be included in the inter-layer RPS subset when the total number is equal to the number of direct reference layers specified by the VPS, as the information in this case is known.

The second aspect is to signal a set of candidate inter-layer prediction reference picture sets (ILP-RPS) in the VPS and a candidate can be indicated in a slice header to be used for derivation of the ILP-RPS for the picture.

In version 2 of this document, the proposed syntax and semantics have been changed with the two flags use_default_il_rps_only_flag and default_il_rps_all_or_one_layer_flag being replaced by one flag all_ref_layers_active_flag.

# Introduction

Currently, signaling of inter-layer prediction in slice header is always needed for non-base layer pictures as long as NumDirectRefLayers[ nuh_layer_id ] is greater than 0. However, for various typical scenarios, there can be no need to change the inter-layer prediction slice by slice and/or no need to signal any inter-layer prediction in the slice header. In these cases, the bits in the slice header can be saved. Such scenarios include:

- In typical multiview video coding scenarios, the direct reference layer pictures as specified in video parameter set (VPS) are all used for inter-layer prediction and all inserted into at least one reference picture list of the current picture.

- For scalable video coding, it is common to use just the picture belonging to the direct reference layer that is closest to the current picture in decoding order (thus with highest quality) for inter-layer prediction.

For the case other than the two typical cases, it is also observed that repeating inter-layer prediction signalling in slice header may not be efficient due to the following considerations:

- The list of inter-layer reference pictures should be the same for all slices in the same picture.

- The list of inter-layer reference pictures usually does not change often from picture to pictures. Even it changes often, it may still follow certain fixed patterns, for example, pictures with high values of TemporalID may use less inter-layer reference pictures.

Thus, it would be more efficient if the list of candidates inter-layer reference picture set (IL-RPS) that are used often or highly likely to be used is available so that inter-layer prediction signalling in the slice header can be done by simply referring to the available candidate IL-RPS.

# Proposed improvements

This contribution proposes mainly the following changes:

- A flag is added to the VPS to specify whether one of the two default modes for the two typical cases is in use. If true, the second flag is added. When the second flag equals 1, it specifies the use of the typical case where all direct reference layers are present and included in the inter-layer RPS subset of each picture that refers to the VPS. When the second flag equals 0, it specifies the use of the typical case where for each picture referring to the VPS the highest direct reference layer picture is present and only that picture is included in the inter-layer RPS. Note that the first typical case is mainly for MV-HEVC and the second one is mainly for SHVC.

- When neither of the two typical cases is in use, a list of candidate inter-layer RPS candidates is signalled for each layer, and in the slice header an index may be signalled to indicate the use of one of the inter-layer RPS candidates for derivation of the inter-layer RPS.

# Specification change

This section provides the proposed modifications (highlighted with yellow) in specification.

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| max_one_active_ref_layer_flag | u(1) |
| all_ref_layers_active_flag | u(1) |
| if( !all_ref_layers_active_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++) { |  |
| if( NumDirectRefLayers[ layer_id_in_nuh[ i ] ] > 0 ) |  |
| num_ilp_ref_pic_sets_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_ilp_ref_pic_sets_minus1[ i ]; j++ ) |  |
| ilp_ref_pic_set( i, j ) |  |
| } |  |
| ... |  |
| } |  |

all_ref_layers_active_flag equal to 1 specifies that, for each picture referring to the VPS, pictures of all direct reference layers specified by the VPS are present in the same access unit and are included in the inter-layer reference picture set. all_dep_layer_used_flag equal to 0 specifies that the above restriction may or may not apply.

num_ilp_ref_pic_sets_minus1[ i ] plus 1 specifies the number of ilp_ref_pic_set( ) syntax structures included in the VPS for the i-th layer. The value of num_ilp_ref_pic_sets minus1[ i ] shall be in the range of 0 to 63, inclusive.

If num_ilp_ref_pic_sets_minus1[ i ] is present, the variable NumIlpRefPicSets[ nuh_layer_id[ i ] ] is set equal to num_ilp_ref_pic_sets_minus1[ i ] + 1. Otherwise, NumIlpRefPicSets[ layer_id_in_nuh[ i ] ] is set equal to use_default_il_rps_only_flag ? ( NumDirectRefLayers[ layer_id_in_nuh[ i ] ] ? 1 : 0 ) : 0.

| ilp_ref_pic_set( layerIdx, ilpRpsIdx ) { |  | Descriptor |
| --- | --- | --- |
| for( i = layerIdx − 1; ( i >= 0 ) && !( max_one_active_ref_layer_flag && present_in_il_rps_flag[ i + 1 ] ); i− − ) |  |  |
| if( direct_dependency_flag[ layerIdx ][ i ] ) |  |  |
| present_in_il_rps_flag[ i ] |  | u(1) |
| } |  |  |

present_in_il_rps_flag[ i ] equal to 1 specifies that the i-th layer may be a direct reference layer of the pictures in layer layerIdx and is included in the ilpRpsIdx-th inter-layer RPS candidate of the layer with layer index layerIdx.

When all_ref_layers_active_flag is equal to 1, present_in_il_rps_flag[ i ] for the syntax structure ilp_ref_pic_set( layerIdx, 0 ), for layerIdx from 1 to vps_max_layer_minus1, inclusively, is inferred as equal to

- direct_dependency_flag[ layerIdx ][ i ] for i from 0 to layerIdx − 1, inclusive.

When all_ref_layers_active_flag is equal to 0 and present_in_il_rps_flag[ i ] is not present, present_in_il_rps_flag[ i ] is inferred to be equal to 0.

Let the variable nuhLayerId be equal to layer_id_in_nuh[ i ]. The arrays NumInterLayerPics and InterLayerPredLayerIdx are derived as follows:

NumInterLayerPics[ nuhLayerId ][ ilpRpsIdx ] = 0
	for( i = 0, j = 0; i < layerIdx; i++ )
		if( present_in_il_rps_flag[ i ] )
			InterLayerPredLayerIdx[ nuhLayerId ][ ilpRpsIdx ][ j++ ] = i

NumInterLayerPics[ nuhLayerId ][ ilpRpsIdx ] = j

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 && !all_ref_layers_active_flag ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| ilp_ref_pic_set_vps_flag | u(1) |
| if( ilp_ref_pic_set_vps_flag && ( NumIlpRefPicSets[ nuh_layer_id ] > 1 ) ) |  |
| ilp_ref_pic_set_idx | u(v) |
| else if( !ilp_ref_pic_set_vps_flag ) |  |
| ilp_ref_pic_set( LayerIdxInVps[ nuh_layer_id ], NumIlpRefPicSets[ nuh_layer_id ] ) |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| … |  |

inter_layer_pred_enabled_flag equal to 1 specifies that inter-layer prediction may be used in decoding of the current picture. inter_layer_pred_enabled_flag equal to 0 specifies that inter-layer prediction is not used in decoding of the current picture. When not present, the value of inter_layer_pred_enabled_flag is inferred to be equal to 1 if all_ref_layers_active_flag is equal to 1 and NumIlpRefPicSets[ nuh_layer_id ] is greater than 0, and 0, otherwise.

ilp_ref_pic_set_vps_flag equal to 1 specifies that the ILP-RPS of the current picture is derived based on one of the ilp_ref_pic_set( ) syntax structures in the active VPS that is identified by the syntax element ilp_ref_pic_set_idx in the slice header. ilp_ref_pic_set_vps_flag equal to 0 specifies that the ILP RPS of the current picture is derived based on the ilp_ref_pic_set( ) syntax structure that is directly included in the slice headers of the current picture. When not present, ilp_ref_pic_set_vps_flag is inferred to be equal to 1. The ilp_ref_pic_set( ) syntax structure is considered as the NumIlpRefPicSets[ nuh_layer_id ] –th ilp_ref_pic_set( ) syntax structure for the current slice, while the 0-th through NumIlpRefPicSets[ nuh_layer_id ] -1)–th syntax structures are those present in the active VPS.

ilp_ref_pic_set_idx specifies the index, into the list of the ilp _ref_pic_set( ) syntax structures included in the active VPS, of the ilp_ref_pic_set( ) syntax structure that is used for derivation of the ILP RPS of the current picture. The syntax element ilp_ref_pic_set_idx is represented by Ceil( Log2( NumIlpRefPicSets[ nuh_layer_id ] − 1 ) ) bits. When not present, the value of ilp_ref_pic_set_idx is inferred to be equal to 0. The value of ilp_ref_pic_set_idx shall be in the range of 0 to NumIlpRefPicSets[ nuh_layer_id ] − 1, inclusive. When not present, ilp_ref_pic_set_idx is inferred to be equal to 0.

The variable CurrRpsIdx is derived as follows:

–	If ilp_ref_pic_set_vps_flag is equal to 1, CurrIlpRpsIdx is set equal to ilp_ref_pic_set_idx.

–	Otherwise, CurrIlpRpsIdx is set equal to NumIlpRefPicSets[ nuh_layer_id ].

num_inter_layer_ref_pics_minus1 plus 1 specifies the number of pictures that may be used in decoding of the current picture for inter-layer prediction. The length of the num_inter_layer_ref_pics_minus1 syntax element is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of num_inter_layer_ref_pics_minus1 shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.

The variable NumActiveRefLayerPics is derived as follows:

The variable NumActiveRefLayerPics is set equal to NumInterLayerPics[ nuh_layer_id ][ CurrIlpRpsIdx ].

All slices of a picture shall have the same value of NumActiveRefLayerPics and the same value of inter_layer_pred_layer_idc[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive.

inter_layer_pred_layer_idc[ i ] specifies the variable, RefPicLayerId[ i ], representing the nuh_layer_id of the i-th picture that may be used by the current picture for inter-layer prediction. The length of the syntax element inter_layer_pred_layer_idc[ i ] is Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ) bits. The value of inter_layer_pred_layer_idc[ i ] shall be in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] − 1, inclusive.  When not present, the value of inter_layer_pred_layer_idc[ i ] is inferred to be equal to 0.

When i is greater than 0, inter_layer_pred_layer_idc[ i ] shall be greater than inter_layer_pred_layer_idc[ i − 1 ].

The variables RefPicLayerId[ i ] for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, NumActiveMotionPredRefLayers, and ActiveMotionPredRefLayerId[ j ] for each value of j in the range of 0 to NumActiveMotionPredRefLayers − 1, inclusive, are derived as follows:

for( i = 0, j = 0; i < NumActiveRefLayerPics; i++)
	RefPicLayerId[ i ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdx[ nuh_layer_id ][ CurrIlpRpsIdx ][ i ] ]
	if( MotionPredEnabledFlag[ nuh_layer_id ][ InterLayerPredLayerIdx[ nuh_layer_id ][ CurrIlpRpsIdx ][ i ] ] )
		ActiveMotionPredRefLayerId[ j++ ] = RefLayerId[ nuh_layer_id ][ InterLayerPredLayerIdx[ nuh_layer_id ][ CurrIlpRpsIdx ][ i ] ]
}
NumActiveMotionPredRefLayers = j

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–	The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] is greater than TemporalId.

–	The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, the value of SamplePredEnabledFlag[ nuh_layer_id ] [ RefPicLayerId[ i ] ] or MotionPredEnabledFlag[ nuh_layer_id ] [ RefPicLayerId[ i ] ] shall be equal to 1.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).