| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0121r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0107r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Random access of multiple layers |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Definitions of AU, IRAP AU, and CVS are proposed for HEVC layered-extension.

Constraints of IRAP pictures and definitions of IDR/CRA/BLA access units are proposed.

Constraints and Definitions of TSA/STSA and RASL/RADL access unit are proposed.

A minor correction of long-term picture definition is suggested.

# Introduction

In the last Incheon meeting, several contributions suggested and experts discussed on how the random access operates in HEVC layered-extension with consideration of cross-layer alignment. However, many issues were not concluded. In this contribution, several constraints and definitions on access unit and IRAP pictures of HEVC layered-extension are proposed, focusing on the straightforward usage of IRAP pictures in HEVC v1. All the proposals in this contribution assume that the GOP and prediction structures of all layers are identical, and all layers are aligned when random access and splicing operations are performed.

# Proposed definitions on access unit

- Definition of access unit:

The access unit is defined with the picture order count instead of output time.

“all coded pictures associated with the same output time”
      -> “all coded pictures associated with the same picture order count”

Reason: Different pictures belonging to different AUs may have the same output time. 
e.g. frame-level interleaving with FPA SEI.

Different pictures belonging to the same AU may have different output times.
        e.g. time-multiplexing of multiviews

- In Specification text:

access unit: A set of NAL units that are associated with each other according to a specified classification rule, are consecutive in decoding order, and contain the VCL NAL units of all coded pictures associated with the same picture order count and their associated non-VCL NAL units.

- Definition of IRAP access unit:

All the coded picture in an AU shall be IRAP pictures.

Reason: If a picture with nuh_layer_id equal to k in an IRAP AU is not an IRAP picture, the layer with nuh_layer_id equal to k cannot be correctly decoded from the IRAP AU.

- In Specification text:

intra random access point (IRAP) access unit: An access unit in which all the coded pictures are  IRAP pictures.

- Definition of CVS:

The first AU of CVS is an IRAP AU in which NoRaslOutputFlags of all the coded pictures are equal to 1.

Reason: At the starting of a CVS, all RASL pictures of all layers cannot be output.

- In Specification text:

coded video sequence (CVS): A sequence of access units that consists, in decoding order, of an IRAP access unit in which NoRaslOutputFlags of all the coded pictures are equal to 1, followed by zero or more access units that are not IRAP access units in which NoRaslOutputFlags of all the coded pictures are equal to 1, including all subsequent access units up to but not including any subsequent access unit that is an IRAP access unit in which NoRaslOutputFlags of all the coded pictures are equal to 1.

# Constraints on IRAP pictures and Definitions of IRAP AUs

- Definition of IDR and BLA access units:

Keeping the current constrains on IDR and BLA pictures as is;

If a picture in an AU is an IDR picture, all pictures in the AU shall be IDR pictures.

If a picture in an AU is a BLA picture, all pictures in the AU shall be BLA pictures.

- In Specification text:

When the nal_unit_type value nalUnitTypeA is equal to IDR_W_DLP, IDR_N_LP, BLA_W_LP, BLA_W_DLP or BLA_N_LP for a coded picture, the nal_unit_type value shall be equal to nalUnitTypeA for all VCL NAL units of all coded pictures of the same access unit.

The definitions of IDR and BLA access units are proposed

Reason: Aligning IDR pictures and BLA pictures with an IDR AU and a BLA AU is straightforward and easy to define.

- In Specification text:

instantaneous decoding refresh (IDR) access unit: An access unit in which all the coded pictures are IDR pictures

broken link access (BLA) access unit: An access unit in which all the coded pictures are BLA pictures.

- Definition of CRA access unit:

The definition of CRA access unit is also proposed with the constraint same with IDR and BLA, because the GOP structure of enhancement layers shall be aligned with the GOP structure of a base layer.

If a picture in an AU is a CRA picture, all pictures in the AU shall be CRA pictures.

Reason: Aligning CRA pictures with a CRA AU is straightforward and easy to define. When spliced, a CRA AU is changed to a BLA AU.

- In Specification text:

- clean random access (CRA) access unit: An access unit in which all the coded pictures are CRA pictures.

- Definitions of TSA/STSA and RADL/RASL access units:

Because it is desirable that the GOP and prediction structures of all the layers are the same, the alignment of all TSA/STSA and RADL/RASL pictures in an AU has to be same as the alignment of IRAP pictures.

If a picture in an AU is a TSA picture, all pictures in the AU shall be TSA pictures.

If a picture in an AU is a STSA picture, all pictures in the AU shall be STSA pictures.

If a picture in an AU is a RADL picture, all pictures in the AU shall be RADL pictures.

If a picture in an AU is a RASL picture, all pictures in the AU shall be RASL pictures.

- In Specification text:

step-wise temporal sub-layer access (STSA) access unit: An access unit in which all the coded pictures are STSA pictures.

temporal sub-layer access (TSA) access unit: An access unit in which all the coded pictures are TSA pictures.

random access decodable leading (RADL) access unit: An access unit in which all the coded pictures are RADL pictures.

random access skipped leading (RASL) access unit: An access unit in which all the coded pictures are  RASL pictures.

- Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to RefPicLayerId[ i ] ) {  
		an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and   DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] given as inputs
		RefPicSetInterLayer[ i ] = rsPic 
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer.

If the current picture is a RADL picture, there shall be no entry in the RefPicSetInterLayer that is a RASL picture.

NOTE –  An access unit may contain both RASL and RADL pictures.

# Minor change in definition of long-term reference picture

Even if a reference picture is marked as "used for long-term reference", the reference picture picture cannot be a long-term reference picture, when the nuh_layer_id of the reference picture is other than the nuh_layer_id of the current picture. This is because interlayer referene pictures are temporally marked as "used for long-term reference". Therefore, long-term reference picture is redefined as:

- In Specification text:

long-term reference picture: A picture that is marked as "used for long-term reference" within the same layer.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).