| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0123 |
| --- | --- |

| Title: | RCE2: Cross-check of Test A.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xianghui Wei Jinwen Zan | Tel: Email: | +86-10-82882199 weixianghui@huawei.com jinwen.zan@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results performed by Huawei on JCTVC-N0228. The proposed technology of modified cRiceParam update process is cross-checked. The BD-rate and complexity results from our tests match the results provided by proponents.

# Introduction

In JCTVC-N0228, modified cRiceParam update process is proposed. In the first sub-test, cRiceParam update is modified with additional check of absCoeff[idx] > 24*(1<<uiGoRiceParam)). In the second sub-test, additionally, cRiceParam maximum value of 5 is tested.

# Test results

The tests are performed under conditions defined in JCTVC-M1122 [2].

## Test results of 1st sub-test

In the first sub-test, cRiceParam update is modified with additional check of absCoeff[idx] > 24*(1<<uiGoRiceParam)). The following tables summarize BD-rate performance for the proposed method on lossy and lossless conditions.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% | -0.4% | -0.5% | -0.5% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.2% | -0.2% | -0.3% | -1.1% | -1.1% | -1.1% | -2.7% | -3.0% | -3.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% | -0.7% | -0.7% | -0.8% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.2% | -0.2% | -0.2% | -1.0% | -0.9% | -0.9% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.3% | -0.2% | -0.3% | -1.2% | -1.0% | -1.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.8% | 31.7 | 32.0 | -0.4% | 49.8 | 50.0 | -0.2% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 10.6 | -4.4% | 100.4 | 105.7 | -3.6% | 381.6 | 404.2 | -3.4% |
| SC YUV 444 | 11.4 | 11.8 | -2.6% | 128.9 | 133.5 | -2.0% | 325.6 | 342.9 | -2.5% |
| RangeExt | 2.4 | 2.4 | -0.7% | 2.5 | 2.5 | -0.3% | 2.5 | 2.5 | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |

## Test results of 2nd sub-test

In the second sub-test, additionally, cRiceParam maximum value of 5 is tested. The following tables summarize BD-rate performance for the proposed method on lossy and lossless conditions.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% | -0.4% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.2% | -0.2% | -0.2% | -1.0% | -0.9% | -0.9% | -2.8% | -3.2% | -3.1% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.6% | -0.6% | -0.6% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.2% | -0.3% | -0.3% | -0.9% | -0.9% | -0.9% |  |  |  |
| SC YUV 444 | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.1% | -0.1% | -0.2% | -1.1% | -1.1% | -1.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |  |  |  |

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -1.0% | 31.7 | 32.1 | -0.5% | 49.8 | 50.1 | -0.2% |
| Class B | 2.2 | 2.2 | 0.1% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 10.9 | -6.9% | 100.4 | 109.0 | -5.4% | 381.6 | 419.8 | -5.1% |
| SC YUV 444 | 11.4 | 11.9 | -3.4% | 128.9 | 135.3 | -2.8% | 325.6 | 352.1 | -3.4% |
| RangeExt | 2.4 | 2.4 | 0.1% | 2.5 | 2.5 | 0.7% | 2.5 | 2.5 | 0.7% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |

# Conclusion

The BD-rate of two limited motion configurations under lossy and lossless test condition from our tests matches the results provided by JCTVC-N0228.

# References

- M. Budagavi, “RCE2: Results for test A.2 Rice parameter update”, JCTVC-N0228, July 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE 2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, 13th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Incheon, KR, 18-26 Apr., 2013.