| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0124 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0108 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Random layer access |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The concept of random layer access and random layer access pictures are proposed. The random layer access picture is the picture that can be decoded without interlayer prediction from lower layer pictures. Hence, without decoding all pictures in an AU, a random layer access picture in a specific layer can be decoded. Two random layer access picture types are proposed. The first one is the single random layer access picture, which has no dependency from any picture in lower layers and can be successfully decoded without interlayer prediction. The other one is the clean random layer access picture. The clean random layer access picture with nuh_layer_id equal to k has no dependency from any picture with nuh_layer_id less than k, and a picture with nuh_layer_id greater than k also has no dependency from any picture with nuh_layer_id less than k.

# Introduction

In multi-view applications or other multi-layered bitstreams, decoding all layers to access and display a specific picture is redundant. To enable a fast access of a picture in a specific layer, a random layer access picture is proposed. The concept of random layer access is accessing and successfully decoding a picture with nuh_layer_id equal to k, without decoding the pictures with nuh_layer_id less than k. Two random layer access picture types are proposed. The first one is the single random layer access picture, which has no dependency from any picture in lower layers and can be successfully decoded without interlayer prediction. However, the pictures with nuh_layer_id greater than the nuh_layer_id of the single random layer access picture cannot be successfully decoded. The other one is the clean random layer access picture. The clean random layer access picture with nuh_layer_id equal to k has no dependency from any picture with nuh_layer_id less than k, and a picture with nuh_layer_id greater than k also has no dependency from any picture with nuh_layer_id less than k. Hence, after decoding a clean random layer access picture, all upper layer pictures can be successfully decoded. The random layer access is useful for fast access and decoding a specific picture within all pictures in an AU, especially in all intra multiview streams or simulcast streams.

# Proposed specification text

- Random layer access (RLA): The act of starting the decoding process at a non-base layer other than a base layer within an access unit.

Single random layer access (SRLA) picture: A picture with nuh_layer_id equal to k has no dependency from a picture with nuh_layer_id less than k within an access unit.

Clean random layer access (CRLA) picture: A picture with nuh_layer_id equal to k has no dependency from a picture with nuh_layer_id less than k within an access unit. A picture with nuh_layer_id greater than k in the same access unit has no dependency from a picture with nuh_layer_id less than k within an access unit.

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2 3 | TSA_N TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4 5 | STSA_N STSA_R | Coded slice segment of an STSA picture slice_segment_layer_rbsp( ) | VCL |
| 6 7 | RADL_N RADL_R | Coded slice segment of a RADL picture slice_segment_layer_rbsp( ) | VCL |
| 8 9 | RASL_N RASL_R | Coded slice segment of a RASL picture slice_segment_layer_rbsp( ) | VCL |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP sub-layer non-reference VCL NAL unit types | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |
| 16 17 18 | BLA_W_LP BLA_W_RADL BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) | VCL |
| 19 20 | IDR_W_RADL IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22 23 | RSV_IRAP_VCL22 RSV_IRAP_VCL23 | Reserved IRAP VCL NAL unit types | VCL |
| 24 25 | SLRA_NUT_N SLRA_NUT_R | Coded slice segment of a SLRA picture slice_segment_layer_rbsp( ) | VCL |
| 26 27 | CLRA_NUT_N CLRA_NUT_R | Coded slice segment of a CLRA picture slice_segment_layer_rbsp( ) | VCL |
| 28..31 | RSV_VCL24.. RSV_VCL31 | Reserved non-IRAP VCL NAL unit types | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

When nal_unit_type has a value in the range of 24 to 27 and nuh_layer_id is equal to 0, inter_layer_pred_enabled_flag shall be equal to 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).