| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0125 |
| --- | --- |

| Title: | RCE3: Cross-check of Test 3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xianghui Wei Jinwen Zan | Tel: Email: | +86-10-82882199 weixianghui@huawei.com jinwen.zan@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results performed by Huawei on JCTVC-N0287. The proposed technology of multi-stage base color and index map is cross-checked. The BD-rate and complexity results from our tests match the results provided by proponents.

# Introduction

In JCTVC-N0287, a new mode for screen content coding is used. In the new mode, each intra block is represented by several base colors and an index map for each pixel to indicate which base color is used as the reconstructed value for that specific pixel.

# Test results

The tests are performed under conditions defined in JCTVC-M1123 [2].

## Test results of lossy condition

The following table summarizes BD-rate performance for the proposed method on lossy condition.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -15.8% | -14.7% | -16.2% | -13.1% | -13.4% | -14.1% | -10.4% | -11.4% | -11.7% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -41.1% | -41.4% | -41.9% | -43.1% | -43.1% | -43.7% | -44.2% | -44.1% | -44.6% |
| SC YUV 444 | -26.0% | -24.8% | -25.2% | -25.4% | -26.3% | -26.3% | -26.6% | -27.9% | -27.6% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 140% |  |  | 137% |  |  | 136% |  |  |
| Dec Time[%] | 92% |  |  | 93% |  |  | 93% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -11.3% | -11.9% | -12.0% | -9.0% | -10.2% | -10.0% |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -35.1% | -35.2% | -35.7% | -36.9% | -37.0% | -37.5% |  |  |  |
| SC YUV 444 | -21.0% | -22.4% | -22.4% | -21.7% | -23.5% | -23.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 105% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -7.7% | -8.2% | -7.9% | -6.3% | -7.0% | -6.7% |  |  |  |
| Class B | 0.1% | -0.1% | -0.2% | 0.1% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -29.6% | -29.5% | -30.0% | -32.6% | -32.6% | -33.0% |  |  |  |
| SC YUV 444 | -15.6% | -16.3% | -16.9% | -16.3% | -17.1% | -17.4% |  |  |  |
| RangeExt | 0.1% | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 104% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |  |  |  |

## Test results of lossless condition

The following table summarizes BD-rate performance for the proposed method on lossless condition.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.6% | 31.7 | 31.7 | -0.1% | 49.8 | 49.8 | -0.1% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 18.6 | -30.3% | 100.4 | 189.8 | -26.0% | 381.6 | 747.3 | -23.7% |
| SC YUV 444 | 11.4 | 15.0 | -12.8% | 128.9 | 169.6 | -12.0% | 325.6 | 466.8 | -12.0% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 108% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  | 105% |  |  |

# Conclusion

The BD-rate under lossy and lossless test condition from our tests matches the results provided by JCTVC-N0287.

# References

- W. Zhu, J. Xu, W. Ding, “RCE3 Test2: Multi-stage Base Color and Index Map”, JCTVC-N0287, July 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra Coding Methods for Screen Content”, JCTVC-M1123, 13th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Incheon, KR, 18-26 Apr., 2013.