| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0126 |
| --- | --- |

| Title: | RCE3: Cross-check of Test 3.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xianghui Wei Jinwen Zan | Tel: Email: | +86-10-82882199 weixianghui@huawei.com jinwen.zan@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results performed by Huawei on JCTVC-N0205. The proposed technology of intra motion compensation is cross-checked. The BD-rate and complexity results from our tests match the results provided by proponents.

# Introduction

In JCTVC-N0205, a CU-level intra motion compensation tool is proposed to remove redundancy of repeated pattern in text and graphic region. When intra motion compensation is enabled for a CU, either horizontal motion or vertical motion is signaled.

# Test results

The tests are performed under conditions defined in JCTVC-M1123 [2].

## Test results of limited motion within 64

In this test, horizontal and vertical motions are limited to a range of 0~-63. The following tables summarize BD-rate performance for the proposed method on lossy and lossless conditions.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -8.5% | -8.4% | -8.4% | -7.3% | -7.4% | -7.3% | -6.3% | -6.4% | -6.3% |
| Class B | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| SC RGB 444 | -31.8% | -31.7% | -32.0% | -31.1% | -31.2% | -31.3% | -30.6% | -30.5% | -30.5% |
| SC YUV 444 | -26.7% | -26.5% | -26.9% | -26.0% | -25.9% | -26.2% | -25.4% | -25.3% | -25.3% |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 128% |  |  | 129% |  |  | 130% |  |  |
| Dec Time[%] | 84% |  |  | 85% |  |  | 85% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6.2% | -6.6% | -6.4% | -5.5% | -5.7% | -5.8% |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -25.9% | -25.6% | -25.8% | -25.0% | -24.8% | -24.9% |  |  |  |
| SC YUV 444 | -22.9% | -23.1% | -23.4% | -22.5% | -22.6% | -22.7% |  |  |  |
| RangeExt | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 113% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.9% | -4.3% | -4.3% | -3.4% | -3.7% | -3.7% |  |  |  |
| Class B | 0.0% | -0.3% | -0.2% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -20.1% | -19.1% | -19.2% | -19.6% | -18.6% | -18.6% |  |  |  |
| SC YUV 444 | -17.5% | -17.0% | -17.2% | -17.3% | -16.6% | -16.8% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 110% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |  |  |  |

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.5% | 31.7 | 32.8 | -1.4% | 49.8 | 50.7 | -0.7% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 15.2 | -25.2% | 100.4 | 154.4 | -19.9% | 381.6 | 572.1 | -16.6% |
| SC YUV 444 | 11.4 | 18.2 | -20.7% | 128.9 | 204.0 | -18.9% | 325.6 | 524.4 | -17.0% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 134% |  |  | 110% |  |  | 107% |  |  |
| Dec Time[%] | 88% |  |  | 103% |  |  | 101% |  |  |

## Test results of limited motion with in LCU

In this test, the vertical motion is further limited so that displaced block does not go beyond LCU boundary. The following tables summarize BD-rate performance for the proposed method on lossy and lossless conditions.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -7.9% | -7.8% | -7.8% | -6.9% | -6.9% | -6.8% | -5.9% | -6.0% | -5.9% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -27.5% | -27.5% | -27.8% | -27.0% | -27.1% | -27.3% | -26.6% | -26.6% | -26.6% |
| SC YUV 444 | -24.1% | -24.0% | -24.3% | -23.5% | -23.6% | -23.7% | -23.0% | -23.0% | -22.9% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 127% |  |  | 129% |  |  | 130% |  |  |
| Dec Time[%] | 87% |  |  | 87% |  |  | 88% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -5.8% | -6.1% | -6.0% | -5.1% | -5.4% | -5.3% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -22.2% | -22.0% | -22.3% | -21.5% | -21.4% | -21.5% |  |  |  |
| SC YUV 444 | -20.5% | -20.7% | -21.0% | -20.2% | -20.2% | -20.4% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 112% |  |  | 113% |  |  |  |  |  |
| Dec Time[%] | 107% |  |  | 106% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -3.5% | -3.9% | -3.4% | -3.1% | -3.2% | -3.1% |  |  |  |
| Class B | 0.0% | -0.1% | -0.3% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -17.3% | -16.4% | -16.5% | -17.0% | -16.0% | -16.0% |  |  |  |
| SC YUV 444 | -15.8% | -15.3% | -15.5% | -15.9% | -15.4% | -15.5% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Enc Time[%] | 109% |  |  | 110% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 105% |  |  |  |  |  |

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -2.3% | 31.7 | 32.8 | -1.4% | 49.8 | 50.6 | -0.6% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.0% | 2.6 | 2.6 | 0.0% |
| SC RGB 444 | 10.1 | 14.1 | -21.5% | 100.4 | 143.1 | -17.0% | 381.6 | 555.6 | -14.8% |
| SC YUV 444 | 11.4 | 16.7 | -18.5% | 128.9 | 187.8 | -16.6% | 325.6 | 510.7 | -16.1% |
| RangeExt | 2.4 | 2.4 | 0.0% | 2.5 | 2.5 | 0.0% | 2.5 | 2.5 | 0.0% |
| Enc Time[%] | 132% |  |  | 109% |  |  | 107% |  |  |
| Dec Time[%] | 90% |  |  | 103% |  |  | 101% |  |  |

# Conclusion

The BD-rate of two limited motion configurations under lossy and lossless test condition from our tests matches the results provided by JCTVC-N0205.

# References

- D. Kwon, M. Budagavi, “RCE3: Results of test 3.3 on Intra motion compensation”, JCTVC-N0205, July 2013.

- L. Guo, “HEVC Range Extensions Core Experiment 3 (RCE 3): Intra Coding Methods for Screen Content”, JCTVC-M1123, 13th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Incheon, KR, 18-26 Apr., 2013.