| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0127r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0109r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On decoded picture buffer |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Informing the maximum decoded picture buffer (DPB) size for each layer set to decoders is beneficial for session negotiation. In this proposal, the maximum decoded picture buffer (DPB) size per a layer set is signalled in VPS extension for AU-level DPB management or for picture-level DPB management.

In VPS extension, two syntax elements layer_set_total_max_dec_pic_buffering_minus1[ i ] and max_num_direct_dependent_layers are signaled for AU-level DPB management, while layer_max_dec_pic_buffering_minus1[ i ][ j ] and max_direct_dependent_layer_of_layer_set[ i ] are signaled for picture-level DPB management. num_layers_of_layer_set[ i ] is signalled to avoid a parsing dependency from VPS to VPS extension.

# Introduction

Signalling the maximum decoded picture buffer (DPB) size for each layer set is beneficial for session negotiation. In this proposal, the maximum DPB size per a layer set is signalled in VPS extension for AU-level DPB management or picture-level DPB management. layer_set_total_max_dec_pic_buffering_minus1[ i ] is signaled for AU-level DPB management, while layer_max_dec_pic_buffering_minus1[ i ][ j ] is signaled for picture-level DPB management. layer_set_total_max_dec_pic_buffering_minus1[ i ] specify the maximum decoded picture buffer size for the i-th layer set. layer_max_dec_pic_buffering_minus1[ i ][ j ] specify the maximum decoded picture buffer size for the j-th layer of the i-th layer set. num_layers_of_layer_set[ i ] is signalled to avoid a parsing dependency from VPS to VPS extension.

Additionally, two syntax elements max_num_direct_dependent_layers and max_direct_dependent_layer_of_layer_set[ i ] are proposed. max_num_direct_dependent_layers specifies the maximum number of directly dependent layers. max_num_direct_dependent_layer_of_layer_set[ i ] specify the maximum number of directly dependent layers for the i-th layer set. Those syntax elements are beneficial for setting up the DPB buffers allocated for inter-layer prediction in a decoder.

# Proposal

| vps_extension( ) { | Descriptor |
| --- | --- |
| … | u(3) |
| max_num_direct_dependent_layers | u(6) |
| vps_number_layer_sets_minus1 | u(10) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) { |  |
| layer_set_total_max_dec_pic_buffering_minus1[ i ] | u(10) |
| max_num_direct_dependent_layer_of_layer_set[ i ] | u(6) |
| layer_level_dpb_size_signaling_flag | u(1) |
| if( layer_level_dbp_size_signaling_flag ) { |  |
| num_layers_of_layer_set[ i ] | u(6) |
| for( j = 0; j < num_layer_of_layer_set[ i ]; j++ ) |  |
| layer_max_dec_pic_buffering_minus1[ i ][ j ] | u(7) |
| } |  |
| } |  |
| … |  |
| } |  |

max_num_direct_dependent_layers specifies the maximum number of directly dependent layers. NumDirectRefLayers[ i ] shall be equal to or less than max_direct_dependent_layers.

layer_set_total_max_dec_pic_buffering_minus1[ i ] specify the maximum decoded picture buffer size for the i-th layer set.

max_num_direct_dependent_layer_of_layer_set[ i ] specify the maximum number of directly dependent layers for the i-th layer set.

layer_level_dpb_size_signaling_flag indicates whether the decoded picture buffer size of each layer set in layer-level. layer_level_dpb_size_signaling_flag equal to 1 indicates that num_layer_of_layer_set[ i ] and layer_max_dec_pic_buffering_minus1[ i ][ j ] are present.

num_layers_of_layer_set[ i ] specify the total number of layers for the i-th layer set.

layer_max_dec_pic_buffering_minus1[ i ][ j ] specify the maximum decoded picture buffer size for the j-th layer of the i-th layer set.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).