| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0128 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0110 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Reference picture marking and picture removal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Two proposals are presented in this contribution.

First, the marking and the picture removal of the re-sampled reference pictures and the decoded pictures used for up-scaling are proposed. The re-sampled reference pictures are marked as “not needed for output” by default, and marked as “unused for reference” immediately after used for inter-layer prediction. The decoded pictures with the original spatial resolution are marked as “unused for reference” immediately after the re-sampled reference pictures are generated, if those pictures are not used for inter-prediction by the following pictures.

Second, the picture removal process from a decoded picture buffer (DPB) is proposed. When the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB except the pictures belonging to the same access unit are emptied. The purpose of the second item is to avoid the removal of inter-layer reference pictures before inter-layer prediction.

# Introduction

In this proposal, the marking and the picture removal of the re-sampled reference pictures and the decoded pictures used for up-scaling are proposed. The re-sampled reference pictures are marked as “not needed for output” by default, and marked as “unused for reference” immediately after used for inter-layer prediction. Hence, the re-sampled reference pictures are removed after inter-layer prediction is done. The decoded pictures with the original spatial resolution are marked as “unused for reference” immediately after the re-sampled reference pictures are generated, if those pictures are not used for inter-prediction by the following pictures.

In addition, the picture removal process from a decoded picture buffer (DPB) is proposed. If a DBP is operated in AU-level, all picture storage buffers in the DPB except the pictures belonging to the same access unit are emptied, when the value of NoOutputOfPriorPicsFlag is equal to 1. If a DBP is operated in Picture-level, all pictures with nuh_layer_id equal to currLayerId are removed from a DPB, when the value of NoOutputOfPriorPicsFlag is equal to 1. It is because if all picture buffers in a DPB are emptied when the value of NoOutputOfPriorPicsFlag is equal to 1, the inter-layer prediction reference pictures are removed before they are used for inter-layer prediction. Another way to resolve this problem is to force that NoOutputOfPriorPicsFlag can be equal to 1 only when nuh_layer_id is equal to 0. However, this constraint is not desirable for a layer-level random access or a picture-level DPB management.

# Proposed text for marking rules for re-sampled reference pictures

- G.8.1.2 Decoding process for inter-layer reference picture set

Output of this process is an updated list of inter-layer reference pictures RefPicSetInterLayer.

The variable currLayerId is set equal to nuh_layer_id of the current decoded picture

The list RefPicSetInterLayer is first emptied and then derived as follows.

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to RefPicLayerId[ i ] ) {  
		an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and   DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] given as inputs
		RefPicSetInterLayer[ i ] = rsPic 
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

If picX is not used for inter-prediction in the following pictures, picX is marked as “unused for reference” immediately after the resampled reference picture rsPic is generated.

There shall be no entry equal to "no reference picture" in RefPicSetInterLayer.

If the current picture is a RADL picture, there shall be no entry in the RefPicSetInterLayer that is a RASL picture.

NOTE –  An access unit may contain both RASL and RADL pictures.

- G. 8. 1. 3 Marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0

Output of this process is:

–	a potentially updated marking as "used for short-term reference" for some decoded pictures.

The following applies.

for( i = 0; i <  NumActiveRefLayerPics; i++ ) 
	RefPicSetInterLayer[ i ] is marked as "not needed for output" and "unused for reference"

All the resampled inter-layer reference pictures are emptied without output after used for inter-layer prediction.

# Proposed text for picture removal from DPB

- F.13.3.2  Removal of pictures from the DPB

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of the picture n and proceeds as follows:

–	The decoding process for RPS as specified in subclause Error: Reference source not found is invoked.

–	When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD, such that when the value of NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB except the pictures belonging to the same access unit are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	When both of the following conditions are true for any pictures k in the DPB, all such pictures k in the DPB are removed from the DPB:

–	picture k is marked as "unused for reference"

–	picture k has PicOutputFlag equal to 0 or its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e. DpbOutputTime[ k ] is less than or equal to CpbRemovalTime( m )

–	For each picture that is removed from the DPB, the DPB fullness is decremented by one.

- F.13.5.2.2 Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the current picture is removed from the CPB and proceeds as follows:

The decoding process for RPS as specified in subclause 8.3.2 is invoked.

–	If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and with nuh_layer_id equal to 0 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, or sps_max_dec_pic_buffering_minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

2.	The value of NoOutputOfPriorPicsFlag derived for the decoder under test is applied for the HRD as follows:

–	If NoOutputOfPriorPicsFlag is equal to 1, all picture storage buffers in the DPB except the pictures belonging to the same access unit are emptied without output of the pictures they contain, and the DPB fullness is set equal to 0.

–	Otherwise (NoOutputOfPriorPicsFlag is equal to 0), all picture storage buffers  containing a picture that is marked as "not needed for output" and "unused for reference" are emptied (without output), and all non-empty picture storage buffers in the DPB are emptied by repeatedly invoking the "bumping" process specified in subclause Error: Reference source not found, and the DPB fullness is set equal to 0.

–	Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1 or with nuh_layer_id not equal to 0), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. The variable currLayerId is set equal to nuh_layer_id of the current decoded picture and when one or more of the following conditions are true, the "bumping" process specified in subclause Error: Reference source not found is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB that are marked as "needed for output" is greater than sps_max_num_reorder_pics[ HighestTid ] from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId (when currLayerId is not equal to 0). [Ed. (CY): to simplify the text for extensions, it is better to use a consistent term of active SPS for “active SPS” and “active layer SPS”. This might require we editorially modify the version 1 spec., specifying only one active SPS is allowed for nuh_layer_id equal to 0 of a coded video sequence. Similar change may be done for PPS.]

–	sps_max_latency_increase_plus1[ HighestTid ] of the active SPS (when currLayerId is equal to 0) or the active layer SPS for the value of currLayerId is not equal to 0 and there is at least one picture with nuh_layer_id equal to currLayerId in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount[ currLayerId ] is greater than or equal to SpsMaxLatencyPictures[ HighestTid ] derived from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

–	The number of pictures with nuh_layer_id equal to currLayerId in the DPB is greater than or equal to sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1 from the active SPS (when currLayerId is equal to 0) or from the active layer SPS for the value of currLayerId.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).