| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0129r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0111r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The syntax single_layer_for_non_irap_flag can be used for enabling a single-loop-like decoding, because decoding all non-IRAP pictures needs not decoding of dependent layers, even if the flag is adopted for the adaptive resolution change and the trick mode. For single-loop-like decoding, the following syntax modifications are proposed.

- The semantics modification of single_layer_for_non_irap_flag to generalize the functionality. In the proposed text, more than two layers are allowed in IRAP access units.

- The syntax elements max_tid_il_ref_pics_plus1[ i ] and default_one_target_output_layer_flag are optionally signalled according to the value of single_layer_for_non_irap_flag.

- Signalling of inter-RPS syntax can be skipped when single_layer_for_non_irap_flag is equal to 1 and the current picture is not an IRAP picture.

- Introduction

This contribution proposes the syntax and semantics modification related to the syntax element single_layer_for_non_irap_flag for single-loop-like decoding. When single_layer_for_non_irap_flag is equal to 1, IRAP access units or pictures may have multiple layers while non-IRAP access units or pictures shall have a single layer, in the proposed text (highlighted in Green).

In addition, The syntax elements max_tid_il_ref_pics_plus1[ i ] and default_one_target_output_layer_flag are optionally signalled according to the value of single_layer_for_non_irap_flag. When single_layer_for_non_irap_flag is equal to 1, max_tid_il_ref_pics_plus1[ i ] and default_one_target_output_layer_flag is not signalled (highlighted in Sky-blue).

When single_layer_for_non_irap_flag is equal to 0 and the current picture is non-IRAP picture, singaling syntaxes related to inter-layer reference picture set can be skipped. (highlighted in Yellow)

# Proposed specification text

## Video parameter set extension syntax and semantics

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_layer_for_non_irap_flag | u(1) |
| if(!single_layer_for_non_irap_flag && ! inter_layer_prediction_disabled_for_non_irap_flag) { |  |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| } |  |
| ... |  |
| if( numOutputLayerSets > 1 && !single_layer_for_non_irap_flag ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| … |  |
| single_layer_for_non_irap_flag | u(1) |
| } |  |

single_layer_for_non_irap_flag equal to 1 indicates that all the VCL NAL units of an non-IRAP access unit have the same nuh_layer_id value, and one or more nuh_layer_id values are used by the VCL NAL units of an IRAP access unit. single_layer_for_non_irap_flag equal to 0 indicates that nuh_layer_id values may or may not be constrained beyond constraints specified in other parts of this Recommendation | International Standard.

default_one_target_output_layer_flag equal to 1 specifies that only the highest layer in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers. When not present, default_one_target_output_layer_flag is inferred to be equal to 1.

## Slice header syntax and semantics

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !single_layer_for_non_irap_flag ¦¦ (nal_unit_type>15 && nal_unit_type<24)) { |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| } |  |
| … |  |
| } |  |

- Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).