| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0129 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0111 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On single layer for non-IRAP pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The syntax and semantics modification related to the syntax element single_layer_for_non_irap_flag and a new syntax inter_layer_prediction_disabled_for_non_irap_flag are proposed for single loop decoding of non-IRAP pictures in HEVC multi-layered extensions. When single_layer_for_non_irap_flag is equal to 1, IRAP access units or pictures may have multiple layers while non-IRAP access units or pictures shall have a single layer, in the proposed text. In addition, the syntaxes that can be inferred without signalling when single_layer_for_non_irap_flag is equal to 1 are optionally signaled according to the value of single_layer_for_non_irap_flag. The proposed syntax inter_layer_prediction_disabled_for_non_irap_flag indicates that all non-IRAP pictures in output layer sets can be decoded in a single loop.

# Introduction

This contribution proposes the syntax and semantics modification related to the syntax element single_layer_for_non_irap_flag for single loop decoding of HEVC multi-layered extension. When single_layer_for_non_irap_flag is equal to 1, IRAP access units or pictures may have multiple layers while non-IRAP access units or pictures shall have a single layer, in the proposed text. In addition, the syntaxes that can be inferred without signalling when single_layer_for_non_irap_flag is equal to 1 are optionally signaled according to the value of single_layer_for_non_irap_flag. In addition, a new syntax inter_layer_prediction_disabled_for_non_irap_flag is proposed. When inter_layer_prediction_disabled_for_non_irap_flag is equal to 1, the inter-layer prediction is disallowed in non-IRAP pictures. It indicates that all non-IRAP pictures in output layer sets can be decoded in a single loop.

# Proposed specification text

## Video parameter set extension syntax and semantics

When single_layer_for_non_irap_flag is equal to 0 and the current picture is non-IRAP picture, singaling syntaxes max_tid_il_ref_pics_plus1[ i ] or default_one_target_output_layer_flag can be optionally skipped. The proposed flag inter_layer_prediction_disabled_for_non_irap_flag indicates that all non-IRAP pictures can be decoded in a single loop.

| vps_extension( ) { | Descriptor |
| --- | --- |
| ... |  |
| single_layer_for_non_irap_flag | u(1) |
| inter_layer_prediction_disabled_for_non_irap_flag | u(1) |
| if(!single_layer_for_non_irap_flag && ! inter_layer_prediction_disabled_for_non_irap_flag) { |  |
| for( i = 0; i< vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| } |  |
| ... |  |
| if( numOutputLayerSets > 1 && !single_layer_for_non_irap_flag ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i < numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| max_one_active_ref_layer_flag | u(1) |
| … |  |
| } |  |

single_layer_for_non_irap_flag equal to 1 indicates that all the VCL NAL units of an non-IRAP access unit have the same nuh_layer_id value, and one or more nuh_layer_id values are used by the VCL NAL units of an IRAP access unit. single_layer_for_non_irap_flag equal to 0 indicates that nuh_layer_id values may or may not be constrained beyond constraints specified in other parts of this Recommendation | International Standard.

inter_layer_prediction_disabled_for_non_irap_flag equal to 1 indicates that all non-IRAP pictures in non-IRAP access unit cannot be predicted from a decoded picture with nuh_layer_id other than the current nuh_layer_id.

default_one_target_output_layer_flag equal to 1 specifies that only the highest layer in each of the default output layer sets is a target output layer. default_one_target_output_layer_flag equal to 0 specifies that all layers in each of the default output layer sets are target output layers. When not present, default_one_target_output_layer_flag is inferred to be equal to 1.

## Slice header syntax and semantics

When single_layer_for_non_irap_flag is equal to 0 and the current picture is non-IRAP picture, singaling syntaxes related to inter-layer reference picture set can be skipped.

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !single_layer_for_non_irap_flag ¦¦ (nal_unit_type>15 && nal_unit_type<24)) { |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( NumSamplePredRefLayers[ nuh_layer_id ] > 0 && NumActiveRefLayerPics > 0 ) |  |
| inter_layer_sample_pred_only_flag | u(1) |
| } |  |
| … |  |
| } |  |

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).