| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0130r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0112r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On temporal sub-layer management |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In HEVC layered-extension, several constraints are proposed to allow different frame rates for different layers. In addition, if frame rates for non-base layers are greater than a frame rate of a base layer, the maximum number of sub-layers in non-base layers can be greater than the maximum number of sub-layers in the base layer. Then, it is asserted that signalling the increased number of sub-layers, profile-tier-level information and sub-layer ordering information for additional sub-layers of non-base layer in video parameter set extension is needed.

- Definition of sub-access unit in which nuh_layer_ids of all VCL NAL units are not equal to 0 is suggested to explaing the following proposals.

- The TemporalId values of all pictures in an access unit or a sub-AU shall be the same.

- The TemporalId value of a picture in a sub-AU shall be greater than the TemporalId value of a picture in an AU that has a picture with nuh_layer_id equal to 0.

- A picture belonging to a sub-AU shall be a TSA picture, an STSA picture or a trailing picture.

- The increased number of sub-layers, profile-tier-level information and sub-layer ordering information for additional sub-layers of non-base layer are signalled in video parameter set extension.

- sps_max_sub_layers_minus1 and profile_tier_level( 1, sps_max_sub_layers_minus1 ) are signaled in an SPS with nuh_layer_id greater than 0 to allow different frame rates for each layer.

# Definitions

Assuming that an access unit may not have a picture with nuh_layer_id equal to 0 when temporal rates of non-base layers are greater than a temporal rate of a base layer, a sub-access unit is define as follows:

sub-access unit (sub-AU): An access unit in which nuh_layer_ids of all VCL NAL units are not equal to 0.

# Proposed constraints

In HEVC layered-extension, in order to support different frame rates for different layers, the following constraints are proposed.

- The TemporalId values of all pictures in an access unit or a sub-AU shall be the same.

- The nuh_temporal_id_plus1 values of VCL NAL units belonging to an access unit or a sub-AU shall be the same.

- The TemporalId value of a picture in a sub-AU shall be greater than the TemporalId value of a picture in an AU that has a picture with nuh_layer_id equal to 0.

- The TemporalId value of a VCL NAL unit belonging to a sub-AU shall be greater than the TemporalId value of a VCL NAL unit belonging to an AU that has a VCL NAL unit with nuh_layer_id equal to 0.

- An sub-AU boundary can be detected from the difference of TemporalId values between VCL NAL units belonging to a sub-AU and VCL NAL units belonging to an AU containing a VCL NAL unit with nuh_layer_id equal to 0.

- A picture belonging to a sub-AU shall be a TSA picture, an STSA picture or a trailing picture.

# Signalling additional sub-layer related information in VPS extension

If frame rates for non-base layers are greater than a frame rate of a base layer, the maximum number of sub-layers in non-base layers can be greater than the maximum number of sub-layers in the base layer. Then, it is asserted that signalling the increased number of sub-layers, profile-tier-level information and sub-layer ordering information for additional sub-layers of non-base layer in video parameter set extension is needed.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| vps_max_ext_sub_layers | u(3) |
| if(vps_max_ext_sub_layers>0) { |  |
| ext_profile_tier_level( 1, vps_max_ext_sub_layers-1) |  |
| vps_ext_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( vps_ext_sub_layer_ordering_info_present_flag ? 0 : vps_ext_max_ext_sub_layers ); i < vps_max_ext_sub_layers; i++ ) { |  |
| vps_max_ext_dec_pic_buffering_minus1[ i ] | ue(v) |
| vps_max_ext_num_reorder_pics[ i ] | ue(v) |
| vps_max_ext_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| ext_profile_tier_level( extNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| for( i = 0; i < extNumSubLayersMinus1 ; i++ ) { |  |
| sub_layer_profile_present_flag[ i ] | u(1) |
| sub_layer_level_present_flag[ i ] | u(1) |
| } |  |
| if(extNumSubLayersMinus1 > 0 ) |  |
| for( i = extNumSubLayersMinus1 ; i < 8; i++ ) |  |
| reserved_zero_2bits[ i ] | u(2) |
| for( i = 0; i < extNumSubLayersMinus1 ; i++ ) { |  |
| if( sub_layer_profile_present_flag[ i ] ) { |  |
| sub_layer_profile_space[ i ] | u(2) |
| sub_layer_tier_flag[ i ] | u(1) |
| sub_layer_profile_idc[ i ] | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| sub_layer_profile_compatibility_flag[ i ][ j ] | u(1) |
| sub_layer_progressive_source_flag[ i ] | u(1) |
| sub_layer_interlaced_source_flag[ i ] | u(1) |
| sub_layer_non_packed_constraint_flag[ i ] | u(1) |
| sub_layer_frame_only_constraint_flag[ i ] | u(1) |
| sub_layer_reserved_zero_44bits[ i ] | u(44) |
| } |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

vps_max_ext_sub_layers plus 1 specifies the extended number of temporal sub-layers that are present in non-base layers, however not present in a base layer. The value of vps_max_ext_sub_layers plus vps_max_sub_layers plus 1 shall be in the range of 0 to 6, inclusive.

- vps_ext_sub_layer_ordering_info_present_flag equal to 1 specifies that vps_ext_max_dec_pic_buffering_minus1[ i ], vps_ext_max_num_reorder_pics[ i ], and vps_ ext_max_latency_increase_plus1[ i ] are present for vps_max_ext_sub_layers sub-layers. vps_ext_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of vps_max_dec_pic_buffering_minus1[ vps_max_sub_layers_minus1 ], vps_max_num_reorder_pics[ vps_max_sub_layers_minus1 ], and vps_max_latency_increase_plus1[ vps_max_sub_layers_minus1 ] apply to all extended sub-layers.

- vps_max_ext_dec_pic_buffering_minus1[ i ] plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers when HighestTid is equal to i + vps_max_ext_sub_layers. The value of vps_max_dec_pic_buffering_minus1[ i ] shall be in the range of 0 to MaxDpbSize − 1, inclusive. When i is greater than 0, vps_max_ext_dec_pic_buffering_minus1[ i ] shall be greater than or equal to vps_max_ext_dec_pic_buffering_minus1[ i − 1 ]. When vps_max_ext_dec_pic_buffering_minus1[ i ] is not present for i in the range of 0 to vps_max_ext_sub_layers, inclusive, due to vps_ext_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_dec_pic_buffering_minus1[ vps_max_sub_layers_minus1 ].

- vps_max_ext_num_reorder_pics[ i ] indicates the maximum allowed number of pictures that can precede any picture in the CVS in decoding order and follow that picture in output order when HighestTid is equal to i + vps_max_ext_sub_layers. The value of vps_max_ext_num_reorder_pics[ i ] shall be in the range of 0 to vps_max_ext_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, vps_max_ext_num_reorder_pics[ i ] shall be greater than or equal to vps_max_ext_num_reorder_pics[ i − 1 ]. When vps_max_ext_num_reorder_pics[ i ] is not present for i in the range of 0 to vps_max_ext_sub_layers, inclusive, due to vps_ext_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_num_reorder_pics[ vps_max_sub_layers_minus1 ].

- vps_max_ext_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of VpsMaxLatencyPictures[ i + vps_max_ext_sub_layers ], which specifies the maximum number of pictures that can precede any picture in the CVS in output order and follow that picture in decoding order when HighestTid is equal to i + vps_max_ext_sub_layers.

- When vps_max_ext_latency_increase_plus1[ i ] is not equal to 0, the value of VpsMaxLatencyPictures[ i ] is specified as follows:

VpsMaxLatencyPictures[ i + vps_max_ext_sub_layers] = vps_max_ext_num_reorder_pics[ i ] + vps_max_ext_latency_increase_plus1[ i ] − 1

- When vps_max_ext_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

The value of vps_max_ext_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_ext_latency_increase_plus1[ i ] is not present for i in the range of 0 to vps_max_ext_sub_layers, inclusive, due to vps_ext_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to vps_max_latency_increase_plus1[ vps_max_sub_layers_minus1 ].

# A minor modification of SPS

sps_max_sub_layers_minus1 and profile_tier_level( 1, sps_max_sub_layers_minus1 ) are signaled in an SPS with nuh_layer_id greater than 0 to allow different frame rates.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| if( nuh_layer_id = = 0 ) |  |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| … |  |
| } |  |

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).