| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0131r2 |
| --- | --- |

| Title: | SHVC HLS: On interlayer reference picture set |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | yongjin9.cho@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This proposal is a follow-up proposal of JCTVC-N0131 that addresses the comment in the joint HLS BoG report at the JCT3V session. The proposed inter-layer reference picture initialization process in JCTVC-N0131 was agreed in the joint HLS BoG [1], but there was a comment that the proposed initialization process is not efficient for multiview scalability. To address the comment, we propose the inter-layer reference picture list initialization be applied only to SHVC, because an initialization process only for multiview inter-layer reference pictures has been adopted in this meeting [2]. This document proposes to have inter-layer reference picture lists for SHVC be initialized in a descending order such that reference pictures of better quality come at the head of the inter-layer reference list.

# Introduction

Tables 1 shows the current decoding process for inter-layer picture set. With the current decoding process, the inter-layer reference picture list is constructed in an ascending order of nuh_layer_id. Then, the best inter-layer reference picture is put at the very last of the inter-layer reference picture list, which would result in worse coding performance than having the list in a descending order. Hence, we propose to initialize the inter-layer reference picture list in a descending order of nuh_layer_id such that a better coding efficiency could be expected.

- G.8.1.2 Decoding process for inter-layer reference picture set

for( i = 0; i < NumActiveRefLayerPics; i++ ) {
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to RefPicLayerId[ i ] ) {  
		an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and   DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] given as inputs
		RefPicSetInterLayer[ i ] = rsPic 
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

Table 1. Current inter-layer reference picture set decoding process ([3])

# Proposal

We propose to initialize a reference picture set in such a way that closer inter-layer reference pictures to the current layer are listed before farther inter-layer reference pictures in each reference picture list – i.e., in an descending order of nuh_layer_id. This follows the convention of placing closer reference pictures in reference picture lists in the version 1 HEVC and legacy H.264 specifications. The proposed change to the decoding process for inter-layer reference picture set is shown in Table 2 (highlighted in yellow).

- G.8.1.2 Decoding process for inter-layer reference picture set

for( i = NumActiveRefLayerPics-1; i >= 0; i-- )
	if( there is a picture picX in the DPB that is in the same access unit as the current picture and has
			nuh_layer_id equal to RefPicLayerId[ i ] ) {  
		an interlayer reference picture rsPic is derived by invoking the subclause Error: Reference source not found with picX and   DirectRefLayerIdx[ currLayerId ][ RefPicLayerId[ i ] ] given as inputs
		RefPicSetInterLayer[ i ] = rsPic 
		RefPicSetInterLayer[ i ] is marked as "used for long-term reference"
	} else
		RefPicSetInterLayer[ i ] = "no reference picture"
}

Table 2. Proposed inter-layer reference picture list decoding process

# References

- J. Boyce, “BoG report on SHVC/MV-HEVC HLS topics,” Document JCTVC-N0374, the 14th JCTVC meeting, Vienna, AT, Jul. 2013

- A. Norkin et al., “MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction,” Document JCT3V-E0239, the 5th JCT3V meeting, Vienna, AT, Jul. 2013

- J. Chen et. al., “SHVC Working Draft 2,” Document JCTVC-M1008, the 13th JCTVC meeting, Inchon, KR, Apr. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).