| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0133 |
| --- | --- |

| Title: | HLS: Frame packing arrangement SEI message for bit depth extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

A minor change of frame packing arrangement (FPA) SEI message is proposed for delivering extended bit-depth data up to 14 bits in nominally 8bit bitstreams. Two layered bitstreams are carried with FPA SEI messages. One constituent picture can be decoded compatibly as an ordinary 8 bit image, or can be decoded an extended bits (10~14bits) image with a supplementary picture data, which are differential data between the original data and the coded 8 bits data, in a scalable manner.

# Proposal

A minor change of frame packing arrangement (FPA) SEI message is proposed for delivering extended bit-depth data up to 14 bits in nominally 8bit bitstreams. Since 8 bit en/decoder are the most widely spread in products, an intermediate solution to convey wide bits content through such en/decoders can promote high-quality video services, e.g. high quality movie contents.

The figure 1 illustrates conceptually how to arrange frame packed data to carry extended bit-depth data with a conventional 8 bits en/decoder. The fundamental idea is that the first frame is the coded 8-bits data and the secondary frame is the differential data between the original data and the coded 8-bits data. Two ways packing both the coded 8-bit picture (primary picture) and the differential picture (supplementary picture) are proposed. One is interleaving the primary pictures and the secondary picture in frame-level. The other one is packing the half resolution of the primary pictures and the secondary pictures into a frame in side-by-side (or top-bottom).

Encoding    Differential data          Original data            Coded 8 MSBs 
process      (supplementary)                                   (primary)

Decoding   Reconstructed data       Decoded 8 MSBs         Differential data 
process      (extended bits)           ( base 8bits )            (supplementary)

Figure 1. Extended bits delivery with two-layered frame packing arrangement.

Y12bits is Original Luma data (Assuming original data is 12 bits)

Y8MSBs is Most significant 8 bits of Y12bits

Y’8MSBs is Reconstructed Y8MSBs (with quantization error)

Ydiff-8bits is Differential data, which is calculated as follows:

Ydiff-8bits = clip8( Y12bits – (Y’8MSBs << N) )

Note: N is the bit-depth of original data minus 8. Here, N is equal to 4.

Ydiff-8bits is encoded and decoded as a supplementary data,

while Y8MSBs is Most significant 8 bits of Y16bits as a primary data.

The decoded image Y’12bits is reconstructed as follows:

Y’12bits = ( Y’8MSBs << N )+ Y’diff-8bits

where Y’diff-8bits  is the decoded data of Ydiff-8bits.

Preliminary simulation results of differential coding for the extended bit-depth are shown in JCTVC-N0218, which are tested with 10bits sequences. The total overhead is not significant.

# Proposed specification text

content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table D-9.

Table D-9–Definition of content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the frame packed constituent frames |
| 1 | Indicates that the two constituent frames form the left and right views of a stereo view scene, with frame 0 being associated with the left view and frame 1 being associated with the right view |
| 2 | Indicates that the two constituent frames form the right and left views of a stereo view scene, with frame 0 being associated with the right view and frame 1 being associated with the left view |
| 3 | Indicates that the two constituent frames form the main and difference views(8 bits frames) representing a 10, 12, 14 bits frame, with frame 0 being associated with the main view and frame 1 being associated with the difference view between original 10, 12, 14 bits view and the 8bits main view. |

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).