| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0134 |
| --- | --- |

| Title: | HLS: Frame packing arrangement SEI message for full resolution 3D |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

A minor change of frame packing arrangement (FPA) SEI message is proposed. By adding a flag, which indicates whether the current video sequence is the primary half resolution 3D data or the complementary (secondary) data, the full resolution 3D video as well as the half resolution 3D video can be delivered in a scalable manner, with the proposed FPA SEI message.

# Introduction

This contribution is a follow-up proposal of JCTVC-L0293. It is necessary to provide stereoscopic 3D video service supporting the compatibility with 2D video system, before the professional 3D CODEC is feasible. The H.264/AVC based frame packing 3D systems have been successfully used in Cable TV systems. HEVC-based frame packing 3D can be also used intermediately, before the HEVC-based 3D CODECs (MV-HEVC or 3D-HEVC) are fully feasible. The limitation of the frame packing 3D like side-by-side is the visual quality of decoded frames. By sub-sampling a left image and a right image into a single frame, a half resolution 3D image is reconstructed at decoder, and up-scaled for display, which may provide unsatisfactory visual quality in high-end 3D display.

In this proposal, we propose a minor change of the FPA SEI message to enable the 3D video delivery in full resolution as well as half resolution. By delivering two simulcast bitstreams, which are frame-packed with a primary half resolution 3D video or a complementary (secondary) half resolution 3D video, respectively. If both the primary bitstreams and the secondary bitstreams are received, the full resolution 3D video can be reconstructed. If only the primary bitstreams is received, a half resolution 3D video can be reconstructed. This mechanism enables to deliver 3D video data in a scalable manner.

# Proposal

The figure 1 illustrates conceptually how to arrange two video sequences to carry full resolution 3D video with the FPS SEI message. When a primary half resolution 3D data is frame-packed and coded into a bitstream, a complementary (secondary) half resolution 3D data can be frame-packed and coded into another bitstream. Two bitstreams can be transmitted in simulcast, or conveyed in an elementary stream. While the half resolution 3D can be decoded and displayed when only the primary bitstream is delivered, if both primary and secondary frame-packed bitstreams are delivered, the full resolution 3D video can be reconstructed by rearranging two decoded frames, as shown in Figure 1.

Figure 1. Full resolution 3D delivery mechanism with FPS SEI messages.

# Proposed specification text

content_interpretation_type indicates the intended interpretation of the constituent frames as specified in Table D-9.

Table D-9–Definition of content_interpretation_type

| Value | Interpretation |
| --- | --- |
| 0 | Unspecified relationship between the frame packed constituent frames |
| 1 | Indicates that the two constituent frames form the left and right views of a stereo view scene, with frame 0 being associated with the left view and frame 1 being associated with the right view |
| 2 | Indicates that the two constituent frames form the right and left views of a stereo view scene, with frame 0 being associated with the right view and frame 1 being associated with the left view |
| 3 | Indicates that the two constituent frames form the left and right complementary views of a stereo view scene, with frame 0 being associated with the complementary left view and frame 1 being associated with the complementary right view |
| 4 | Indicates that the two constituent frames form the right and left complementary views of a stereo view scene, with frame 0 being associated with the complementary right view and frame 1 being associated with the complementary left view |

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).