| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0137 |
| --- | --- |

| Title: | Non-RCE2: Golomb-rice parameter initialization for transform-skip and transquant-bypass modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | V. Kolesnikov, C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 vladimir.kolesnikov@cisra.canon.com.au chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

RCE2 of the 14th JCT-VC meeting includes a number of sub-tests relating to entropy coding of the residual for transform skip and transquant bypass blocks. In particular, the update formula for the golomb-rice parameter is modified, which is applied once per instance of coef_abs_level_remaining (up to 16 instances per sub-block).  This contribution proposes to modify the initialisation of the golomb-rice parameter, which is applied once per sub-block.

# Introduction

During the Incheon meeting proposals were submitted to address the issue of efficient entropy modelling of mixed content. In proposals [1] and [2] the Golomb-Rice parameter (cRiceParam) update mechanism was modified for the transform-skip and transquant-bypass modes.

This contribution proposes a different approach whereby the initial value is modified for this blocks, while the existing update mechanism is retained.

# Description of the proposed change

This contribution proposes to use the following formula to initialize cRiceParam prior to coding each sub-block:

cRiceParam = 2, if any of transform-skip or transquant-bypass modes is active and the number of greater-than-one coefficients in the current sub-set is equal to or greater than 8;

cRiceParam = 0, otherwise.

The condition on the number of greater-than-one coefficients is already present in the design of residual coefficient coding (for the ‘high throughput binarisation’ of coeff_abs_level_greater1_flag) and therefore the existing condition check may be used for this new purpose.

# Experiment Results

Table 3.1 shows results for RCE2 [3] Lossless test conditions.

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 5.2 | 5.3 | -0.6% | 31.7 | 31.8 | -0.2% | 49.8 | 49.9 | -0.1% |
| Class B | 2.2 | 2.2 | 0.0% | 2.6 | 2.6 | 0.1% | 2.6 | 2.6 | 0.1% |
| SC RGB 444 | 10.1 | 10.2 | -1.2% | 100.4 | 101.4 | -0.8% | 381.6 | 386.3 | -0.8% |
| SC YUV 444 | 11.4 | 11.5 | -0.9% | 128.9 | 129.7 | -0.3% | 325.6 | 329.2 | -0.6% |
| RangeExt | 2.4 | 2.4 | -2.3% | 2.5 | 2.5 | -1.7% | 2.5 | 2.5 | -1.7% |
| Enc Time[%] | 93% |  |  | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  | 100% |  |  |

Table 3.2 shows results for RCE2 Lossy test conditions.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.3% | -0.3% | -0.3% | -0.6% | -0.7% | -0.6% | -1.0% | -1.0% | -1.0% |
| SC YUV 444 | 0.0% | -0.1% | 0.0% | -0.2% | -0.2% | -0.2% | -0.4% | -0.5% | -0.5% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.3% | -0.4% | -0.3% | -0.6% | -0.6% | -0.6% |  |  |  |
| SC YUV 444 | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.2% | -0.1% | -0.1% | 0.1% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.1% | 0.1% | 0.1% | -0.5% | -0.4% | -0.5% |  |  |  |
| SC YUV 444 | 0.1% | 0.4% | 0.3% | 0.1% | 0.2% | 0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  | Reference | Tested |  | Reference | Tested |  |
| Class F | 3.7 | 3.7 | -2.4% | 21.5 | 21.8 | -1.4% | 33.6 | 33.9 | -1.3% |
| Class B | 1.5 | 1.5 | -3.1% | 1.6 | 1.6 | -1.9% | 1.6 | 1.6 | -2.0% |
| SC RGB 444 | 7.9 | 8.0 | -1.6% | 79.1 | 80.1 | -1.4% | 307.4 | 310.6 | -1.6% |
| SC YUV 444 | 8.6 | 8.8 | -1.7% | 96.1 | 97.6 | -1.4% | 250.1 | 253.5 | -1.3% |
| RangeExt | 2.4 | 2.4 | -2.3% | 2.5 | 2.5 | -1.7% | 2.5 | 2.5 | -1.7% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  | 100% |  |  |

Table 3.3 shows results for non-RCE2 Lossless 10-bit test conditions (these are the lossless test conditions of RCE2 but with internal bit-depth set to 10).

# Working Draft changes

#### 9.3.3.9 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, baseLevel, a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture and a variable cIdx specifying the colour component of the current block.

Output of this process is the binarization of the syntax element.

The variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block scan index i, cLastAbsLevel is set equal to 0 and cRiceParam is derived as follows:

  - If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0 and cu_transquant_bypass_flag         is equal to 0 and general_profile_idc is less than 4 then cRiceParam is set equal to 0;

  - Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 or cu_transquant_bypass_flag         is equal to 1 or general_profile_idc is greater than 3) cRiceParam is derived as follows:

- If numGreater1Flag is less than 8 then cRiceParam is set equal to 0;

- Otherwise (numGreater1Flag is not less than 8) cRiceParam is set equal to 2.

- Otherwise (this process is not invoked for the first time for the current sub-block scan index i), cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

# Conclusion

This contribution presents an approach to the Golomb-Rice parameter adaptivity whereby the initialisation is modified for transform skip and transquant bypass blocks, while leaving the update mechanism unchanged.

# References

- M. Budagavi “AHG8: Coefficient level cRiceParam updates for screen content coding”, Joint Collaborative Team on Video Coding, JCTVC-M0316, Incheon, KR, April 2013.

- J. Sole, M. Karczewicz “AhG8: Simplified update of the coefficient level Rice parameter”, Joint Collaborative Team on Video Coding, JCTVC-M0366, Incheon, KR, April 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, Joint Collaborative Team on Video Coding, JCTVC-M1122, Incheon, KR, April 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).