| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0139 |
| --- | --- |

| Title: | SCE2: Results on test 2.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Patrice Onno Edouard François Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the results for SCE2 test 2.1 on position derivation when using Base Layer motion information. The proposed method modifies the rounding of the colocated position in the Base Layer, when used for motion derivation. The average luma EL+BL BD-rate with SHM 2.0 anchors is reported to be: -0.2% for RA 2x, -0.1% for LD-B 2x and -0.1% for LD-P 2x, the other ratios showing no noticeable gain.

# Introduction

## Rationale

Figure 1: motion derivation and access to compressed motion buffer of Base Layer

Currently, there are 2 approaches in SHVC to reuse motion information from a Base Layer by looking up the motion at the colocated position in the Base Layer of:

- For Reference frame index based approach, section G.8.1.4.2.1, “Derivation process for inter layer motion”: the center of an Enhancement Layer 16x16 block;

- For TextureRL approach: section H.8.5.3.2.11, “Derivation process for inter-layer luma motion vector prediction”: the center of a PU in Merge mode.

In case of Reference frame index approach, the BL motion vector is actually considered as Temporal Motion Vector predictor in the Merge and AMVP candidate MV lists.

In case of TextureRL approach, the BL motion vector is added as a new candidate in the Merge and AMVP candidate MV lists, in addition to the Spatial and Temporal Motion Vector predictors. .

In both cases, motion information is essentially retrieved from the compressed motion buffer. Given input coordinates xRL, yRL, the position that is used is found through the equations:

xRL  = ( xRL >> 4 ) << 4

yRL  = ( yRL >> 4 ) << 4

However, this corresponds to truncating said position: as depicted in Figure 1: above, we can imagine accessing the bottom-right CU of a 16x16 block. In that case, motion from CU numbered 1 is then used.

However, CU numbered 4 is closer and should yield a better candidate. This essentially raises the question of how to round the colocated position, e.g. to the nearest neighbour among CUs 1,2,3, and 4.

## Proposal

It was proposed in [1] to more generally define the rounding operation of this position as:

xRL  = ( ( xRL + R ) >> S ) << S

yRL  = ( ( yRL + R ) >> S ) << S

with R = 0, S = 4 being what is currently used in SHM 2.0.

There are two possible sets of parameters depending on memory compression. Test 2.1 uses R=4 and S=4, given the 4:1 compression (unchanged from SHM 2.0).

This proposal does not change the current memory requirements for storing motion data compared to SHM2.0.

# Test

The SCE2 test description [3] lists tests for single tools tests and combinations of tools. This document reports the results of Test 2.1 as described in the SCE2 description.

## Settings

SHM-2.0 is used as anchor, and the proponents in SCE2 have implemented their techniques on top of it. {RA,LB,LP} conditions are tested with Class A&B sequences [2].

Here are the macro settings for Test 2.1, as found in “TypeDef.h”:

#define M0112_POSITION_ROUNDING          1

#define M0141_MOTION_COMP                0

#if M0141_MOTION_COMP

#define HALF_COMP                        0

#endif

#define SHARP_M0258_CE2_5_3_1            0

## Results

We would like to thank Sato Kazushi and Sony for cross-checking those results in contribution N0xxx.

Table 1: mandatory results

# Conclusion

In this contribution, a simple modification to the rounding of the colocated position in the Base Layer, when used for motion derivation, is processed. In light of the coding gain for such a negligible change, we recommend adoption of the proposed changes.

# References

[1] “Non-SCE5: Position derivation for using Base Layer motion in SHVC”, JCTVC-M0112, April 2013

[2] “Common SHM test conditions and software reference configurations”, JCTVC-M1009, April 2013

[3] “HEVC Scalable Extensions Core Experiment 2 (SCE2): Combination of inter-layer syntax prediction and motion data compression”, JCTVC-M1102, April 2013

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft modification

## TextureRL approach

Section H.8.5.3.2.11 is modified as follows:

The variables mvLXRLand availableFlagLXRL with X being 0 or 1, are derived as follows:

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xPb + nPbW / 2		(H-1)

yPCtr = yPb + nPbH / 2		(H-2)

- The reference layer motion vector is derived as follows:

- The reference layer sample location derivation process as specified in subclause G.6.1 is invoked with  xPCtr, yPCtr as inputs and the outputs being assigned to  xPCtrRL and yPCtrRL ;

- The variable blPb specifies the luma prediction block covering the modified location given by ( ( ( xPCtrRL + 4 ) >> 4 ) << 4, ( ( yPCtrRL + 4 ) >> 4 ) << 4 ) inside the reference layer picture specified by RlPic.

- The luma location ( xPRL, yPRL ) is set equal to the top-left sample of the collocated luma prediction block specified by blPb relative to the top-left luma sample of the reference layer picture specified by RlPic.

- The arrays predFlagLXRL[ x ][ y ], mvLXRL[ x ][ y ] and refIdxLXRL[ x ][ y ] are set equal to the corresponding arrays of the reference layer picture specified by RlPic, PredFlagLX[ x ][ y ], mvLX[ x ][ y ] and RefIdxLX[ x ][ y ], respectively with X being the value of X this process is invoked for. The variables mvLXRL, refIdxLXRL and availableFlagLXRL are derived as follows

        - If colPb is coded in an intra prediction mode, both components of mvLXRL are set equal to 0, availableFlagLXRL is set equal to 0, and refIdxLXRL is set equal to -1;

          - Otherwise, If predFlagLXRL[ xPRL ][ yPRL ] is equal to 0, both components of mvLXRL are set equal to 0, and refIdxLXRL is set equal to -1, and availableFlagLXRL is set equal to 0.

          - Ohterwise, refIdxLXRL is set equal to  RefIdxLXRL [ xPRL ][ yPRL ], and availableFlagLXRL is set equal to 1. The two components of mvLXRL are derived as follows

mvLXRL [0] =  (mvLXRL[ xPRL ][ yPRL ][0]* ScaledW+ offset[0])/ PicWRL

mvLXRL [1] =  (mvLXRL[ xPRL ][ yPRL ][1]* ScaledH+ offset[1]) / PicHRL

          - where

            - offset[0] is set to (PicWRL/2-1) if mvLXRL[ xPRL ][ yPRL ][0] >0, and (1- PicWRL/2) otherwise;

            - offset[1] is set to (PicHRL/2-1) if mvLXRL[ xPRL ][ yPRL ][1] >0, and  (1- PicHRL/2) otherwise.

## reference index based approach

Section G.8.1.4.2.1 is modified as follows:

The variables predMode[ xP ][ yP ], mvLX[ xP ][ yP ], refIdxLX[ xP ][ yP ] and refPicOrderCntLX[ xP ][ yP ] are derived as follows.

- The center location (xPCtr, yPCtr) of the luma prediction block is derived as follows

xPCtr = xP + 8		(G-3)

yPCtr = yP + 8		(G-4)

- The derivation process for reference layer luma sample location specified in subclause G.6.1Error: Reference source not found is invoked with luma location ( xPCtr , yPCtr ) given as the inputs and ( xRef , yRef ) as output.

- The collocated position (xRL, yRL) is derived as follows

xRL = ( ( xRef + 4 ) >> 4 ) << 4		(G-5)

yRL = ( ( yRef + 4 ) >> 4 ) << 4		(G-6)