| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0140 |
| --- | --- |

| Title: | SCE3: Results of test 3.3 on Interlayer SAO filtering for SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Jonathan Taquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents the results and the complexity analysis of the test 3.3 of the SCE3 regarding the simplification of the Interlayer SAO contribution initially presented in JCTVC-L0234. This simplification was presented in JCTVC-M0114 and consists in applying the principle of the high-pass pre-filtering step of JCTVC-L0234 directly at the time of the determination of the Edge Index in the SAO process. It is reported that in average, over the SHM2 mandatory configurations, a BDR gain of -0.6%(Y), -0.6%(U), -0.6%(V) is obtained and this gain can reach up to -2.3%(Y), -1.6%(U), -1.6%(V) for the LD-P SNR configuration.

# Introduction

The proposed simplification is to re-use the High-Frequency Interlayer SAO (HF-IL-SAO) principle presented in JCTVC-L0234 [1] which was tested in the last SCE4 in JCTVC-M0265 [2].  In this proposal from Qualcomm, the SAO filtering process is applied on the high frequencies of the base layer reconstructed pixels.

In the proposed method and in order to still apply the SAO filtering on the high frequencies of the reconstructed base layer, the SAO Edge Offset process of HEVC has been only modified when the Edge Index value is determined. The SAO type, SAO class and the Band Offset process remain unchanged. The proposed modification is just extending the number of neighbours for the SAO Edge Offset process to determine which of the 4 offsets values has to be used.

In HEVC, the Edge Index of the current pixel c for the SAO Edge is determined as follows:

EdgeIndex(c) = sign (c - cn2) - sign (cn1- c) +2,                                                         (1)

where the definition of the function sign(.) is given by the following relationships

sign(x) = 1, when x>0

sign(x) = -1, when x<0

sign(x) = 0, when x=0

and where the neighboring pixel “cn1” and “cn2” are presented in Figure 1.

In the proposed method, the Edge Index is determined by the following formula by using an extended pixel neighborhood (cn3 and cn4) depicted in Figure 1:

| EdgeIndex(c) = sign[((2c-cn1-cn2+2)>>2)-((2cn2-c-cn4+2)>>2)] - sign[((2cn1-cn3-c+2)>>2)-((2c-cn1-cn2+2)>>2)]+2 | (2) |
| --- | --- |

By comparing the two formulas (1) and (2), the new EdgeIndex function of (2) is changed by adding a simple high-pass filtering on each term for the sign(.) function in equation (1).

In terms of complexity, as for the classical SAO implementation, some computations can be shared between neighboring pixels. So in eq. (2) only the difference between one pixel and its neighbor pixels needs to be computed.

Figure 1: Neighboring pixels for the SAO process.

# Results

This section presents the results of the proposed SAO method to filter the base layer reconstructed pixel by using the SHM2.0 testing conditions defined in the document [3]. In the following results, the SAO processing presented above is applied twice as for the HF-IL-SAO in the previous SCE4.2.1 [2].

In average for mandatory sequences, -0.6%(Y), -0.6%(U), -0.6%(V) BDR gains are obtained with the proposed method. Yet, for Ratio 2x and 1.5x the BDR gain is -0.5% for YUV component and reaches -1.6% average gain for all SNR configurations.

Table 1: BD-Rate savings for the proposed modifications compared to SHM2.0.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1.0% | -1.3% | -1.3% |  |  |  |  |  |  |
| Class B | -0.2% | -0.3% | -0.3% | -0.2% | -0.4% | -0.5% |  |  |  |
| Overall (Test vs Ref) | -0.4% | -0.6% | -0.6% | -0.2% | -0.4% | -0.5% |  |  |  |
| Overall (Test vs single layer) | 12.3% | 14.2% | 14.0% | 10.3% | 9.3% | 8.8% |  |  |  |
| Overall (Ref vs single layer) | 12.8% | 14.9% | 14.6% | 10.5% | 9.8% | 9.3% |  |  |  |
| EL only (Test vs Ref) | -0.8% | -1.0% | -1.0% | -0.1% | -0.4% | -0.4% |  |  |  |
| Enc Time[%] | 95.8% |  |  | 92.2% |  |  |  |  |  |
| Dec Time[%] | 97.4% |  |  | 93.1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.0% | -0.6% | -0.7% |  |  |  | -2.3% | -1.8% | -2.0% |
| Class B | -0.2% | -0.2% | -0.2% | -0.3% | -0.4% | -0.3% | -1.0% | -0.8% | -0.6% |
| Overall (Test vs Ref) | -0.5% | -0.3% | -0.4% | -0.3% | -0.4% | -0.3% | -1.4% | -1.1% | -1.0% |
| Overall (Test vs single layer) | 18.7% | 32.8% | 31.5% | 15.8% | 28.1% | 28.8% | 12.8% | 30.7% | 32.8% |
| Overall (Ref vs single layer) | 19.2% | 33.3% | 32.0% | 16.2% | 28.8% | 29.1% | 14.4% | 32.1% | 34.1% |
| EL only (Test vs Ref) | -0.8% | -0.7% | -0.7% | -0.3% | -0.4% | -0.3% | -2.2% | -1.8% | -1.7% |
| Enc Time[%] | 96.3% |  |  | 94.5% |  |  | 96.3% |  |  |
| Dec Time[%] | 97.9% |  |  | 95.3% |  |  | 107.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.5% | -0.7% |  |  |  | -2.1% | -1.5% | -1.9% |
| Class B | -0.3% | -0.3% | -0.2% | -0.4% | -0.4% | -0.4% | -0.9% | -1.0% | -0.8% |
| Overall (Test vs Ref) | -0.4% | -0.3% | -0.4% | -0.3% | -0.4% | -0.4% | -1.2% | -1.1% | -1.0% |
| Overall (Test vs single layer) | 27.9% | 38.5% | 39.3% | 24.3% | 32.5% | 35.4% | 22.8% | 33.3% | 38.0% |
| Overall (Ref vs single layer) | 28.5% | 39.0% | 39.7% | 24.8% | 33.0% | 35.9% | 24.3% | 34.7% | 39.5% |
| EL only (Test vs Ref) | -0.8% | -0.7% | -0.7% | -0.3% | -0.3% | -0.3% | -1.8% | -1.7% | -1.7% |
| Enc Time[%] | 95.9% |  |  | 95.5% |  |  | 96.8% |  |  |
| Dec Time[%] | 99.5% |  |  | 99.1% |  |  | 106.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -1.0% | -0.8% | -0.8% |  |  |  | -3.5% | -2.2% | -2.5% |
| Class B | -0.7% | -0.6% | -0.6% | -1.0% | -1.0% | -0.8% | -1.9% | -1.4% | -1.2% |
| Overall (Test vs Ref) | -0.8% | -0.6% | -0.7% | -1.0% | -1.0% | -0.8% | -2.3% | -1.6% | -1.6% |
| Overall (Test vs single layer) | 25.7% | 37.2% | 38.3% | 21.7% | 31.6% | 34.6% | 20.5% | 32.4% | 37.3% |
| Overall (Ref vs single layer) | 26.6% | 38.0% | 39.1% | 22.8% | 32.8% | 35.6% | 23.4% | 34.6% | 39.4% |
| EL only (Test vs Ref) | -1.2% | -1.0% | -1.1% | -1.1% | -1.1% | -0.9% | -3.3% | -2.6% | -2.6% |
| Enc Time[%] | 95.6% |  |  | 94.9% |  |  | 95.2% |  |  |
| Dec Time[%] | 99.0% |  |  | 97.9% |  |  | 106.6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

# Visual quality evaluation

The proposed scheme has globally the same visual quality than the SHM2.0 when tested at the same QP. Yet the proposed scheme has a larger impact on bitrate reduction than PSNR increase for the tested QP. Indeed, compared to the BDR measure, the bitrate reduction only, is about the same. Of course for some images the proposed scheme corrects some artefacts as presented in Figure 2.

Figure 2: Sub part of Kimono1 sequence (SNR RA test, QP0=38, QP1=34) frame number 2. Left image is SHM2.0 and right is proposed scheme.

# Complexity assessment

Complexity assessment modules recommended by AhG17/SCE3 were implemented in the source code and were enabled to perform memory access assessment. Table 2 gives these results for PU based. In average, the proposed method increases from 101% to 105% the memory bandwidth. For the worst case, the proposed method increases a little bit the SNR case where the gain is larger.

Table 2: Memory access assessment for proposed algorithms PU based.

|  | AI HEVC 2x |  |  |  | AI HEVC 1.5x |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 |  | DDR3 | Pure | DDR2 |  | DDR3 |  |  |  |
| Class A | 101% | 101% |  | 101% |  |  |  |  |  |  |  |
| Class B | 103% | 103% |  | 103% | 105% | 104% |  | 105% |  |  |  |
| Overall | 102% | 102% |  | 102% | 105% | 104% |  | 105% |  |  |  |
|  | RA HEVC 2x |  |  |  | RA HEVC 1.5x |  |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 |  | DDR3 | Pure | DDR2 |  | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% |  | 100% |  |  |  |  | 106% | 103% | 103% |
| Class B | 100% | 100% |  | 100% | 101% | 101% |  | 101% | 103% | 102% | 102% |
| Overall | 100% | 100% |  | 100% | 101% | 101% |  | 101% | 104% | 103% | 103% |
|  | LD-P HEVC 2x |  |  |  | LD-P HEVC 1.5x |  |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 |  | DDR3 | Pure | DDR2 |  | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% |  | 100% |  |  |  |  | 108% | 104% | 104% |
| Class B | 101% | 101% |  | 101% | 102% | 102% |  | 103% | 105% | 103% | 103% |
| Overall | 100% | 100% |  | 100% | 102% | 102% |  | 103% | 106% | 103% | 103% |
|  | LD_B HEVC 2x |  |  |  | LD-B HEVC 1.5x |  |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 |  | DDR3 | Pure | DDR2 |  | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% |  | 100% |  |  |  |  | 106% | 104% | 103% |
| Class B | 100% | 101% |  | 101% | 101% | 101% |  | 102% | 103% | 102% | 102% |
| Overall | 100% | 100% |  | 100% | 101% | 101% |  | 102% | 105% | 103% | 102% |
|  |  |  |  | Avearge |  |  |  |  |  |  |  |
|  | Scalability |  |  |  | SPATIAL |  |  |  | SNR |  |  |
|  |  |  |  | Pure | DDR2 | DDR3 |  | Pure | DDR2 | DDR3 |  |
|  | Test vs Ref |  |  | 102% | 101% | 102% |  | 105% | 103% | 103% |  |
|  |  |  | The worst case vs HEVC single layer |  |  |  |  |  |  |  |  |
|  | Scalability |  |  | SPATIAL |  |  | SNR |  |  |  |  |
|  | IL Processing |  |  | Block | Picture |  | Block | Picture |  |  |  |
|  | Reference |  |  | 145% | 160% |  | 200% | 218% |  |  |  |
|  | Tested |  |  | 152% | 168% |  | 200% | 220% |  |  |  |

Table 3 gives the memory access assessment for picture based configuration as recommended by AHG7. In average, no impact on memory access is observed with the proposed method for spatial scalability. For SNR scalability, an average reduction of memory access of 1% is obtained with the proposed Inter layer SAO. Indeed, the proposed modification increases the amount of blocks selected in the base layer frame, consequently the amount of motion compensation operations in the classical reference frames decreases.

Table 3: Memory access assessment for proposed algorithms Picture based.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |  |  |
| Class A | 100% | 100% | 100% |  |  |  |  |  |  |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% |  |  |  |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 100% | 100% |  |  |  | 99% | 99% | 99% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% | 99% |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 99% | 99% |  |  |  | 99% | 98% | 98% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% | 99% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% | 99% |
|  | LD_B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| Class A | 100% | 99% | 99% |  |  |  | 99% | 98% | 98% |
| Class B | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% | 99% |
| Overall | 100% | 100% | 100% | 100% | 100% | 100% | 99% | 99% | 99% |
|  |  |  | Avearge |  |  |  |  |  |  |
|  | Scalability |  |  | SPATIAL |  |  | SNR |  |  |
|  |  |  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |  |
|  | Test vs Ref |  | 100% | 100% | 100% | 99% | 99% | 99% |  |

# Conclusion

This contribution gives the results of test 3.3 of SCE3 regarding the simplification of the High Frequency Interlayer SAO. As presented in our first contribution Error: Reference source not found, the proposed simplification reduces the complexity while preserving the coding efficiency of the initial method presented in [1]. The BDR performance of the proposed solution respectively gives -1.6%(Y), -1.2%(U), -1.1%(V) over the SHM 2.0 for SNR configuration without any impact on the worst case complexity.

According to these results, we recommend to adopt this interlayer SAO modification.

# References

- W. Pu, J. Chen, K. Rapaka, M. Karczewicz, “High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-L0234, January 2013.

- W. Pu, J. Chen, X. Li, M. Karczewicz, “SCE4: High Frequency Pass Inter Layer Sample Adaptive Offset Filter”, Joint Collaborative Team on Video Coding, JCTVC-M0265, April 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, Joint Collaborative Team on Video Coding, JCTVC-M1009, April 2013.

- G. Laroche, P. Onno, J. Taquet, C. Gisquet and E. François, “Non-SCE4: On Interlayer SAO in SHVC”, Joint Collaborative Team on Video Coding, JCTVC-M0114, April 2013.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).