| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0141 |
| --- | --- |

| Title: | AHG5: on chroma QP for HEVC RExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François Christophe Gisquet Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution relates to the chroma QP in HEVC RExt. In the current HEVC RExt draft, one chroma QP table, linking the chroma QP (QPC) to the luma QP (QPY), is specified for each one of the three color formats 4:2:0, 4:2:2 and 4:4:4. This contribution contains three proposals. Proposal 1 consists in adding one syntax element in the PPS or SPS in case of 4:2:2 or 4:4:4 content, indicating which table is selected. This reportedly gives more flexibility to control the chroma QP depending on the content. Proposal 2 consists in removing the 4:2:2 table. This change reportedly simplifies the design, since only the 4:2:0 table, already present in the HEVC V1 specification, and the 4:4:4 table, based on a straightforward QPC derivation, are kept. Proposal 3 combines proposals 1 and 2 by removing the 4:2:2 table and adding a syntax element to select the table among the 2 available tables in case of 4:2:2 or 4:4:4 content.

The coding efficiency impact of removing the 4:2:2 table is reportedly small, mainly resulting in a slightly different balance between the luma and chroma quality. The proposed added syntax element is reported to provide a simple mechanism to easily and flexibly control this repartition.

# Introduction

In the current HEVC RExt Draft [1], the QP used for chroma is deduced from the QP used for luma and from the color format. The following process applies in case of non-monochrome format:

The variables qPCb and qPCr are set equal to the value of QpC as specified in Table 8-10 based on the index qPi equal to qPiCb and qPiCr, respectively, and qPiCb and qPiCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_cb_qp_offset + slice_cb_qp_offset )	(8-1)

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_cr_qp_offset + slice_cr_qp_offset )	(8-2)

The chroma quantization parameters for the Cb and Cr components, Qp′Cb and Qp′Cr, are derived as follows:

Qp′Cb = qPCb + QpBdOffsetC		(8-3)

Qp′Cr = qPCr + QpBdOffsetC		(8-4)

Table 8-1 – Specification of QpC as a function of qPi and ChromaArrayType

| ChromaArrayType = 1 | qPi | < 30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | > 43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QpC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

| ChromaArrayType = 2 | qPi | < 33 | 33 ≤ qPi < 39 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | > 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | = (qPi – 1) | 37 | 38 | 39 | 40 | 40 | 41 | 42 | 42 | 43 | 44 | 44 | 45 | = qPi − 6 |

| ChromaArrayType = 3 | qPi | < 51 | ≥ 51 |
| --- | --- | --- | --- |
|  | QPC | = qPi | 51 |

The figure below depicts QPC as a function of qPi, for the 3 tables.

Hence three different tables are used:

- The 4:2:0 table, already specified in the HEVC V1 specification.

- The 4:4:4 table, which is very straightforward since it just corresponds to the clipped value of qPi.

- The 4:2:2 table, which includes specific values that cannot be deduced from a simple equation.

# Performance analysis

We have evaluated the usage of each one of these 3 tables for each type of color format, using the AHG7 test conditions. In this section, we are focusing on the YUV 4:2:2 and YUV 4:4:4 content cases. Results for RGB 4:4:4 are provided in Annex. Results on Super-High tier are not impacted by our proposal, so they are not shown. Full results are provided in attached excel files.

For information, we measure an average YUV BDR measure, computed using the following average YUV PSNR:

- YUV 4:4:4 – PSNRYUV = (6.PSNRY + 4.PSNRU + 4.PSNRV) / 14

- YUV 4:2:2 – PSNRYUV = (6.PSNRY + 2.PSNRU + 2.PSNRV) / 10

The weights have been derived from the YUV 4:2:0 weighting proposed in contribution JCTVC-G399, corresponding to:

- YUV 4:2:0 – PSNRYUV = (6.PSNRY + PSNRU + PSNRV) / 8

As in YUV 4:2:2 case, the chroma components are twice higher than in 4:2:0, the weights for U and V are doubled compared to the YUV 4:2:0 case. In YUV 4:4:4 case, the chroma components are twice higher than in 4:2:2, therefore the weights for U and V are doubled compared to the YUV 4:2:2 case.

In addition, we also provide BDR performance evaluated using separate per-channel bit-rate measures.

## Results with YUV 4:2:2 content

Tables 2 to 4 summarize the results obtained for the YUV 4:2:2 sequences. Table 2 reports the BDR variations using the total bit-rate measures, while table 3 reports the BDR variations using per-channel bit-rate measures. Table 4 shows the average bitrate repartition among the Y, U and V channels. The left part shows the average over 3 high QPs (32,37,42) and the right part for QP=42.

From these tables, the following points can be observed:

- The 4:2:0 chroma QP table may lead to a non-negligible loss in luma, but this loss is well balanced by the gain in chroma (an average YUV BDR gain of 0.3 to 0.8% is observed in Main-tier). This is also visible in the per-channel measures.

- The 4:4:4 chroma QP table leads to a slight gain in luma, balanced by a loss in chroma (the indicative average YUV BDR shows a limited loss, from 0.2% to 0.6% for the 3 Main-tier configurations).

- The impact on per-channel bitrate repartition remains limited, especially when using the 4:4:4 table.

Conclusion:	For the YUV 4:2:2 content, other tables than the 4:2:2 table can be used without noticeable impact. The 4:2:2 table does not look very useful.

Table 2: BDR results for YUV 4:2:2 sequences, using non-separate channel BDR

| YUV 4:2:2 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 1.8% | -3.7% | -4.9% | -0.3% | 0.3% | -0.5% | -0.7% | 0.0% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -0.3% | 1.0% | 1.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 1.3% | -5.4% | -5.8% | -0.8% | 0.4% | -1.0% | -1.3% | -0.1% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -0.9% | 3.7% | 3.5% | 0.3% | -0.2% | 0.4% | 0.5% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 0.8% | -4.3% | -4.5% | -0.8% | 0.2% | -0.6% | -0.9% | -0.1% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -1.0% | 5.2% | 5.3% | 0.6% | -0.3% | 0.7% | 0.8% | 0.1% |

Table 3: BDR results for YUV 4:2:2 sequences, using separate channel BDR

| YUV 4:2:2 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 0.0% | -0.2% | -0.4% | -0.3% | 0.0% | -0.1% | 0.0% | 0.0% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | 0.0% | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 0.2% | -1.8% | -1.8% | -0.8% | 0.1% | -0.3% | -0.3% | -0.1% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -0.2% | -0.4% | -0.4% | 0.3% | 0.0% | -0.2% | -0.3% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 0.2% | -1.9% | -2.0% | -0.8% | 0.1% | -0.3% | -0.3% | -0.1% |
| table 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | -0.2% | -0.3% | -0.2% | 0.6% | -0.1% | -0.2% | -0.3% | 0.1% |

Table 4: variation of per-channel bitrate repartition

| YUV 4:2:2 | Average 3 Cfgs (QPs 32,37,42) |  |  | Average 3 Cfgs for QP42 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| table 4:2:0 | 87.6% | 7.3% | 5.1% | 86.1% | 8.0% | 5.9% |
| table 4:2:2 | 91.0% | 5.2% | 3.8% | 91.6% | 4.7% | 3.7% |
| table 4:4:4 | 92.7% | 4.2% | 3.1% | 94.4% | 3.1% | 2.6% |

## Results with YUV 4:4:4 content

Tables 5 to 7 summarize the results obtained for the YUV 4:4:4 sequences. Table 5 reports the BDR variations using the total bit-rate measures, while table 6 reports the BDR variations using per-channel bit-rate measures. Table 7 shows the average bitrate repartition among the Y, U and V channels.

The following points can be observed:

- The 4:2:0 chroma QP table leads to a noticeable gain in chroma that compensates well the luma loss (the indicative average YUV BDR shows a gain from 0.8 to 1.3% in Main-tier).

- The 4:2:2 chroma QP table leads to a slight loss in luma, well balanced by a slightly higher gain in chroma.

- The impact on the per-channel bitrate repartition at high QPs remains limited when using the 4:2:2 table. The impact of the 4:2:0 table is more important.

Conclusion: the 4:2:0 table (and also the 4:2:2 table) could be used alternatively to 4:4:4 table. In particular switching to the 4:2:0 table would permit a different repartition of the luma and chroma quality.

Table 5: BDR results for YUV 4:4:4 sequences, using non-separate channel BDR

| YUV 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 3.7% | -3.5% | -4.8% | -0.8% | 0.6% | -0.6% | -0.8% | -0.1% |
| table 4:2:2 | 0.5% | -0.8% | -0.9% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 4.5% | -6.1% | -6.6% | -0.8% | 1.3% | -1.2% | -1.5% | -0.1% |
| table 4:2:2 | 2.0% | -3.0% | -3.0% | -0.2% | 0.5% | -0.5% | -0.5% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 4.1% | -6.0% | -7.3% | -1.3% | 1.1% | -1.2% | -1.7% | -0.3% |
| table 4:2:2 | 2.3% | -4.0% | -4.9% | -0.8% | 0.6% | -0.7% | -1.0% | -0.2% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 6: BDR results for YUV 4:4:4 sequences, using separate channel BDR

| YUV 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 0.1% | -0.1% | -0.3% | -0.8% | 0.0% | 0.0% | 0.0% | -0.1% |
| table 4:2:2 | 0.0% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 1.0% | -1.5% | -0.4% | -0.8% | 0.4% | -0.4% | 0.1% | -0.1% |
| table 4:2:2 | 0.4% | 0.3% | 0.7% | -0.2% | 0.1% | 0.1% | 0.3% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 1.3% | -1.7% | -0.9% | -1.3% | 0.4% | -0.5% | -0.1% | -0.3% |
| table 4:2:2 | 0.6% | -0.2% | 0.4% | -0.8% | 0.2% | 0.0% | 0.2% | -0.2% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 7: variation of per-channel bitrate repartition

| YUV 4:4:4 | Average 3 Cfgs (QPs 32,37,42) |  |  | Average 3 Cfgs for QP42 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| table 4:2:0 | 77.1% | 12.6% | 10.2% | 75.2% | 13.4% | 11.4% |
| table 4:2:2 | 82.5% | 9.6% | 7.9% | 83.9% | 8.6% | 7.5% |
| table 4:4:4 | 85.6% | 7.9% | 6.5% | 89.0% | 5.9% | 5.2% |

# Proposed modifications

From these observations, three solutions are proposed to add flexibility in the chroma QP derivation and to simplify the current design.

## Proposal 1

Proposal 1 consists in adding a syntax element chroma_table_idc in the PPS or SPS, to signal in case of 4:4:4 or 4:2:2 color format, which table is used. The related syntax changes are shown in Annex.

Advantages

- Flexibility: more flexibility is given to control the chroma QP, depending on the content. In particular, for 4:4:4 content, this syntax element allows selecting the relevant table depending if the content is RGB or YUV.

## Proposal 2

Proposal 2 consists in keeping only 2 chroma QP tables, namely the 4:2:0 and 4:4:4 tables. By default, in case of 4:2:2 or 4:4:4 content, the 4:4:4 table is used.

Advantages

- Simplification: the HEVC RExt design is simplified since only the straightforward 4:4:4 table is added to the table of HEVC V1 version.

## Proposal 3

Proposal 3 consists in combining proposals 1 and 2. A syntax element chroma_420_not_used_flag is added in the PPS or SPS. It is signaled in case of 4:4:4 or 4:2:2 color format and indicates which table is used.

The changes in the specification are provided in the annex section.

Advantages

- Simplification: only the straightforward 4:4:4 table is added in the HEVC RExt version compared to the HEVC V1 version.

- Flexibility: more flexibility is given to control the chroma QP, depending on the content. This can be in particular of interest for YUV 4:2:2 or YUV 4:4:4 content, which may benefit from using the 4:2:0 table instead of the 4:4:4 one.

# Conclusion

In this contribution 3 proposals are made related to the chroma QP in the HEVC RExt:

- First it is proposed to add a syntax element, in case of 4:2:2 or 4:4:4 content, to select one table among the defined chroma QP tables; this gives more flexibility to control the chroma QP.

- Second it is proposed to remove the chroma QP table corresponding to the 4:2:2 case, and just keeping the 4:2:0 and 4:4:4 conversion tables; this results in a simplified design.

- Third proposal combines the 2 previous ones: the 4:2:2 table is removed and a syntax element is added to select the table in case of 4:2:2 or 4:4:4 content.

The coding efficiency impact of removing the 4:2:2 table is small, mainly resulting in a slightly different balanced repartition between the luma and chroma quality. The proposed added syntax element is a simple mechanism to easily control this repartition.

# References

- D. Flynn, J. Sole, T. Suzuki, “HEVC Range Extensions Draft 3”, JCTVC-M1005, 13th meeting, Incheon, April 2013.

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-L1006, 12th meeting, Geneva, Jan 2013.

# Patent rights declaration

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex1 – syntax changes

Removed text is highlighted in purple, added text in yellow.

## Syntax changes in Proposal 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| if( ChromaArrayType > 1 ) |  |
| chroma_table_idc | ue(1) |
| … |  |
| } |  |

7.4.2.2	Sequence parameter set RBSP semantics

…

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause Error: Reference source not found. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive.

chroma_table_idc specifies the chroma look-up table used to derive the chroma quantization parameters from the luma quantization parameter. When chroma_table_idc is equal to 0, the first one of tables 8-9 is used. When chroma_table_idc is equal to 1, the second one of tables 8-9 is used. When chroma_table_idc is equal to 2, the third one of tables 8-9 is used. When not present, chroma_table_idc is set equal to 0.

…

8.6.1	Derivation process for quantization parameters

…

Table 8-8 – Specification of QPC as a function of qPi and ChromaArrayType chroma_table_idc

| ChromaArrayType = 1 chroma_table_idc = 0 | qPi | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

| ChromaArrayType = 2 chroma_table_idc = 1 | qPi | < 33 | 33 ≤ qPi < 39 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | > 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | = (qPi – 1) | 37 | 38 | 39 | 40 | 40 | 41 | 42 | 42 | 43 | 44 | 44 | 45 | = qPi − 6 |

| ChromaArrayType = 3 chroma_table_idc = 2 | qPi | < 51 | ≥ 51 |
| --- | --- | --- | --- |
|  | QPC | = qPi | 51 |

…

## Syntax changes in Proposal 2

8.6.1	Derivation process for quantization parameters

…

Table 8-9 – Specification of QPC as a function of qPi and ChromaArrayType

| ChromaArrayType = 1 | qPi | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

| ChromaArrayType = 2 | qPi | < 33 | 33 ≤ qPi < 39 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | > 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | = (qPi – 1) | 37 | 38 | 39 | 40 | 40 | 41 | 42 | 42 | 43 | 44 | 44 | 45 | = qPi − 6 |

| ChromaArrayType = 3 > 1 | qPi | < 51 | ≥ 51 |
| --- | --- | --- | --- |
|  | QPC | = qPi | 51 |

…

## Syntax changes in Proposal 3

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| if( ChromaArrayType > 1 ) |  |
| chroma_420_not_used_flag | u(1) |
| … |  |
| } |  |

7.4.2.2	Sequence parameter set RBSP semantics

…

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in subclause Error: Reference source not found. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive.

chroma_420_not_used_flag specifies the chroma look-up table used to derive the chroma quantization parameters from the luma quantization parameter. When chroma_420_not_used_flag is equal to 1, the second one of tables 8-9 is used. When chroma_420_not_used_flag is equal to 0, the first one of tables 8-9 is used. When not present, chroma_420_not_used_flag is set equal to 0.

…

8.6.1	Derivation process for quantization parameters

…

Table 8-10 – Specification of QPC as a function of qPi and ChromaArrayType chroma_420_not_used_flag

| ChromaArrayType = 1 chroma_420_not_used_flag = 0 | qPi | <30 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | >43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | 29 | 30 | 31 | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 | 37 | = qPi − 6 |

| ChromaArrayType = 2 | qPi | < 33 | 33 ≤ qPi < 39 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | > 50 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | QPC | = qPi | = (qPi – 1) | 37 | 38 | 39 | 40 | 40 | 41 | 42 | 42 | 43 | 44 | 44 | 45 | = qPi − 6 |

| ChromaArrayType = 3 chroma_420_not_used_flag = 1 | qPi | < 51 | ≥ 51 |
| --- | --- | --- | --- |
|  | QPC | = qPi | 51 |

…

# Annex2 – Results for the RGB 4:4:4 content

The tables below summarize the results obtained for the RGB 4:4:4 sequences.

We report an average YUV BDR, based on an average PSNR computed for the three components as follows:

- RGB 4:4:4 – PSNRRGB = (PSNRR + PSNRG + PSNRB) / 3

From the results in the tables below, it must be concluded that the table 4:4:4 is actually the best one. Using another table always leads to noticeable loss in the G component, not balanced by the gain in the R and B components.

Conclusion

Keeping the 4:4:4 table is therefore recommended for the RGB 4:4:4 content.

Table 11: BDR results for RGB 4:4:4 sequences, using non-separate channel BDR

| RGB 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 8.0% | -1.5% | -1.5% | 0.9% | 1.3% | -0.2% | -0.2% | 0.2% |
| table 4:2:2 | 1.1% | -0.2% | -0.3% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 15.6% | -1.8% | -0.3% | 3.3% | 3.7% | -0.6% | 0.2% | 0.8% |
| table 4:2:2 | 6.0% | -0.6% | -0.5% | 1.4% | 1.1% | -0.1% | -0.1% | 0.2% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 17.0% | -1.9% | 0.0% | 3.6% | 3.9% | -0.7% | 0.3% | 0.9% |
| table 4:2:2 | 8.7% | -0.5% | -0.5% | 2.3% | 1.8% | 0.0% | -0.1% | 0.4% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 12: BDR results for RGB 4:4:4 sequences, using separate channel BDR

| RGB 4:4:4 | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 0.3% | 0.4% | 0.4% | 0.9% | 0.0% | 0.3% | 0.1% | 0.2% |
| table 4:2:2 | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Random Access Main-tier |  |  |  | Random Access High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 4.4% | 0.6% | 1.5% | 3.3% | 1.1% | 0.1% | 0.4% | 0.8% |
| table 4:2:2 | 1.7% | 0.5% | 1.2% | 1.4% | 0.3% | 0.1% | 0.2% | 0.2% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | Low delay B Main-tier |  |  |  | Low delay B High-tier |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| table 4:2:0 | 5.1% | 0.8% | 2.4% | 3.6% | 1.1% | 0.2% | 0.5% | 0.9% |
| table 4:2:2 | 2.6% | 1.1% | 2.1% | 2.3% | 0.5% | 0.3% | 0.4% | 0.4% |
| table 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

Table 13: variation of per-channel bitrate repartition

| RGB 4:4:4 | Average 3 Cfgs (QPs 32,37,42) |  |  | Average 3 Cfgs for QP42 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| table 4:2:0 | 31.6% | 40.5% | 27.8% | 31.5% | 36.4% | 32.1% |
| table 4:2:2 | 39.7% | 34.4% | 25.9% | 44.2% | 28.0% | 27.9% |
| table 4:4:4 | 45.9% | 30.3% | 23.8% | 54.7% | 22.0% | 23.3% |