| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0142 |
| --- | --- |

| Title: | AHG18: On 16-bits support for Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François Jonathan Taquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 edouard.francois@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution relates to Intra-coding of high bit-depth (16-bits) monochrome content in HEVC RExt. The goal is to address the sources of errors due to internal accuracy limitations without explicitly extending the bit-depth of the internal operations in the HEVC design. The proposed solution re-uses the HEVC 4:2:0 8-bit design to code 16-bit 4:0:0 content. The input 16-bit monochrome picture is first converted into a color picture of lower bit-depth (typically 8 bits per component) by splitting the MSBs and LSBs of the 16-bit samples into luma and chroma components. Then the color picture is coded, with specific adaptations related to the quantization in order to take into account the range difference between the MSBs and LSBs. The performance of the concept is illustrated on preliminary results.

# Introduction

At the 13th JCTVC meeting, requirements to support efficient coding of 12-bit, 14-bit, and 16-bit monochrome content have been expressed in the context of medical applications, [JCTVC-M0190]. The document also requires support for efficient reversible (i.e., lossless) and diagnostically acceptable irreversible coding. In document JCTVC-M0178, solutions are explored to efficiently support high bit-depth content by increasing the internal registers bit-depth mostly in transform and quantization processes.

In this proposal, we are focusing on monochrome high bit-depth content and we are investigating a solution that prevents to increase the internal registers bit-depth. The solution consists in re-using the 8-bit design to encode monochrome 16-bit content.

# Proposal

The principle of the proposed idea is to split the samples of the input picture of bit-depth B into two sub-pictures of lower bit-depth, one containing the MSBs of the input samples, and the other one containing the LSBs of the input samples. The sub-pictures are then packed into a color picture whose components are of lower bit-depth than B and that can be coded using existing HEVC coding tools. The process is illustrated in figure 1. The decoding process is shown in figure 2.

Figure 1: encoder side

Figure 2: decoder side

The frame-rearrangement applied here consists in splitting the MSBs and LSBs in the luma and chroma components of a 4:4:4 or 4:2:2 picture, as shown in figures 3 and 4.

Figure 3: 4:4:4 packing

Figure 4: 4:2:2 packing

The quantization of the U and V component is adapted to take into account the range difference between the MSB and the LSB. In particular, the HEVC design is modified to support negative QPs, and the chroma QP is derived from the luma QP as follows:

QPC = QPY + 48

# Experiments

Very first experiments have been produced. We have used native 16-bit RGB pictures, in which we have only considered the G-channel.

As reference, the G-channel is coded using the HM-RExt3.0 as a 4:0:0 picture using an internal bit-depth of 8, 10, 12 or 14 bits.

In our solution the G-channel is packed into a 4:4:4 picture, with Y component corresponding to the 8 MSBs of the input picture samples, U component corresponding to the 8 LSBs of the input picture samples, and V component is empty (filled by value of 128). Then the 4:4:4 8 bits picture is coded with an internal bit-depth of 8.The HM-RExt3.0 has been modified to support negative QPs and a delta QP between luma and chroma of 48.

The picture is intra coded, with the following settings.

Test 1:

- RDOQ, RDOQTS off

- DBF, SAO off

- IPCM, TransQuantBypass off

Test 2:

- RDOQ, RDOQTS off

- DBF, SAO off

- IPCM, TransQuantBypass on

The PSNR is computed on 16 bits.

We present in the following graphs the results for a given picture (‘Honolulu Cathedral’), for the 2 settings. The other cases are provided in the attached excel file. X-axis corresponds to the spent bits, y-axis to the obtained 16-bits PSNR (in dB). Results using our proposal are named ‘FP IBDI 8’. In test 2, an infinite PSNR can be reached for very low QPs.

Figure 5: Honolulu Cathedral picture, 1920x1080, test1.

Figure 6: Honolulu Cathedral picture, 1920x1080, test2.

# Conclusion

This contribution provides preliminary results for coding 16-bit monochrome content using existing 8-bit HEVC tools, without requiring to increase the internal registers bit-depth of the coding process. The 16 bits samples are split into MSBs and LSBs, respectively put into the luma and chroma components of a color picture. First experiments illustrate the concept and the ability to reach an objective quality higher than the one obtained by coding a 16-bit content with an internal bit-depth of 14. Thanks to this solution, it is not necessary to modify the internal accuracy of the quantization, transform and entropy coding processes.

# References

- P. Amon, A. Hutter, U.-E. Martin, N. Wirsz, “Requirements for medical imaging applications”, JCTVC-M0190, 13th meeting, Incheon, April 2013.

- K. Sharman, N. Saunders, J. Gamei, “AHG5: Range Extensions and High Bit Depths”, JCTVC-M0178, 13th meeting, Incheon, April 2013.

# Patent rights declaration

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).