| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0143 |
| --- | --- |

| Title: | On Mode Dependent Intra Smoothing for Range Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Christophe Gisquet Tangi Poirier Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33(0)299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the Mode Dependant Intra Smoothing (MDIS) of HEVC for Range Extension. The modification consists in simplifying the MDIS usage by using the same minimum condition for TU sizes greater than 4x4 when the input sequence color format is 4:4:4. In average for RExt Intra configurations, it is reported that the proposed modification gives -0.34%(Y), -0.14%(U), -0.21%(V) BDR gain.

# MDIS for RExt

In HEVC version 1, the Mode Dependant Intra Smoothing is applied only for Luma component. In the current version of HEVC Range Extension, the MDIS filter is applied on both Luma and Chroma components. This method consists in applying a smoothing filter on the reference samples for Intra prediction according to the predictor mode index and the TU size. These filtering dependencies can be summarized by the following table.

Table 1 Filtering decision rule for Intra input sample

| TU Size. Mode index; 4x4; 8x8; 16x16; 32x32. (PLANAR) 0; 0; 1; 1; 1. (DC) 1; 0; 0; 0; 0. 2; 0; 1; 1; 1. 3; 0; 0; 1; 1. 4; 0; 0; 1; 1. 5; 0; 0; 1; 1. 6; 0; 0; 1; 1. 7; 0; 0; 1; 1. 8; 0; 0; 1; 1. 9; 0; 0; 0; 1. (HOR) 10; 0; 0; 0; 0. 11; 0; 0; 0; 1. 12; 0; 0; 1; 1. 13; 0; 0; 1; 1. 14; 0; 0; 1; 1. 15; 0; 0; 1; 1. 16; 0; 0; 1; 1. 17; 0; 0; 1; 1. | TU Size. Mode index; 4x4; 8x8; 16x16; 32x32. 18; 0; 1; 1; 1. 19; 0; 0; 1; 1. 20; 0; 0; 1; 1. 21; 0; 0; 1; 1. 22; 0; 0; 1; 1. 23; 0; 0; 1; 1. 24; 0; 0; 1; 1. 25; 0; 0; 0; 1. (VER) 26; 0; 0; 0; 0. 27; 0; 0; 0; 1. 28; 0; 0; 1; 1. 29; 0; 0; 1; 1. 30; 0; 0; 1; 1. 31; 0; 0; 1; 1. 32; 0; 0; 1; 1. 33; 0; 0; 1; 1. 34; 0; 1; 1; 1. |
| --- | --- |

# Proposed modification

The current version of HEVC RExt uses the same table dependencies as the HEVC version 1. But this table is not the best performing one for the extended color formats, especially for 4:4:4.

So in this contribution, we propose to replace the mode and TU size dependencies of current version of HEVC RExt (represented in Table 1) with the following dependencies:

Table 2: Proposed Filtering decision rule for Intra input sample for 4:4:4 color format

| TU Size. Mode index; 4x4; 8x8; 16x16; 32x32. (PLANAR) 0; 0; 1; 1; 1. (DC) 1; 0; 0; 0; 0. 2; 0; 1; 1; 1. 3; 0; 1; 1; 1. 4; 0; 1; 1; 1. 5; 0; 1; 1; 1. 6; 0; 1; 1; 1. 7; 0; 1; 1; 1. 8; 0; 1; 1; 1. 9; 0; 1; 1; 1. (HOR) 10; 0; 0; 0; 0. 11; 0; 1; 1; 1. 12; 0; 1; 1; 1. 13; 0; 1; 1; 1. 14; 0; 1; 1; 1. 15; 0; 1; 1; 1. 16; 0; 1; 1; 1. 17; 0; 1; 1; 1. | TU Size. Mode index; 4x4; 8x8; 16x16; 32x32. 18; 0; 1; 1; 1. 19; 0; 1; 1; 1. 20; 0; 1; 1; 1. 21; 0; 1; 1; 1. 22; 0; 1; 1; 1. 23; 0; 1; 1; 1. 24; 0; 1; 1; 1. 25; 0; 1; 1; 1. (VER) 26; 0; 0; 0; 0. 27; 0; 1; 1; 1. 28; 0; 1; 1; 1. 29; 0; 1; 1; 1. 30; 0; 1; 1; 1. 31; 0; 1; 1; 1. 32; 0; 1; 1; 1. 33; 0; 1; 1; 1. 34; 0; 1; 1; 1. |
| --- | --- |

This modification doesn’t need to store an additional table because the dependencies of 8x8 and 16x16 is the same as the current mode dependencies of the 32x32 TU size, for both Luma and Chroma components.

# Experimental results

Table 3 gives the results of the proposed MDIS modification for 4:4:4 sequences for Intra configuration according to the common test condition JCTVC-K1006 [1].  In average the proposed modification gives 0.34%(Y), 0.14%(U), 0.21%(V) BDR gain. For inter configurations, the proposed method gives small average gains. The runtime is not affected at encoder and decoder side.

Table 3 Tests results of the proposed modification.

|  | All Intra Main-tier |  |  |  | All Intra High-tier |  |  |  | All Intra Super-High-tier |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| RGB 4:4:4 | -0.5% | -0.3% | -0.3% | -0.4% | -0.4% | -0.2% | -0.3% | -0.3% | -0.3% | -0.1% | -0.2% | -0.2% |
| YCbCr 4:4:4 | -0.2% | -0.1% | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |
| YCbCr 4:2:2 | N.A. |  |  | N.A. |  | N.A. |  | N.A. |  | N.A. |  | N.A. |
| Overall | -0.4% | -0.2% | -0.3% |  | -0.4% | -0.1% | -0.2% |  | -0.3% | -0.1% | -0.2% |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 100% |  |  |  | 100% |  |  |  |

# WD modification

Inputs to this process are:

–	the neighbouring samples p[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1,

–	a variable nTbS specifying the transform block size.

Outputs of this process are the filtered samples pF[ x ][ y ], with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1.

The variable filterFlag is derived as follows:

–	If one or more of the following conditions are true, filterFlag is set equal to 0:

- predModeIntra is equal to INTRA_DC.

- nTbS is equal 4.

–	Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 26 ), Abs( predModeIntra − 10 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Error: Reference source not found.

- If chroma_format_idc is equal to 3 (4:4:4), the variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ 32 ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

- Otherwise the variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

# Conclusion

In this contribution, we propose a minor modification of the Mode Dependant Intra Smoothing for HEVC Range Extension when the color format is 4:4:4. In average, the proposed method gives -0.34%(Y), -0.14%(U), -0.21%(V) BDR gain for Intra configurations.

The proposed modification is very simple and doesn’t need any additional table compared to the current HEVC version 1.

According to the results of this minor modification, we recommend to adopt this modification for HEVC Range Extension.

# References

- D. Flynn, K. Sharman, “Common test conditions and software reference configurations for HEVC Range Extensions”, JCTVC-L1006, 12th JCT-VC Meeting, 10-19 Arp., 2012.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).