| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0146 |
| --- | --- |

| Title: | AHG14: On resampling & color gamut scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kemal Ugur A. Aminlou | Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

JCTVC-M0214 proposed a method for increasing the bit-depth of enhancement layer to achieve color gamut scalability. An important use-case mentioned in JCTVC-M0214 is the enhancement layer increasing both the spatial resolution and bit-depth of the base layer signal and this is achieved by using a cascaded process, where a resampling filter is utilized first on base layer reconstructed picture and then the bit-depth of the upsampled picture is increased. It is argued that this results in redundant computations and this contribution instead proposes to change the resampling process to achieve spatial resolution and bit-depth increase.

# Introduction & Proposal

JCTVC-M0214 proposed a method for increasing the bit-depth of enhancement layer to achieve color gamut scalability. An important use-case mentioned in JCTVC-M0214 is the enhancement layer increasing both the spatial resolution and bit-depth of the base layer signal. For example, the base layer could be of HD content using BT.709 (which uses 8-bit sample depth) and enhancement layer could be of UltraHD content using BT.2020 (which uses 10-bit sample depth).

JCTVC-M0214 achieves this by using a cascaded process, where a resampling filter is utilized first on base layer reconstructed picture and then the bit-depth of the upsampled picture is increased. The details of this are illustrated in Figure 1. In Figure 1, the reconstructed base layer first undergoes an upsampling filter and the samples of the reconstructed base layer is multiplied with filter coefficients and a rounding offset is added to the sum and finally this is rounded to the base layer bit-depth. This is followed by clipping operation so that the values stay between 0 and the maximum value of the base-layer bit-depth. Afterwards, samples of the upsampled picture undergo bit-depth scaling and the inter-layer prediction picture for enhancement layer is generated.

Figure 1 Cascaded process for upsampling and bit-depth scaling. R’0,n refers to decoded base layer, Rups’0,n refers to upsampled base layer picture and P’0,n is prediction picture for coding enhancement layer.

It is argued that this results in redundant computations and this contribution instead proposes to change the resampling process to achieve spatial resolution and bit-depth increase. The proposal is illustrated in Figure 2. In Figure 2, the reconstructed base layer first goes to an upsampling filter where the samples of the reconstructed base layer is multiplied with filter coefficients and a rounding offset is added to the sum. The rounding offset depends both on accuracy of filter coefficients and the difference between enhancement and base layer bit-depth. Then, this sum is shifted to enhancement layer bit-depth and similarly, the shift depends both on accuracy of filter coefficients and the difference between enhancement and base layer bit-depth. Finally, a clipping operation is performed so that the values stay between 0 and the maximum value of the enhancement-layer bit-depth.

Figure 2 Details of the proposed process for upsampling and bit-depth scaling. R’0,n refers to decoded base layer, and P’0,n is prediction picture for coding enhancement layer.

# Specification text

The proposal is implemented to specification text as follows.

- G.8.1.4 Resampling process for inter layer reference pictures

Input to this process is

- A cropped decoded reference layer picture rlPic.

Output of this process is

–	The resampled reference layer picture rsPic.

The variables ScaledW and ScaledH are set equal to the width and height of the current picture in units of luma samples, respectively. The variable rsPicSample specifies the resampled sample values of rsPic of the luma and chroma components. The variable rsPicMotion specifies the resampled motion field of rsPic.

The variables PicWRL and PicHRL are set equal to the width and height of the cropped decoded reference layer picture rlPic in units of luma samples, respectively. The variable rlPicSample specifies the sample values of rlPic of the luma and chroma components. The variable rlPicMotion specifies the motion field of rlPic.

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if ScaledW is equal to PicWRL and ScaledH is equal to PicHRL

  - rsPicSample is set to rlPicSample,

  - When sps_inter_layer_mfm_enable_flag is equal to 1, rsPicMotion is set to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture sample resampling process as specified in subclause G.8.1.4.1 is invoked with the sample values of rlPicSample bit-depth of base-layer samples, nBase and bit-depth of enhancement-layer samples, nEnh,  as input, and with the resampled sample values of  rsPicSample as output.

  - When sps_inter_layer_mfm_enable_flag is equal to 1, the picture motion field resampling process as specified in subclause G.8.1.4.2 is invoked with the compressed motion field of rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

- Luma sample interpolation process

Inputs to this process are

–	the luma reference sample array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture.

–	bit-depth of base-layer samples, nBase.

–	bit-depth of enhancement-layer samples, nEnh.

Output of this process is a interpolated luma sample value intLumaSample.

Table G-1 specifies the 8-tap filter coefficients fL[ p, x ] with p = 0 ... 15 and x = 0 ... 7 used for the luma resampling process.

Table G-1 – 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 4 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 6 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 7 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 10 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 11 | −1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 12 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 14 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 15 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in subclause G.6.2 is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )	(G-1)

xPhase = ( xRef16 ) % 16	(G-2)

- The variables yRef and yPhase are derived as follows

yRef     = ( yRef16 >> 4 )	(G-3)

yPhase = ( yRef16 ) % 16	(G-4)

- The sample value tempArray[ n] with n = 0..7, is derived as follows.

yPosRL = Clip3( 0, RefLayerPicHeightInSamplesL – 1, yRef + n – 1 ) 	(G-5)

refW      = RefLayerPicWidthInSamplesL

tempArray[n] = fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 3), yPosRL ] +
	fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 2), yPosRL ] +
	fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 1), yPosRL ] +
	fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef      ), yPosRL ] +
	fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 1), yPosRL ] +	(G-6)
	fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 2), yPosRL ] +
	fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 3), yPosRL ] +
	fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 4), yPosRL ]

- The interpolated luma sample value intLumaSample is derived as follows.

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(G-7)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + (1<<(12 + nBase – nEnh – 1)) >> (12 +nBase – nEnh)

intLumaSample = Clip3( 0, ( 1 << nEnh) – 1 , intLumaSample )	(G-8)

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).