| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0149 |
| --- | --- |

| Title: | Non-SCE1: Results of test 1.2 on sampling offset signalling with accurate interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

Up-sampling scheme achieving aligned sampling grids between the EL and the upsampled BL pictures, even when the input BL and EL sequences have non-zero phase shift, was studied in SCE1. In order to allow this feature support four additional interpolation filters for Luma and one Chroma interpolation filter are needed to be added to SHVCM. For 2 from 4 additional Luma interpolation filters more accurate coefficients design was presented in Incheon meeting. This contribution presents test results. Test is identical to test 1.2, just for 2 fractional positions interpolation filter with lower deviation from identical frequency response is used. If down-sampler phase shift is known and signalled with 1/16 accuracy then there is no performance improvement from more accurate filter design. But if down-sampler phase shift is known (signalled) with 1/8 accuracy then more accurate filter design provides 0,3% Luma BD-rate gain. More accurate filter design reduces performance gap between 1/16 and 1/8 accuracy for down-sampler phase shift signalling by half with no additional cost.

# Introduction

In the current SHVC working draft [1], up-sampling for inter layer prediction is designed based on an assumption that the sampling grids of BL and EL pictures have no phase shift. Under this assumption for ×2 and ×1,5 spatial scalability support only 1 additional Luma and 3 Chroma interpolation filters were added into SHVC design (compare to HEVC motion compensation interpolation filters set). These filters are listed in Table 1,2. Interpolation filters used in HEVC are marked “HEVC”. Interpolation filters added for ×2 and ×1,5 spatial scalability support are marked as “SHVC”.

If base layer and enhancement layer pictures are misaligned due to non-zero phase shift used in down-sampling process then SHM2.0 up-sampling (strongly assumes that BL and EL sampling grids are aligned) is not adequate. If with additional syntax decoder is informed about down-sampler phase shift then it can be taken into account (as it was suggested in [2]) and some performance improvement was reported.

The goal for SCE1 [3] was to identify performance impact of taking into account this down-sampler phase shift and checking dependency between performance and accuracy of down-sampler phase shift.

In order to perform SCE1 test alternative down-sampler was released by Inter Digital. Non-zero down-sampler phase shift used was ¼ for ×2 spatial scalability tests and 3/16 for ×1,5 spatial scalability tests.

In order to study dependency between performance and accuracy 3 ×1,5 spatial scalability tests were performed with signalled down-sampler phase shift 3/16 (absolutely accurate), 1/8 (rounded to 1/8 accuracy) and 1/4 (rounded to 1/4 accuracy).

In order to perform SCE1 tests four additional Luma and one additional Chroma interpolation filters are needed. They marked as “SCE1” in Tables 1 and 2.

It should be mentioned that Luma interpolation filter for phase shift 3/8 was not tested (not used in any ctc or SCE1 test).

Table 1. SHM2.0 and additional Luma interpolation filters studied in SCE1.

fL[ 16-p, k ] = fL[ p, 7-k ] (0≤k≤7)

| phase p | Luma interpolation filter coefficients |  |  |  |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |  |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 | HEVC |
| 1 | 0 | 1 | -3 | 63 | 4 | -2 | 0 | 0 | SCE1 |
| 2* | 0 | 2 | -6 | 61 | 9 | -3 | 1 | 0 | SCE1 |
| 3 | -1 | 3 | -8 | 60 | 13 | -4 | 1 | 0 | SCE1 |
| 4 | -1 | 4 | -10 | 58 | 17 | -5 | 1 | 0 | HEVC |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 | SHM 2.0 |
| 6 | NA | NA | NA | NA | NA | NA | NA | NA | Not tested |
| 7* | -1 | 4 | -11 | 45 | 34 | -10 | 4 | -1 | SCE1 |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 | HEVC |

Table 2. SHM2.0 and additional Chroma interpolation filters studied in SCE1.

fC[ 16-p, k ] = fC[ p, 3-k ] (0≤k≤3)

| phase p | interpolation filter coefficients |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |  |
| 0 | 0 | 64 | 0 | 0 | HEVC |
| 1 | -2 | 62 | 4 | 0 | SHM 2.0 |
| 2 | -2 | 58 | 10 | -2 | HEVC |
| 3 | -4 | 56 | 14 | -2 | SCE1 |
| 4 | −4 | 54 | 16 | −2 | HEVC |
| 5 | −6 | 52 | 20 | −2 | SHM 2.0 |
| 6 | −6 | 46 | 28 | −4 | HEVC |
| 7 | -4 | 42 | 30 | -4 | SHM 2.0 |
| 8 | −4 | 36 | 36 | −4 | HEVC |

Filters listed in Table 1 were analysed in [4] and for several fractional positions more accurate filter design was suggested. Namely for target phase shift 1/8

{-1,     2,    -5,    62,     8,    -3,     1,     0}/64

and for target phase shift  7/16

{ -1,     4,   -11,    44,    35,   -10,     4,    -1} /64

were recommended. Fractional positions where alternative filters were used are marked by * in Table 1.

The filter design principle in [4] is minimization the difference between frequency response of designed FIR and ideal filter. All filters previously used in HEVC and SHVC design follow this principle. Minimal difference between frequency response of designed FIR and ideal filter leads to minimal error accumulation during interpolation.

Fig. 1. Frequency response of Luma re-sampling filters under comparison for target phase 1/8.

Fig. 2. Frequency response of Luma re-sampling filters under comparison for target phase 7/16.

Frequency responses of filters used in SCE1 and proposed in [4] are shown on Fig. 1 and 2. Difference is very small (especially for target phase shift 7/16) and so performance difference is expected to be small as well.

# Test results overview

Test results in SCE1 were obtained using down-sampled with non-zero phase shift base-layer sequences and reported compare to SHM2.0 as anchor. Performance for different accuracy of down-sampler phase signalling is summarized in Table 3. In Table 3 test results vs SHM2.0 are reported. These results allow checking dependency between performance and accuracy of down-sampler phase signalling. Figure 1 shows the graph for this dependency. This is easy to notice that if down-sampler phase shift is known (and signalled) with ¼ accuracy then only half of the gain from EL and BL sampling grid alignment could be achieved. But performance difference between 1/8 and 1/16 accuracy for down-sampling phase shift is 0,6% (if with filter design from SCE1).

If more accurate filter design recommended in [4] is used then performance difference between 1/8 and 1/16 accuracy for down-sampling phase shift signalling becomes 0,3% only.

Table 3. Summary of performance test vs SHM2.0 using non-zero down-sampler phase shift (non ctc sequences).

| Test | x2 |  |  | x1,5 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| 0 (SHM2.0) | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| 1/16 (SCE1) | -6,7 | -5,2 | -5,1 | -6,3 | -5,9 | -5,5 |
| 1/16 (M0425) |  |  |  | -6,3 | -5,8 | -5,5 |
| 1/8 (SCE1) |  |  |  | -5,7 | -5,3 | -4,9 |
| 1/8 (M0425) |  |  |  | -6,0 | -5,5 | -5,2 |
| 1/4 (SCE1) |  |  |  | -3,9 | -3,7 | -3,4 |
| 1/4 (M0425) |  |  |  | -4,0 | -3,9 | -3,6 |

Figure 3. Performance vs bits for down-sampling phase shift signalling.

Table 4 reports performance vs HEVC single layer. This analysis is useful for understanding whether the usage of non-zero phase shift in down-sampling process improves performance of scalable video codec. Let’s compare two systems:

- zero phase shift is used in down-sampling process and SHM2.0 re-sampling scheme is used;

- Non-0 phase shift is used in down-sampling process, additional syntax element tells decoder about this phase shift and re-sampling process is adjusted accordingly.

From Table 4 it is clear that zero down-sampler phase shift with SHM2.0 re-sampling scheme performs the best. Non-zero phase shift in down-sampling process, even it is signalled to decoder and taken into account in up-sampling process, doesn’t allow better compression by scalable codec compare to single layer. Performance drop (compare to single layer HEVC) is 0,4…0,8% higher if non-zero phase shift is used during down-sampling. From view point of scalable system performance improvement application of non-zero phase shift in down-sampling apparently gives no benefits.

Table 4. Summary of performance test vs HEVC single layer.

| Down-sampler | Re-sampling | x2 |  |  | x1,5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |
| 0-phase shift | SHM2.0 | 21,8 | 31,3 | 31,3 | 18,6 | 26,1 | 27,5 |
| Non-0 phase shift | 1/16 (SCE1) | 22,2 | 31,5 | 31,7 | 19,4 | 26,8 | 28,1 |
|  | 1/16 (M0425) |  |  |  | 19,5 | 26,9 | 28,1 |
|  | 1/8 (SCE1) |  |  |  | 20,2 | 27,6 | 28,8 |
|  | 1/8 (M0425) |  |  |  | 19,9 | 27,2 | 28,5 |
|  | 1/4 (SCE1) |  |  |  | 22,6 | 29,7 | 30,8 |
|  | 1/4 (M0425) |  |  |  | 22,4 | 29,5 | 30,6 |

Together with performance average memory access was estimated in SCE1 [5]. There is no more than 0,05% in average memory access for filters set tested in SCE1 and more accurate one tested here.

# Conclusions

This contribution presents performance test results for up-sampling scheme achieving aligned sampling grids between the EL and the up-sampled BL pictures, with more accurate interpolation filter design. Only 2 filters in a set are different from tests in SCE1. Filters were published at Incheon meeting. Based on reported test results 1/16 accuracy for down-sampler phase shift seems redundant; 1/8 accuracy provides most part of the gain (~6%) on test content with non-zero down-sampler phase shift. Usage of more accurate up-sampling filter design improves performance by 0,3% compare to SCE1 test with no additional cost. It is recommended to use 1/8 accuracy for down-sampling phase shift signalling and accurate re-sampling filter design. Usage of non-zero phase shift in down-sampling process doesn’t improve performance of SHVC compare to HEVC single layer.

# Reference

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Working Draft 2,” JCT-VC document, JCTVC-M1008, Incheon, South Korea, April 2013.

- L. Guo, J. Chen, M. Karczewicz (Qualcomm), J. Dong, Y. Ye, Y. He (InterDigital), K. Ugur, J. Lainema (Nokia), “Signaling of Phase Offset in Up-sampling Process and Chroma Sampling Location,” JCT-VC document, JCTVC-M0465, Incheon, South Korea, April 2013.

- E. Alshina, J. Dong, L. Guo, “HEVC Scalable Extensions Core Experiment 1 (SCE1): Support for additional resampling phase shifts,” JCT-VC document, JCTVC-M1101, Incheon, South Korea, April 2013.

- E.Alshina, A.Alshin, “About phase calculation and up-sampling filter coefficients in JCTVC-M0188 and JCTVC-M0322,” JCT-VC document, JCTVC-M0425, Incheon, South Korea, April 2013

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Incheon, South Korea, April 2013.

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text.

In Table , we propose the syntax of the sampling grid information, followed by the semantics.

Table 5 Syntax of sampling grid information

| sampling_grid_information( ) { | Descriptor |
| --- | --- |
| phase_offset_present_flag | u(1) |
| if ( sampling_grid_info_present_flag ) { |  |
| horizontal_phase_offset8 | u(2) |
| vertical_phase_offset8 | u(2) |
| chroma_phase_x_flag | u(1) |
| chroma_phase_y | u(2) |
| } |  |

phase_offset_present_flag equal to 1 specifies horizontal_phase_offset8, vertical_phase_offset8, chroma_phase_x_flag, and chroma_phase_y are presented in sampling_grid_information(), sampling_grid_info_present_flag equal to 0 specifies horizontal_phase_offset8, vertical_phase_offset8, chroma_phase_x_flag, and chroma_phase_y are not presented in sampling_grid_information(). When not present, sampling_grid_info_present_flag is inferred to be equal to 0.

horizontal_phase_offset8 specifies the horizontal phase offset of the samples in the current layer with respect to lower layer in 1/8-th pixel units and it is used to calculate the reference layer sample locations used in re-sampling. The value of horizontal_phase_offset16 should be in the range from 0 to 3 inclusive. When horizontal_phase_offset8 is not present, the value of horizontal_phase_offset8 is inferred to be zero.

vertical_phase_offset8 specifies the vertical phase offset of the samples in the current layer with respect to lower layer in 1/8-th pixel units and it is used to calculate the reference layer sample locations used in re-sampling. The value of vertical_phase_offset16 should be in the range 0 to 3 inclusive. When vertical_phase_offset8 is not present, the value of vertical_phase_offset8 is inferred to be zero.

chroma_phase_x _flag specifies the horizontal phase shift of the chroma components in units of half luma samples of a picture or layer picture in the CVS. When chroma_phase_x_flag is not present, it shall be inferred to be equal to 0. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

chroma_phase_y specifies the vertical phase shift of the chroma components in units of half luma samples of a picture or layer picture in the CVS. When chroma_phase_y is not present, it shall be inferred to be equal to 0. The value of chroma_phase_y shall be in the range of 0 to 2, inclusive. The phase shift refers to the spatial displacement between the top left chroma sample and the top left luma sample.

sampling_grid_information( ) is proposed to be signaled as part of SPS extension, as shown in Table .

Table 6 Signaling sampling grid information in SPS extension

| sps_extension ( ) { | Descriptor |
| --- | --- |
| ...... |  |
| sampling_grid_information() |  |
| } |  |

## Upsampling using signaled sampling grid offsets

The variables phaseXC and phaseYC are derived as follows:

phaseXC = chroma_phase_x_flag 	
phaseYC = chroma_phase_y

The variable xRef16 is derived as follows:

–	If cIdx is equal to 0, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX =  1 << ( iShiftX - 5 );

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – (horizontal_phase_offset8<<1);

–	Otherwise, the variable xRef16 is derived as follow:

iShiftX = 16;

iAddX = ( ( (PicWRL* phaseXC) << (iShiftX - 2) ) +(ScaledW >> 1) ) / ScaledW+ (1 << ( iShiftX - 5));

iDeltaX= 4* phaseXC;

iShiftXM4 = iShiftX - 4;

iScaleX = ( ( PicWRL << iShiftX ) + ( ScaledW >> 1 ) ) / ScaledW;

xRef16 = ( (xP*iScaleX + iAddX) >> iShiftXM4 ) – iDeltaX –horizontal_phase_offset8;

The variable yRef16 is derived as follows:

–	If cIdx is equal to 0, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY =  1 << ( iShiftY - 5  );

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16  = ( (yP*iScaleY + iAddY) >> iShiftYM4) –(vertical_phase_offset8<<1);

–	Otherwise, the variable yRef16 is derived as follow:

iShiftY = 16;

iAddY  = ( ( (PicHRL* phaseYC) << (iShiftY - 2) ) +(ScaledH >> 1) ) / ScaledH+ (1 << ( iShiftY - 5 ));

iDeltaY= 4*phaseYC;

iShiftYM4 = iShiftY - 4;

iScaleY = ( ( PicHRL << iShiftY ) + ( ScaledH >> 1 ) ) / ScaledH;

yRef16 = ((yP*iScaleY + iAddY) >> iShiftYM4) – iDeltaY – (vertical_phase_offset8);