| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0150 |
| --- | --- |

| Title: | AhG17: complexity analysis of SHM2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Aelxander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

This contribution contains performance and complexity analysis of SHM2.0 (RefIdx framework) compare to HEVC single layer coding. Complexity assessment methodology developed by AhG-17 was used. Complexity was evaluated assuming 2 layers scalable system.  Two different implementation: Picture base and PU based interlayer processing were studied. It is reported that memory access SHM2.0 in the worst case 200% (for PU-based inter-layer processing) and 218% (for Picture-based inter-layer processing) compare to single layer HEVC. In average memory access compare to single layer HEVC is 106-111%(spatial scalability tests) and 97-98% (SNR scalability tests) assuming PU-based inter-layer processing and 152-163% assuming Picture-based inter-layer processing.

# Introduction

As was agreed in Incheon meeting [1] the single layer HEVC decoder (for high resolution/quality layer) is natural comparison point for scalable HEVC extension. Complexity analysis below was made assuming 2 layers decoding. The memory access was evaluated for the worst case and in average (using actual bit-streams). In the absolutely worst for memory access case in single layer HEVC decoding is bi-prediction in 8×8 block.

Current SHVC working draft [1] contains only 2 major tools: interlayer texture prediction and motion vector prediction from base layer. So-called RefIdx framework is targeting for high-level only scalable HEVC extension solution. RefIdx framework supposes not to have changes compare to HEVC lower than in slice header.

Implementation-wise RefIdx approach can be implemented with picture based inter-layer processing. It means that whole reconstructed reference layer picture is re-sampled (if needed) and put into the reference list. No low-level modifications in HEVC decoder for dependent layer are needed in this case. But inter-layer processing shall be done for whole frame, even for regions where inter-layer texture prediction is not use. In the absolutely worst case whole both reference and dependent layers pictures are predicted using 8×8 bi-prediction and additionally inter-layer processing is performed. So for picture-based inter-layer processing in the worst case the complexity is higher than for 2 layers simulcast decoding. Inter-layer processing includes reading from reconstructed reference layer picture, re-sampling (if needed) and writing to the new picture which will be inter-layer reference picture for dependent layer. This processing the most likely will be implemented on block basis, with uniform block size. During discussion in Incheon meeting 32×32 block was suggested as reasonable.

Figure 1 demonstrates the worst case memory access for 2 layers decoding with picture-base inter-layer processing.

Figure 1. The worst case complexity analysis for picture-based inter-layer processing.

Figure 2. The worst case complexity analysis for PU-based inter-layer processing.

Another implementation option is inter-layer processing on a fly. If syntax tells decoder that reference picture is actually inter-layer reference then only PU size area should be processed by inter-layer filter. Assuming this implementation either inter-layer texture prediction or MC can be used for concrete PU. Situation when both MC interpolation and inter-layer filtering will be applied is not possible. So the absolutely worst case for this implementation is both reference and dependent layer pictures are predicted using 8×8 bi-directional motion compensation. So the worst case complexity here is identical to 2 layers simulcast decoding. Since inter-layer process is local in this case the writing memory access is not needed in inter-layer texture prediction process. The minimum block size for inter-layer processing is 4×8 for this implementation. Figure 2 demonstrates the worst case memory access for 2 layers decoding with picture-base inter-layer processing.

Writing of reconstructed picture was taken into account for both 1 layer HEVC and 2 layers SHVC decoders.

Motion compensation interpolation in HEVC and re-sampling interpolation filter in SHVC conceptually the same. 8 taps filters set is used for Luma and 4 taps filters set is used for Chroma interpolation. Precision for filter coefficients ad design principles are also the same for MC and re-sampling interpolation filters.

For inter-layer process in SNR case no interpolation is needed, just copying for reference layer reconstructed samples was assumed during complexity assessment. If reference layer and dependent layer pictures sizes are identical then in SNR case potentially one can redirect the pointer to reconstructed base layer picture and avoid copying and writing during inter-layer reference picture construction. But reference layer and dependent layer pictures sizes are not mandatory identical even in SNR scalability [3]. So redirecting the pointer to reconstructed base layer picture is impossible in specific cases only (cannot be used for the absolutely the worst case assessment). Results for pointer redirection are included here for information only.

# Test results

Released by AhG17 materials allow the worst case memory access calculation for different memory pattern. Results below reported assuming 8×2 memory pattern. Absolute numbers are different for another memory access pattern, but relative numbers are very similar.

Table 1 summarizes the worst case memory access including all mentioned above processes. Results are reported relatively to single layer decoder working at high resolution/quality layer. Results are reported separately for spatial scalability (assuming that scalability factor is not lower than 3/2) and for SNR scalability.

Table 1. The worst case memory access relatively to single layer HEVC decoding.

| spatial scalability ×1,5 |  | SNR scalability |  |  |
| --- | --- | --- | --- | --- |
| PU-based processing | Picture processing | PU-based processing | Picture processing (copy) | Picture processing (pointer) |
| 145% | 160% | 200% | 218% | 200% |

From Table 1 we can conclude that multi-loop SHVC 2-layers decoder is twice more complex compare to single layer decoder. This is double complexity cannot be avoided in multi-loop scalable system. As mentioned above the complexity for picture based inter-layer processing implementation is higher. Table 1 reports 15-18% extra memory access for picture based compare to PU based inter-layer processing.

Actual memory access was evaluated using SHM2.0 bit-streams and s/w module originally provided by NEC and modified for scalable SHVC extension [1]. Memory access was evaluated for 3 different memory architectures: pure memory access, DDR2 and DDR3.

Table 2. The actual memory access relatively to single layer HEVC decoding.

| Scalability |  |  | SPATIAL |  |  | SNR |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| IL Processing |  | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| PU-based processing |  | 111% | 106% | 106% | 97% | 98% | 98% |
| Picture processing |  | 163% | 152% | 158% | 161% | 158% | 161% |

Actual memory access relatively to HEVC single layer decoder is reported in Table 2. It is easy to notice that with PU-based inter-layer process implementation even 2 layers scalable decoder requires less memory access than high resolution/quality HEVC decoder. But for picture-based inter-layer process implementation scalable 2 layers decoding is 20-60% more complex than high resolution/quality HEVC decoder.

We have also compared two implementations: PU based (as anchor) and Picture based (as test) inter-layer process using actual memory access. This comparison is shown in Table 3. Numbers show that actual memory access increment is 48-66% if whole picture inter-layer processing is implemented.

Table 3.Actual memory access for Picture based inter-layer process relatively to PU-based processing.

|  | SPATIAL |  |  | SNR |  |
| --- | --- | --- | --- | --- | --- |
| Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| 148% | 146% | 152% | 166% | 161% | 164% |

# Comments on Inter-layer tools

Several tools in category of inter-layer filters were tested in SCE3. Table 3 summarizes performance and memory access measurement results for SCE3 and related tools. Both performance and memory access numbers are averaged across all intra, random access, low-delay-B and low-delay-P tests. One number (which is average memory access for 3 tested memory architectures) is reported for simplicity of comparison.

| Test | spatial |  |  |  |  |  | SNR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  | Memory Avg |  | Memory Worst |  | BD-rate |  | Memory Avg |  | Memory Worst |  |
|  | Luma | Chroma | PU | Pic | PU | Pic | Luma | Chroma | PU | Pic | PU | Pic |
| SHM2.0 | 0,00 | 0,00 | 107,5% | 157,4% | 145,0% | 160,0% | 0,00 | 0,00 | 97,8% | 160,2% | 200,0% | 218,0% |
| 3.1 fixed |  |  |  |  |  |  | -1,29 | -0,85 | 98,4% | 161,4% | 200,0% | 219,0% |
| 3.1Pic | -0,14 | -0,02 | 107,3% | 157,1% | 145,0% | 160,0% | -1,47 | -0,25 | 97,9% | 160,5% | 200,0% | 219,0% |
| 3.1PU | -0,18 | 0,01 | 105,9% | 218,2% | 145,0% | 204,0% | -1,85 | -0,12 | 97,1% | 219,7% | 200,0% | 237,0% |
| 3.1PicLn0 | -0,30 | -0,03 | 107,3% | 160,7% | 145,0% | 160,0% | -1,47 | -0,25 | 97,9% | 160,5% | 200,0% | 219,0% |
| 3.1PULn0 | -0,50 | 0,27 | 106,4% | 218,2% | 145,0% | 204,0% | -1,85 | -0,12 | 97,1% | 219,7% | 200,0% | 237,0% |
| 3.2AUF | -0,27 | -0,49 | 107,3% | 157,2% | 145,0% | 160,0% | -1,98 | -1,40 | 96,5% | 163,4% | 200,0% | 220,0% |
| 3.3ILSAO | -0,50 | -0,51 | 109,5% | 157,1% | 145,0% | 160,0% | -1,64 | -1,26 | 101,3% | 158,6% | 200,0% | 220,0% |
| 3.4.1Chr | -0,40 | -8,44 | 108,2% | 158,2% | 145,0% | 160,0% | -0,29 | -6,55 | 99,1% | 159,9% | 200,0% | 219,0% |
| 3.4.2ChrS | -0,40 | -8,23 | 109,0% | 159,3% | 145,0% | 160,0% | -0,29 | -6,54 | 99,1% | 164,3% | 200,0% | 219,0% |
| N0229 | -0,66 | -16,20 | 108,1% | 158,7% | 145,0% | 160,0% | -0,45 | -14,50 | 99,0% | 164,1% | 200,0% | 219,0% |
| 3.5.1BLF | -0,43 | -0,02 | 108,0% | 227,7% | 145,0% | 204,0% |  |  |  |  |  |  |

Memory access is reported relatively to HM10.1 single layer decoding of high resolution/quality layer. Two models for inter-layer processing (PU-based and Picture-based) were used for analysis.

In SHM2.0 additional inter-layer reference picture is introduced, which requires both read and writes memory access. As was mentioned above if inter-layer processing is implemented “on a fly” then average memory access of SHM2.0 is more or less equal to the memory access during single layer decoding. If whole picture is processed instead inter-layer processing “on a fly” then 50-60% extra memory access is needed in average.

Most of SCE3 tools use the same structure with SHM2.0 with additional processing of inter-layer reference picture. For these tools extra memory access due to additional inter-layer processing is 1-2% (both in the worst case and in average).

Three lines framed in Table 3 correspond to the tools implemented with one more inter-layer reference frame compare to SHM2.0. By doing this PU-based switching between two different inter-layer processing schemes was achieved. If inter-layer processing is implemented “on a fly” then there is no harm form this additional inter-layer reference (in the worst case memory access is equal to SHM2.0, in average variations is 2% compare to SHM2.0). But if whole picture inter-layer processing is implemented then 50-60% extra memory access is needed.

The conclusion one can derive from SCE3 tools comparison is following. RefIdx framework in SHVC allows 2 types of implementations: with PU and Picture based inter-layer processing. In terms of the worst case memory access the difference between two implementations is 15-18% (PU-based inter-layer processing is cheaper). In terms of average memory access 50-60% extra memory access is needed for every additional inter-layer reference picture if inter-layer processing is performed for whole picture. But with PU base inter-layer processing additional inter-layer reference pictures requires no additional memory access in average.

# Conclusions

# Memory access assessment results are reported for scalable 2 layers SHM2.0 system relatively to single layer HEVC decoder.

# In the worst case memory usage by scalable codec

# is twice higher compare to single layer decoder (assuming PU-based inter-layer processing)  equivalent 2 layers simulcast decoding;

# is ×2,2 higher compare to single layer decoder (assuming Picture-based inter-layer processing)  noticeable exceed 2 layers simulcast decoding.

# Actual memory access estimated using SHM2.0 bit-streams

# is ~3% higher compare to single layer decoder if PU-based inter-layer processing is implemented;

# is ~50% higher compare to single layer decoder if Picture-based inter-layer processing is implemented.

# Reference

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Incheon, South Korea, April 2013

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Working Draft 2,” JCT-VC document, JCTVC-M1008, Incheon, South Korea, April 2013.

- A. K. Ramasubramonian, X. Li, Y. Chen, V. Seregin (Qualcomm), “Signaling of extended spatial scalability for SHVC,” JCT-VC document, JCTVC-M0309, Incheon, South Korea, April 2013

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).