| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0151 |
| --- | --- |

| Title: | SCE3: Results of test 3.1 on switchable alternative inter-layer filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Vadim Seregin, Jianle Chen, Xiang Li, Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA Elena Alshina, Alexander Alshin, YongJin Cho 416 Maetan 3-Dong, Yeongtong-Gu, Suwon, Korea, 443-742 | Tel: Email: | +1-858-651-7749 Wei.Pu@qti.qualcomm.com +82 10 3026 1305 elena_a.alshin@samsung.com |
| Source: | Qualcomm Incorporated, Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

The integer position is not filtered in the up-sampling process of the current SHVC test model. This proposal provides simulation results for filtering 0-phase positions during inter-layer processing. Integer position filter coefficients are fixed. But filter is switchable. Two methods of switching are evaluated. In the first one, the encoder selectively enables the fixed smoothing filter for each picture and signals the decision using one bit in slice header. This scheme provides in 1.5% gain in SNR scalability test. In the second scheme the reconstructed regular interlayer reference picture and filtered with alternative filter interlayer reference picture are both inserted into the enhancement layer reference picture list. The second scheme provides in 1.9% gain in SNR scalability test. Additionally switching between SHM2.0 re-sampling filter and alternative one (essentially different spectral characteristics) was investigated in spatial scalability tests. For spatial scalability first scheme (Picture based switching) provides in average 0,5% luma BD-rate gain; the second scheme (PU based switching) provides in average 0,3% BD-rate gain for Luma.

# Introduction

De-noising filter for SNR scalability was proposed in [1], switchable design for de-noising filter was studied in [2]. This technique is included into SCE3 [3].

# Algorithm description

Trivial up-sampling filter[0 0 0 64 0 0 0 0]/64 for integer position (phase zero) is used in SHM2.0. The five tap smoothing filter

[-1, 3, 12, 3,−1]/16						 (1)

On integer position proposed in JCTVC-M0087 [1] is used as the alternative filter in this proposal. In [1] de-noising filter (1) was always applied for inter-layer texture prediction in SNR tests for refIdx framework. In TextureRL framework (1) was adaptively switched on and off on CU basis. So the gain from [1] is higher in TextureRLframework due to the switching.

The goal of this contribution to extend possibility of switching de-noising filter on and off for RefIdx framework, w/o low-level syntax and decoding process changes.

Additionally for spatial scalability, switching between SHM2.0 re-sampling filter and alternative one is investigated. Alternative filter for 0-phase positions is the sane with SNR case (1). For fractional positions alternative re-sampling filters were designed as combination of SHM2.0 re-sampling filter and de-noising.

## Picture Level Switchable Filter

The encoder determines whether the pixels in integer positions are filtered or not according to the HAD of the integer position re-sampled reference layer picture and the integer position un-filtered interlayer reference picture. It should be noticed that encoder is implemented using fast search (with-out multi-pass encoding fro whole picture) and so gain shown is not the maximum possible gain can be achieved with picture base switching.

## PU-level Switchable Filter with Two Interlayer Reference Pictures

Both the regular interlayer reference picture and the filtered with alternative filter interlayer reference pictures are inserted into the reference picture list for the enhancement layer picture coding. In case of SNR scalability regular inter-layer reference picture is just reconstructed base layer; alternative inter-layer reference picture is reconstructed base layer picture filtered with de-noising filter (1).

If the current enhancement layer slice is a P slice, both of the two interlayer reference pictures are added to the end of list0.

If the current enhancement layer slice is a B slice, the default interlayer reference picture is added to the end of reference list 0 and the alternative filtered interlayer reference picture is added after it. In reference list 1, the alternative filtered interlayer reference picture is added to the end and the default interlayer reference picture is added after it.

As a result de-noising filter is switched on and off on PU basis; but no block-level syntax elements are added.

# Experimental Results

Performance tests.

Both schemes: Picture based and PU-based switching were integrated into SHM-2.0. The R-D performance is tested under the common SHVC test conditions [4] using AI, LDP, LDB and RA tests. In addition, the performance in SNR test for non-switchable de-noising filter is provided for comparison. The brief summary of test results is presented in Table 1. Average numbers for BD rate calculated in AI, LDP, LDB and RA tests are reported.

In SNR tests the main gain (BD-rate reduction1.3%) is provided by just applying non-switchable de-noising filter. Since in SNR case reference layer is encoded with lower quality (higher QP) a lot of coding noise is present and de-noising improves inter-layer prediction quality.

At the same time small performance degradation was observed for BasketBallDrive and BQTerrace if de-noising filter is always applied during inter-layer texture prediction.

This problem is resolved by allowing Picture based switching. Average BD-rate reduction in SNR test reaches 1.5%.

Performance has been improved even further if PU based switching is allowed. Average BD-rate reduction in SNR test reaches 1.9% with PU-based switching. In this case more bits are needed to indicate which inter-layer reference is used, but more accurate prediction compensates losses due to these extra bits in signalling.

In case of spatial scalability switching between regular and alternative re-sampling filters brings 0,3% BD-rate reduction for Picture based switching and 0,5% BD-rate reduction for PU-base switching.

In both tests only Luma filtering was modified. Chroma BD-rate variations are reported just to confirm that there is no gain shift between luma and chroma.

Table 1. Performance comparison for different designs of alternative inter-layer filtering.

|  | SNR |  | ×2 |  | ×1,5 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| Non-switchable | -1,3% | -0,9% | × | × | × | × |
| Pic. based (0-phase) | -1,5% | -0,2% | -0,1% | 0,0% | -0,1% | 0,0% |
| PU based (0-phase) | -1,9% | -0,1% | -0,2% | 0,1% | -0,1% | 0,1% |
| Pic. based (all phases) | -1,5% | -0,2% | -0,3% | 0,0% | -0,3% | 0,0% |
| PU based (all phases) | -1,9% | -0,1% | -0,5% | 0,2% | -0,5% | 0,3% |

In Tables 2 and 3 details of test results for switchable alternative 0-phase Luma inter-layer filters can be found.

If additionally no-zero phase Luma filter is switchable then results are summarised in Tables 4 and 5.

Complexity assessment.

The complexity assessment modules in the source code [5] and the worst case calculation in the Excel template [5] were modified specially for the proposed algorithm. Results for complexity assessment are summarized in Table 2 and 3.

SHM2.0. If inter-layer processing in implemented on PU-basis (on a fly) then for SHm2.0 in the worst case memory access is needed for following processes:

- 8×8 bi-prediction MC on a base layer

- reconstructed signal writing  (base layer resolution)

- 8×8 bi-prediction MC on a enhancement layer with MV mapping (for the block)

- reconstructed signal writing (enhancement layer resolution)

If inter-layer processing in implemented on Picture-basis then for SHM2.0 in the worst case memory access is needed for following processes:

- 8×8 bi-prediction MC on a base layer

- reconstructed signal writing  (base layer resolution)

- SHM2.0 inter-layer processing for reconstructed BL picture

- writing of SHM2.0  inter-layer reference picture

- Alternative inter-layer processing for reconstructed BL picture

- Writing of alternative  inter-layer reference picture

- MV mapping for whole picture

Picture based switchable of de-noising filter. If inter-layer processing in implemented on PU-basis (on a fly) then for PU based switchable of de-noising filter in the worst case memory access is needed for following processes:

- 8×8 bi-prediction MC on a base layer

- reconstructed signal writing  (base layer resolution)

- 8×8 bi-prediction MC on a enhancement layer with MV mapping (for the block)

- reconstructed signal writing (enhancement layer resolution)

The inter-layer filter in proposed design is no more complex than 8×8 bi-prediction MC from view point of memory access, so the worst case memory access is the same with SHM2.0.

If inter-layer processing in implemented on Picture-basis then for PU based switchable of de-noising filter in the worst case memory access is needed for following processes:

- 8×8 bi-prediction MC on a base layer

- reconstructed signal writing  (base layer resolution)

- Alternative inter-layer processing for reconstructed BL picture (**)

- Writing of alternative  inter-layer reference picture (***)

- MV mapping for whole picture

- 8×8 bi-prediction MC on a enhancement layer

- reconstructed signal writing (enhancement layer resolution)

Changes compare to SHM2.0 is marked by (**).

PU based switchable of de-noising filter. If inter-layer processing in implemented on PU-basis (on a fly) then for PU based switchable of de-noising filter in the worst case memory access is needed for following processes:

- 8×8 bi-prediction MC on a base layer

- reconstructed signal writing  (base layer resolution)

- 8×8 bi-prediction MC on a enhancement layer with MV mapping (for the block)

- reconstructed signal writing (enhancement layer resolution)

The inter-layer filter in proposed design is no more complex than 8×8 bi-prediction MC from view point of memory access, so the worst case memory access is the same with SHM2.0.

If inter-layer processing in implemented on Picture-basis then for PU based switchable of de-noising filter in the worst case memory access is needed for following processes:

- 8×8 bi-prediction MC on a base layer

- reconstructed signal writing  (base layer resolution)

- SHM2.0 inter-layer processing for reconstructed BL picture

- writing of SHM2.0  inter-layer reference picture

- Alternative inter-layer processing for reconstructed BL picture (***)

- Writing of alternative  inter-layer reference picture (***)

- MV mapping for whole picture

- 8×8 bi-prediction MC on a enhancement layer

- reconstructed signal writing (enhancement layer resolution)

Extra memory access compare to SHM2.0 is marked by (***).

Table 2. The worst case memory usage relative to HEVC single layer decoder.

|  | SNR |  | ×1,5 |  |
| --- | --- | --- | --- | --- |
| Inter-layer processing | PU | Picture | PU | Picture |
| SHM2.0 | 200% | 218% | 145% | 160% |
| Non-switchable | 200% | 219% | 145% | 160% |
| Pic. based on/off | 200% | 219% | 145% | 160% |
| PU based on/off | 200% | 237% | 145% | 160% |

Table 3. Average memory usage compare to HEVC single layer decoder.

| SNR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-layer processing | PU |  |  | Picture |  |  |
| Architecture | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| SHM2.0 | 97% | 98% | 98% | 161% | 158% | 161% |
| Non-switchable | 99% | 98% | 98% | 163% | 159% | 163% |
| Pic. based (0-phase) | 98% | 98% | 98% | 162% | 158% | 162% |
| PU based (0-phase) | 97% | 97% | 97% | 222% | 215% | 222% |
| Pic. based (all phases) | 98% | 98% | 98% | 162% | 158% | 162% |
| PU based (all phases) | 97% | 97% | 97% | 222% | 215% | 222% |

| Spatial |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-layer processing | PU |  |  | Picture |  |  |
| Architecture | Pure | DDR2 | DDR3 | Pure | DDR2 | DDR3 |
| SHM2.0 | 93% | 93% | 93% | 141% | 141% | 145% |
| Pic. based (0-phase) | 80% | 80% | 79% | 121% | 120% | 124% |
| PU based (0-phase) | 80% | 79% | 79% | 161% | 161% | 168% |
| Pic. based (all phases) | 80% | 80% | 79% | 121% | 120% | 124% |
| PU based (all phases) | 80% | 79% | 79% | 161% | 161% | 168% |

Visual quality evaluation.

There are several examples for subjective visual quality improvement below. If base layer is encoded with poor quality (which always the case of SNR test) then it contains coding artifacts which might propagate to enhancement layer during inert-layer texture prediction.

Figure 1 shows the part reconstructed enhancement layer picture with (right) and without (left) de-noising filter applied to inter-layer texture prediction. Noise reduction is quite visible.

Fig. 1. Part of decoded PeopleOnStreet sequence (SNR RA test, QP0=34, QP1=30, POC =88): left – SHM2.0, right PU based switchable de-noising filter is enabled.

On Figure 2 additionally reconstructed base layer is shown (on the left). It contains serious artifact which is less but visible on reconstructed enhancement layer w/o de-noising of inter-later reference. This artifact disappears if de-noising filter is applied during inter-layer texture prediction.

Fig. 2. Part of decoded PeopleOnStreet sequence (SNR LDP test, QP0=38, QP1=34, POC =129): left – SHM2.0 (base layer), middle - SHM2.0 (enhancement layer); right - PU based switchable de-noising filter is enabled.

# Specification Text

Specification text modification for tested algorithms includes:

- For all versions

  - Non-trivial zero-phase filter

- For non-switchable :

  - sps-extension flag which enables this tool

- For picture-based switchable:

  - sps-extension flag which enables this tool

  - slice-segment header flag which tells decoder whether de-noising of inter-layer reference picture is required

- For PU-based switchable:

  - sps-extension flag which enables this tool

  - specification for reference list construction

Specification text modification can be found in JCTVC-M0087 [1] and JCTVC-M0273 [2] packages.

# Acknowledgements

Authors would like to thank Inter Digital, Sharp and Nokia for comprehensive cross-check of multiple tests and valuable comments.

# Conclusion

Switchable alternative Luma interlayer filter was tested in this proposal. Two variants of design, Picture based and PU based switching, were investigated. For only zero-phase alternative Luma filter BD-rate gain in SNR test is 1.5% and 1.9% for Picture and PU based switching respectively. In spatial scalability test the gain is 0.1% and 0.2% for Picture and PU based switching respectively with only zero-phase alternative luma filter. If alternative re-sampling filter is allowed for all phases then gain is 0,3% and 0,5% for Picture and PU based switching respectively.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- A. Alshin and E. Alshina, “SCE4: De-noising filter for SNR scalability,” JCTVC-M0087, Incheon, KR, 18–26 Apr. 2013

- W. Pu, V.Seregin, ,J.Chen, X.Li, M. Karczewicz . A. Alshin and E. Alshina, “Non-SCE4: Switchable Filter on Integer Position,” JCTVC-M0273, Incheon, KR, 18–26 Apr. 2013.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, “Description of HEVC Scalable Extension Core Experiment SCE3: Inter-layer filtering,” JCTVC-M1103, Incheon, South Korea, April 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” JCTVC-M1009, Incheon, South Korea, April 2013.

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Incheon, South Korea, April 2013.

# Test results details

| Table 4. Test results for Picture based switchable inter-layer filter (only 0-phase alternative Luma filter). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Test vs single layer) | 12,7% | 14,9% | 14,6% | 10,4% | 9,8% | 9,3% |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |  |  |  |  |  |  |  |  |  |  |  |  |
| EL only (Test vs Ref) | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% |  |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100,4% |  |  | 100,0% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 97,6% |  |  | 98,7% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |  |  |  |  |  |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  | -3,6% | -0,8% | -0,8% |  |  |  |  |  |  |  |  |  |
| Class B | -0,2% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,4% | -0,1% | 0,0% |  |  |  |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0,1% | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -1,3% | -0,3% | -0,2% |  |  |  |  |  |  |  |  |  |
| Overall (Test vs single layer) | 19,1% | 33,3% | 31,9% | 16,1% | 28,8% | 29,1% | 12,9% | 31,7% | 33,8% |  |  |  |  |  |  |  |  |  |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% | 14,4% | 32,1% | 34,1% |  |  |  |  |  |  |  |  |  |
| EL only (Test vs Ref) | -0,2% | -0,1% | -0,1% | -0,2% | 0,0% | 0,0% | -2,0% | -0,9% | -0,8% |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100,4% |  |  | 100,0% |  |  | 99,8% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 96,1% |  |  | 97,4% |  |  | 117,4% |  |  |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |  |  |  |  |  |  |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |  |  |  |  |  |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  | -3,3% | -0,8% | -0,9% |  |  |  |  |  |  |  |  |  |
| Class B | -0,2% | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | -0,2% | 0,0% | 0,0% |  |  |  |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0,1% | 0,0% | -0,1% | -0,1% | 0,0% | 0,0% | -1,1% | -0,2% | -0,3% |  |  |  |  |  |  |  |  |  |
| Overall (Test vs single layer) | 28,4% | 38,9% | 39,6% | 24,6% | 33,0% | 35,9% | 22,9% | 34,4% | 39,1% |  |  |  |  |  |  |  |  |  |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% | 24,3% | 34,7% | 39,5% |  |  |  |  |  |  |  |  |  |
| EL only (Test vs Ref) | -0,2% | -0,1% | -0,1% | -0,2% | -0,1% | -0,1% | -1,6% | -0,7% | -0,7% |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100,3% |  |  | 99,9% |  |  | 99,6% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 96,2% |  |  | 96,7% |  |  | 117,6% |  |  |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |  |  |  |  |  |  |  |  |
|  | -0,1% | 0,0% |  | -0,1% | 0,0% |  | -1,2% | -0,3% |  |  |  |  |  |  |  |  |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |  |  |  |  |  |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  | -4,9% | -1,0% | -0,8% |  |  |  |  |  |  |  |  |  |
| Class B | -0,3% | 0,0% | 0,0% | -0,2% | 0,0% | -0,1% | -0,9% | 0,0% | 0,1% |  |  |  |  |  |  |  |  |  |
| Overall (Test vs Ref) | -0,3% | 0,0% | 0,0% | -0,2% | 0,0% | -0,1% | -2,0% | -0,3% | -0,2% |  |  |  |  |  |  |  |  |  |
| Overall (Test vs single layer) | 26,3% | 38,0% | 39,1% | 22,6% | 32,8% | 35,5% | 20,9% | 34,2% | 39,2% |  |  |  |  |  |  |  |  |  |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% | 23,4% | 34,6% | 39,4% |  |  |  |  |  |  |  |  |  |
| EL only (Test vs Ref) | -0,3% | -0,1% | -0,1% | -0,3% | -0,1% | -0,1% | -2,8% | -1,0% | -0,9% |  |  |  |  |  |  |  |  |  |
| Enc Time[%] | 100,4% |  |  | 99,8% |  |  | 99,7% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 96,2% |  |  | 97,0% |  |  | 124,9% |  |  |  |  |  |  |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |  |  |  |  |  |  |  |  |  |
|  | -0,1% | 0,0% |  | -0,1% | 0,0% |  | -1,5% | -0,2% |  |  |  |  |  |  |  |  |  |  |

Table 5. Test results for PU based switchable inter-layer filter (only 0-phase alternative Luma filter).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0,1% | 0,2% | 0,2% |  |  |  |  |  |  |
| Class B | -0,2% | 0,2% | 0,2% | -0,1% | 0,1% | 0,2% |  |  |  |
| Overall (Test vs Ref) | -0,2% | 0,2% | 0,2% | -0,1% | 0,1% | 0,2% |  |  |  |
| Overall (Test vs single layer) | 12,5% | 15,1% | 14,8% | 10,4% | 9,9% | 9,5% |  |  |  |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |  |  |  |
| EL only (Test vs Ref) | 0,0% | 0,4% | 0,4% | 0,3% | 0,6% | 0,7% |  |  |  |
| Enc Time[%] | 113,4% |  |  | 110,2% |  |  |  |  |  |
| Dec Time[%] | 116,4% |  |  | 120,2% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,1% | 0,2% |  |  |  | -3,7% | -0,7% | -0,6% |
| Class B | -0,2% | 0,2% | 0,2% | -0,1% | 0,2% | 0,2% | -1,0% | 0,1% | 0,4% |
| Overall (Test vs Ref) | -0,2% | 0,2% | 0,2% | -0,1% | 0,2% | 0,2% | -1,7% | -0,1% | 0,1% |
| Overall (Test vs single layer) | 19,0% | 33,5% | 32,3% | 16,1% | 29,1% | 29,5% | 12,4% | 31,9% | 34,2% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -0,1% | 0,3% | 0,3% | 0,3% | 0,6% | 0,6% | -2,5% | -0,8% | -0,5% |
| Enc Time[%] | 106,4% |  |  | 105,4% |  |  | 103,6% |  |  |
| Dec Time[%] | 123,2% |  |  | 125,5% |  |  | 135,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,1% | 0,1% | 0,1% |  |  |  | -3,2% | -0,8% | -0,8% |
| Class B | -0,2% | 0,0% | 0,0% | -0,1% | 0,1% | 0,0% | -0,7% | -0,1% | 0,1% |
| Overall (Test vs Ref) | -0,2% | 0,1% | 0,0% | -0,1% | 0,1% | 0,0% | -1,4% | -0,3% | -0,2% |
| Overall (Test vs single layer) | 28,3% | 39,1% | 39,8% | 24,7% | 33,2% | 35,9% | 22,5% | 34,3% | 39,2% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% | 24,3% | 34,7% | 39,5% |
| EL only (Test vs Ref) | -0,1% | 0,1% | 0,1% | 0,2% | 0,3% | 0,2% | -2,0% | -0,8% | -0,7% |
| Enc Time[%] | 104,8% |  |  | 103,6% |  |  | 102,5% |  |  |
| Dec Time[%] | 122,4% |  |  | 124,3% |  |  | 131,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0,2% | 0,1% |  | -0,1% | 0,1% |  | -1,6% | -0,1% |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,2% | 0,0% | 0,1% |  |  |  | -4,8% | -1,1% | -0,8% |
| Class B | -0,4% | 0,0% | -0,2% | -0,3% | 0,0% | 0,1% | -1,4% | 0,1% | 0,4% |
| Overall (Test vs Ref) | -0,4% | 0,0% | -0,1% | -0,3% | 0,0% | 0,1% | -2,4% | -0,2% | 0,0% |
| Overall (Test vs single layer) | 26,1% | 38,0% | 39,0% | 22,5% | 32,8% | 35,7% | 20,4% | 34,3% | 39,5% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% | 23,4% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -0,4% | 0,0% | -0,1% | -0,1% | 0,2% | 0,2% | -3,3% | -1,1% | -0,8% |
| Enc Time[%] | 105,6% |  |  | 104,3% |  |  | 103,0% |  |  |
| Dec Time[%] | 124,6% |  |  | 126,8% |  |  | 136,7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0,2% | 0,1% |  | -0,1% | 0,1% |  | -1,9% | -0,1% |  |

Table 6. Test results for Picture based switchable inter-layer filter (both zero and non-zero phase alternative Luma filter).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  |  |  |  |
| Class B | -0,2% | 0,0% | 0,0% | -0,2% | 0,0% | 0,0% |  |  |  |
| Overall (Test vs Ref) | -0,2% | 0,0% | 0,0% | -0,2% | 0,0% | 0,0% |  |  |  |
| Overall (Test vs single layer) | 12,6% | 14,9% | 14,6% | 10,3% | 9,8% | 9,3% |  |  |  |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |  |  |  |
| EL only (Test vs Ref) | -0,2% | 0,0% | 0,0% | -0,2% | 0,0% | 0,0% |  |  |  |
| Enc Time[%] | 100,4% |  |  | 99,9% |  |  |  |  |  |
| Dec Time[%] | 97,7% |  |  | 99,1% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  | -3,6% | -0,8% | -0,8% |
| Class B | -0,3% | 0,0% | 0,0% | -0,3% | 0,0% | 0,0% | -0,4% | -0,1% | 0,0% |
| Overall (Test vs Ref) | -0,2% | 0,0% | 0,0% | -0,3% | 0,0% | 0,0% | -1,3% | -0,3% | -0,2% |
| Overall (Test vs single layer) | 18,9% | 33,3% | 31,9% | 15,9% | 28,8% | 29,1% | 12,9% | 31,7% | 33,8% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -0,3% | -0,1% | -0,1% | -0,4% | -0,1% | -0,1% | -2,0% | -0,9% | -0,8% |
| Enc Time[%] | 100,3% |  |  | 100,0% |  |  | 99,8% |  |  |
| Dec Time[%] | 95,7% |  |  | 97,0% |  |  | 117,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |  |  |  | -3,3% | -0,8% | -0,9% |
| Class B | -0,3% | -0,1% | -0,1% | -0,3% | 0,0% | 0,0% | -0,2% | 0,0% | 0,0% |
| Overall (Test vs Ref) | -0,3% | 0,0% | 0,0% | -0,3% | 0,0% | 0,0% | -1,1% | -0,2% | -0,3% |
| Overall (Test vs single layer) | 28,2% | 38,9% | 39,6% | 24,4% | 33,0% | 35,9% | 22,9% | 34,4% | 39,1% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% | 24,3% | 34,7% | 39,5% |
| EL only (Test vs Ref) | -0,3% | -0,1% | -0,1% | -0,3% | -0,1% | -0,1% | -1,6% | -0,7% | -0,7% |
| Enc Time[%] | 100,2% |  |  | 99,8% |  |  | 99,6% |  |  |
| Dec Time[%] | 95,6% |  |  | 96,8% |  |  | 117,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0,2% | 0,0% |  | -0,2% | 0,0% |  | -1,2% | -0,3% |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,1% | 0,0% | 0,0% |  |  |  | -4,9% | -1,0% | -0,8% |
| Class B | -0,7% | -0,1% | -0,1% | -0,5% | 0,0% | 0,0% | -0,9% | 0,0% | 0,1% |
| Overall (Test vs Ref) | -0,5% | -0,1% | -0,1% | -0,5% | 0,0% | 0,0% | -2,0% | -0,3% | -0,2% |
| Overall (Test vs single layer) | 26,0% | 37,9% | 39,0% | 22,2% | 32,8% | 35,6% | 20,9% | 34,2% | 39,2% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% | 23,4% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -0,6% | -0,2% | -0,2% | -0,6% | -0,1% | -0,2% | -2,8% | -1,0% | -0,9% |
| Enc Time[%] | 100,2% |  |  | 99,8% |  |  | 99,7% |  |  |
| Dec Time[%] | 95,7% |  |  | 97,2% |  |  | 124,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0,3% | 0,0% |  | -0,3% | 0,0% |  | -1,5% | -0,2% |  |

Table 7. Test results for PU based switchable inter-layer filter (both zero and non-zero phase alternative Luma filter).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -0,2% | 0,6% | 0,5% |  |  |  |  |  |  |
| Class B | -0,4% | 0,3% | 0,3% | -0,4% | 0,4% | 0,4% |  |  |  |
| Overall (Test vs Ref) | -0,4% | 0,4% | 0,4% | -0,4% | 0,4% | 0,4% |  |  |  |
| Overall (Test vs single layer) | 12,3% | 15,3% | 15,0% | 10,1% | 10,2% | 9,8% |  |  |  |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |  |  |  |
| EL only (Test vs Ref) | -0,1% | 0,7% | 0,7% | 0,3% | 1,2% | 1,2% |  |  |  |
| Enc Time[%] | 113,7% |  |  | 110,6% |  |  |  |  |  |
| Dec Time[%] | 116,2% |  |  | 119,4% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,2% | 0,5% | 0,6% |  |  |  | -3,7% | -0,7% | -0,6% |
| Class B | -0,5% | 0,3% | 0,4% | -0,4% | 0,4% | 0,4% | -1,0% | 0,1% | 0,4% |
| Overall (Test vs Ref) | -0,4% | 0,4% | 0,4% | -0,4% | 0,4% | 0,4% | -1,7% | -0,1% | 0,1% |
| Overall (Test vs single layer) | 18,7% | 33,8% | 32,6% | 15,7% | 29,4% | 29,7% | 12,4% | 31,9% | 34,2% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -0,3% | 0,5% | 0,6% | 0,1% | 1,0% | 1,0% | -2,5% | -0,8% | -0,5% |
| Enc Time[%] | 106,5% |  |  | 105,2% |  |  | 103,6% |  |  |
| Dec Time[%] | 123,6% |  |  | 125,1% |  |  | 135,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,2% | 0,3% | 0,3% |  |  |  | -3,2% | -0,8% | -0,8% |
| Class B | -0,5% | 0,1% | 0,1% | -0,4% | 0,1% | 0,1% | -0,7% | -0,1% | 0,1% |
| Overall (Test vs Ref) | -0,4% | 0,2% | 0,2% | -0,4% | 0,1% | 0,1% | -1,4% | -0,3% | -0,2% |
| Overall (Test vs single layer) | 28,0% | 39,2% | 40,0% | 24,3% | 33,2% | 36,0% | 22,5% | 34,3% | 39,2% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% | 24,3% | 34,7% | 39,5% |
| EL only (Test vs Ref) | -0,3% | 0,2% | 0,2% | 0,0% | 0,4% | 0,4% | -2,0% | -0,8% | -0,7% |
| Enc Time[%] | 104,9% |  |  | 103,7% |  |  | 102,5% |  |  |
| Dec Time[%] | 122,0% |  |  | 124,6% |  |  | 131,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0,4% | 0,3% |  | -0,4% | 0,3% |  | -1,6% | -0,1% |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,4% | 0,4% | 0,4% |  |  |  | -4,8% | -1,1% | -0,8% |
| Class B | -1,0% | -0,1% | -0,1% | -0,9% | 0,1% | 0,3% | -1,4% | 0,1% | 0,4% |
| Overall (Test vs Ref) | -0,8% | 0,0% | 0,0% | -0,9% | 0,1% | 0,3% | -2,4% | -0,2% | 0,0% |
| Overall (Test vs single layer) | 25,5% | 38,1% | 39,2% | 21,7% | 33,0% | 36,0% | 20,4% | 34,3% | 39,5% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% | 23,4% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -0,9% | 0,0% | 0,0% | -0,6% | 0,4% | 0,5% | -3,3% | -1,1% | -0,8% |
| Enc Time[%] | 105,7% |  |  | 104,4% |  |  | 103,0% |  |  |
| Dec Time[%] | 123,8% |  |  | 126,4% |  |  | 136,7% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | -0,5% | 0,2% |  | -0,5% | 0,3% |  | -1,9% | -0,1% |  |

Table 8. Test results for non-switchable inter-layer filter (based on JCTVC-M0089).

|  | RA HEVC SNR |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -3,6% | -0,8% | -0,8% |
| Class B | 0,1% | -0,8% | -0,9% |
| Overall (Test vs Ref) | -0,9% | -0,8% | -0,9% |
| Overall (Test vs single layer) | 13,3% | 31,1% | 33,1% |
| Overall (Ref vs single layer) | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -1,3% | -1,3% | -1,3% |
| Enc Time[%] | 100,2% |  |  |
| Dec Time[%] | 127,0% |  |  |
| BL Match | Matched |  |  |
|  | LD-B HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -3,3% | -0,7% | -0,9% |
| Class B | -0,1% | -1,0% | -1,1% |
| Overall (Test vs Ref) | -1,0% | -0,9% | -1,1% |
| Overall (Test vs single layer) | 23,1% | 33,5% | 38,0% |
| Overall (Ref vs single layer) | 24,3% | 34,7% | 39,5% |
| EL only (Test vs Ref) | -1,3% | -1,3% | -1,4% |
| Enc Time[%] | 100,0% |  |  |
| Dec Time[%] | 125,1% |  |  |
| BL Match | Matched |  |  |
|  | -1,0% | -0,9% |  |
| Optional Tests |  |  |  |
|  | LD-P HEVC SNR |  |  |
|  | Y | U | V |
| Class A | -4,9% | -1,0% | -0,8% |
| Class B | -0,8% | -0,6% | -0,7% |
| Overall (Test vs Ref) | -2,0% | -0,7% | -0,7% |
| Overall (Test vs single layer) | 20,9% | 33,7% | 38,4% |
| Overall (Ref vs single layer) | 23,4% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -2,7% | -1,4% | -1,4% |
| Enc Time[%] | 100,1% |  |  |
| Dec Time[%] | 127,2% |  |  |