| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0152 |
| --- | --- |

| Title: | SCE3: Results of test 3.4.2 on simplified cross-color inter-layer inter layer prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina, Alexander Alshin, YongJin Cho 416 Maetan 3-Dong, Yeongtong-Gu, Suwon, Korea, 443-742 Jie Dong, Yan Ye, Yuwen He 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +82 10 3026 1305 elena_a.alshin@samsung.com +1 858.210.4809 jie.dong@interdigital.com |
| Source: | Samsung Electronics, Ltd , InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes simplifications for cross-color inter layer prediction. Original design of cross-color inter layer prediction uses re-sampled Luma signal in order to enhance prediction quality of re-sampled Chroma signal. This leads to undesired latency. In proposed modification during Chroma signal enhancement Luma signal from reference layer (before re-sampling) is used. It allows independent inter-layer processing for Luma and Chroma planes. Additionally minor high-level syntax simplification for cross-color inter-layer filter parameters encoding is proposed. The average {Y, U, V} gains achieved on top of SHM2.0 are {-0.8%, -7.5%, -8.5%}, {-0.4%, -9.2%, -9.1%}, {-0.2%, -7.1%, -7.5%}, and {-0.2%, -6.5%, -6.9%} for AI, RA, LDB, and LDP, respectively.

# Introduction

Chroma enhancement for interlayer prediction was proposed in January, 2013[1] and studied in SCE at the last meeting [2]. Specifically, for each Chroma pixel, an appropriate offset which is calculated by linear combination of the surrounding 3×4 luma pixels values is added. Since additional filter is applied for Chroma component only after re-sampling process and re-sampled Luma signal is the input for this process there is un-desirable latency during inter-layer processing. In order to resolve this delay and allow independent inter-layer processing for Luma and Chroma referring to base layer Luma samples was proposed [3] and tested in SCE [4].

Parameters for Chroma enhancement are selected in encoder side for each picture and signalled to the decoder in first independent slice segment header. Simplification for this syntax is proposed in [3]. As result of these simplifications duplicated functionality of syntax elements and one condition check are removed. Additionally variable de-scaling shift is replaced by constant shift which reduces the number of parameters to be signalled and helps efficient implementation.

# Proposed Simplifications for Chroma Enhancement in Inter-layer prediction

Simplification A.

The goal for simplifications in this group is to simplify samples processing, remove duplicated functionality of high-level syntax elements; remove condition check and reduce amount of bits to be encoded in independent slice segment header.

Error: Reference source not found shows the relative luma and chroma pixel positions in 4:2:0 color format. Each chroma pixel (red star) in an ILR picture is enhanced by adding an appropriate offset, where the offset is the result of filtering the surrounding 3×4 luma pixels (blue circles) by 2D non-separable FIR:

.                                           (1)

Values ,  and  are signaled in slice-header for Cb and Cr (separately). Additionally tool can be switched On and Off Cb and Cr separately (with corresponding flag encoding in slice header).

Fig. 1. Cross-color inter-layer filter: circles – Luma, stars –Chroma, red star – current Chroma sample, red dots surround Luma samples taken into account for current Chroma sample processing; green dots surround Luma samples belong to the same block with corrected Chroma sample.

Variable shift is not friendly for implementation. According to our observations in majority of cases  is used. We propose to fix this value equal to 16. This simplifies implementation and saves 10 bits in slice header (in original design  was encoded using 5 bits fixed length code).

If slice level on/off flag for this tool is false then Chroma samples will not be modified. The same will happen if  in (1) is equal to 0. In order to avoid duplicated functionality of syntax elements in a slice header we suggest modifying coding for:

- In original design

  - is encoded using 10-bits fixed length code; if this value is not zero the sign of  was signaled (1 bit flag).

- In proposed design

  - is encoded using 10-bits fixed length code and then sign is always signaled (one condition check during parsing stage is removed).

In terms of specification text proposed modification reduces amount of bits to parse, simplifies parsing since 2 condition checks are removed.  Specification text is provided in Section 7.

Simplification B.

The goal for this simplification is to remove undesired latency and allow independent inter-layer processing for Luma and Chroma components.

SHM2.0 uses inter-layer texture prediction which is independent for Luma and Chroma.

The block diagram for original design of cross-color inter-layer filter is shown Fig. 2. In order to start cross-color filtering of Chroma decoder must wait until up-sampling for Luma component will be finished.

Fig. 2. Original design

Fig. 3. Proposed design.

We would suggest using reconstructed Luma samples from base-layer (before re-sampling) instead up-sampled Luma signal. In this case Luma and Chroma inter-layer texture prediction can be done in parallel. And there is no need for decoder to wait until Luma up-sampling is finished during inter-layer Chroma texture prediction.

# Experimental Results

Performance tests.

The proposed in [1] Chroma enhancement algorithm and proposed here simplification were integrated into SHM-2.0. The R-D performance is tested under the common SHVC test conditions [5]. Summary of test results for original and simplified designs is presented in Table 1. Performance change due to proposed simplifications is marginal.

Table 1. Performance comparison for original and simplified version of cross-color inter-layer prediction.

| SCE3 test | Avg. BD-rate in AI, LDB and RA |  |  | Avg. BD-rate in AI, LDB, RA and LDP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| 3.4.1 (original) | -0,42% | -7,91% | -8,76% | -0,37% | -7,51% | -8,33% |
| 3.4.2 (simplified) | -0,42% | -7,98% | -8,35% | -0,37% | -7,59% | -7,95% |

In Table 2 details of test results for simplified version can be found.

Table 2. Performance test results for proposed simplified design of cross-color inter-layer prediction.

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |  |  |
| Class A | -1,1% | -9,5% | -6,9% |  |  |  |  |  |  |
| Class B | -0,9% | -6,9% | -8,6% | -0,7% | -7,3% | -8,9% |  |  |  |
| Overall (Test vs Ref) | -0,9% | -7,6% | -8,1% | -0,7% | -7,3% | -8,9% |  |  |  |
| Overall (Test vs single layer) | 11,7% | 6,3% | 5,8% | 9,8% | 2,0% | 0,3% |  |  |  |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 10,5% | 9,8% | 9,3% |  |  |  |
| EL only (Test vs Ref) | -1,7% | -8,9% | -9,3% | -2,1% | -10,0% | -11,9% |  |  |  |
| Enc Time[%] | 102,4% |  |  | 101,7% |  |  |  |  |  |
| Dec Time[%] | 107,2% |  |  | 107,8% |  |  |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,6% | -13,7% | -8,3% |  |  |  | -0,4% | -9,0% | -5,3% |
| Class B | -0,4% | -8,7% | -9,8% | -0,3% | -9,5% | -10,5% | -0,3% | -7,6% | -8,2% |
| Overall (Test vs Ref) | -0,4% | -10,2% | -9,3% | -0,3% | -9,5% | -10,5% | -0,4% | -8,0% | -7,4% |
| Overall (Test vs single layer) | 18,7% | 19,4% | 20,1% | 15,9% | 16,9% | 16,3% | 14,0% | 21,4% | 24,6% |
| Overall (Ref vs single layer) | 19,2% | 33,3% | 32,0% | 16,2% | 28,8% | 29,1% | 14,4% | 32,1% | 34,1% |
| EL only (Test vs Ref) | -0,7% | -10,9% | -10,0% | -0,7% | -10,8% | -11,8% | -0,6% | -8,9% | -8,1% |
| Enc Time[%] | 101,1% |  |  | 100,6% |  |  | 100,1% |  |  |
| Dec Time[%] | 108,9% |  |  | 106,7% |  |  | 106,9% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  | LD-B HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,3% | -12,1% | -6,6% |  |  |  | -0,3% | -7,9% | -5,0% |
| Class B | -0,3% | -5,7% | -6,7% | -0,2% | -7,5% | -9,6% | -0,3% | -5,6% | -6,7% |
| Overall (Test vs Ref) | -0,3% | -7,5% | -6,7% | -0,2% | -7,5% | -9,6% | -0,3% | -6,3% | -6,3% |
| Overall (Test vs single layer) | 28,2% | 28,0% | 30,4% | 24,6% | 23,2% | 23,2% | 24,0% | 26,1% | 30,7% |
| Overall (Ref vs single layer) | 28,5% | 39,0% | 39,7% | 24,8% | 33,0% | 35,9% | 24,3% | 34,7% | 39,5% |
| EL only (Test vs Ref) | -0,5% | -7,8% | -6,9% | -0,5% | -8,2% | -10,2% | -0,4% | -6,8% | -6,7% |
| Enc Time[%] | 100,6% |  |  | 100,0% |  |  | 99,7% |  |  |
| Dec Time[%] | 107,9% |  |  | 105,2% |  |  | 105,1% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  | LD-P HEVC SNR |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0,3% | -11,4% | -5,9% |  |  |  | -0,3% | -7,4% | -4,4% |
| Class B | -0,2% | -5,2% | -6,3% | -0,2% | -6,9% | -8,8% | -0,2% | -5,2% | -6,0% |
| Overall (Test vs Ref) | -0,3% | -6,9% | -6,2% | -0,2% | -6,9% | -8,8% | -0,3% | -5,8% | -5,6% |
| Overall (Test vs single layer) | 26,2% | 28,0% | 30,6% | 22,5% | 23,9% | 24,1% | 23,1% | 26,6% | 31,7% |
| Overall (Ref vs single layer) | 26,6% | 38,0% | 39,1% | 22,8% | 32,8% | 35,6% | 23,4% | 34,6% | 39,4% |
| EL only (Test vs Ref) | -0,4% | -7,3% | -6,4% | -0,5% | -7,6% | -9,5% | -0,4% | -6,3% | -6,0% |
| Enc Time[%] | 101,1% |  |  | 100,3% |  |  | 100,1% |  |  |
| Dec Time[%] | 109,4% |  |  | 107,4% |  |  | 107,1% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |

Complexity assessment.

The complexity assessment modules in the source code and the worst case calculation in the Excel template were modified specially for the proposed algorithm.

Memory access is described in details in JCTVC-N0184 [7]. For reader’s convinces we copy the Figure 4 describing memory access from [7].

Fig. 4. Memory access explanation for cross-color inter-layer filtering.

From Figure 1 it is clear that one column of Luma samples on the left border of processed block and two rows of Luma samples on top and bottom border of processed block are required to perform inter-layer cross-color filter. This is depicted on Figure 4: compare to processed block (a) extended Luma block (b) is the input of the inter-layer cross-color filter process.

In proposed design we first derive the position of Luma samples needed for inter-layer cross-color filtering in EL resolution (these samples are surrounded by red-dot line in Fig. 1) and then derive positions of “origin” for these Luma samples on base-layer. This “origin” position is rounded to the nearest integer value. From figure 4 we can see that no more than 2 samples extension of Luma block in base layer resolution is needed for proposed design of inter-layer cross-color filter. So memory access for proposed technique is equivalent to 2 taps horizontal and 3 taps vertical interpolation filter for Luma; which is less compare to 8-taps interpolation during re-sampling. So in case of spatial scalability NO additional memory access is needed for proposed method compare to re-sampling process. In case of SNR scalability test memory access is higher than anchor (SHM2.0) and equivalent to 2-taps filtering in horizontal and 3-taps filtering on vertical direction for Luma component.

Complexity assessment summary is presented in Table 3 (the worst case) and Table 4 (average memory access).

Table 5 reports memory access for anchor SHM2.0 2 layer decoder and proposed tool relatively to HEVC single layer decoder.

No visible change of memory access was found for simplified version compare to original design of this tool (test 3.4.1 in SCE3).  Average memory access difference is 0,0% (for PU-base inter-layer processing) and 0,3% (for Picture based processing).

Table 3. The worst case memory access for cross-color inter-layer filter.

| Scalability |  | SPATIAL |  |  | SNR |  |
| --- | --- | --- | --- | --- | --- | --- |
| IL Processing |  |  | Block | Picture | Block | Picture |
| Reference |  | 145% |  | 160% | 200% | 218% |
| Tested |  |  | 145% | 160% | 200% | 219% |

Table 4. Complexity assessment summary for simplified version of cross-color inter-layer filter.

| PU-based inter-layer processing | Pic-based inter-layer processing |
| --- | --- |
| SPATIAL; SNR. Pure; DDR2; DDR3; Pure; DDR2; DDR3. 101%; 100%; 101%; 101%; 101%; 101%. | SPATIAL; SNR. Pure; DDR2; DDR3; Pure; DDR2; DDR3. 101%; 102%; 101%; 102%; 103%; 103%. |

Table 5. Complexity assessment summary for simplified version of cross-color inter-layer filter (memory access reported compare to HEVC single layer).

|  | PU-base inter-layer processing model. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scalability |  |  | SPATIAL |  |  |  | SNR |  |
|  |  | Pure | DDR2 |  | DDR3 | Pure | DDR2 | DDR3 |
| Reference |  | 111% | 106% |  | 106% | 97% | 98% | 98% |
| Tested |  | 113% | 107% |  | 108% | 98% | 99% | 100% |
|  | PU-base inter-layer processing model. |  |  |  |  |  |  |  |
| Scalability |  |  | SPATIAL |  |  |  | SNR |  |
|  |  | Pure | DDR2 | DDR3 |  | Pure | DDR2 | DDR3 |
| Reference |  | 163% | 152% | 158% |  | 161% | 158% | 161% |
| Tested |  | 164% | 155% | 159% |  | 164% | 163% | 166% |

# Conclusion

Based on analysis presented above for cross-color inter-layer prediction with proposed simplification

- average {Y, U, V} gain {-0.8%, -7.5%, -8.5%}, {-0.4%, -9.2%, -9.1%}, {-0.2%, -7.1%, -7.5%}, and {-0.2%, -6.5%, -6.9%} for AI, RA, LDB, and LDP, respectively

- Luma BD-rate gain is observed despite of tool operates for Chroma component only

- There is no increment for memory access in the worst case associated with this tool

- There is no more than 1%  increment for memory access in average associated with this tool

- performance difference with original design is under noise level

- independent inter-layer processing for Luma and Chroma planes is possible;

- high level syntax is simplified compare to original design

Samsung and InterDigital suggest adopting the proposed simplified design of cross-color inter-layer prediction technique into WD.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- J. Dong, Y. He, Y. Ye, Chroma enhancement for ILR picture JCTVC-L0059 document, January, 2013

- J. Dong, Y. Ye, Y. He, “Chroma enhancement for ILR picture,” JCTVC-M0183, Incheon, South Korea, April 2013.

- E. Alshina, A.Alshina, Y.Cho, “Non SCE4: simplified design of cross-color inter-layer (test 4.2.4),” JCTVC-M0089, Incheon, South Korea, April 2013.

- J. Chen, A. Segall, E. Alshina, S. Liu, J. Dong, “Description of HEVC Scalable Extension Core Experiment SCE3: Inter-layer filtering,” JCTVC-M1103, Incheon, South Korea, April 2013.

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations,” JCTVC-M1009, Incheon, South Korea, April 2013.

- E. Alshina, E. François, “BoG Report on SHVC complexity assessment,” JCTVC-M0455, Incheon, South Korea, April 2013.

# Specification Text

## Sequence parameter set extension syntax and semantics

| sps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_inter_layer_cross_color_enable_flag | u(1) |
| } |  |

sps_inter_layer_cross_color_enable_flag equal to 1 specifies that chroma enhancement is enabled. sps_inter_layer_cross_color_enable_flag equal to 0 specifies that chroma enhancement is disabled. When not present, sps_inter_layer_cross_color_enable_flag is inferred to be equal to 0.

## General slice segment header syntax and semantics

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| ...... |  |
| if ( sps_inter_layer_cross_color_enable_flag ) { |  |
| cb_cross_color_flag | u(1) |
| if (cb_cross_color_flag) { |  |
| for ( i=0; i<11; i++ ) |  |
| cb_filter_coeff_ plus8 [i] | u(4) |
| cb_scaling_factor_abs_minus1 | u(10) |
| cb_scaling_factor_sign | u(1) |
| } |  |
| cr_cross_color_flag | u(1) |
| if ( cr_ cross_color _flag ) { |  |
| for ( i=0; i<11; i++ ) |  |
| cr_filter_coeff_ plus8 [i] | u(4) |
| cr_scaling_factor_abs_minus1 | u(10) |
| cr_scaling_factor_sign | u(1) |
| } |  |
| } |  |
| .. |  |
| } |  |

cb_cross_color_flag equal to 1 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs, cb_scaling_factor_sign, and cb_bit_shifting are present in the slice header. cb_enhancement_flag equal to 0 specifies that cb_filter_coeff_ plus8 [i], cb_scaling_factor_abs, cb_scaling_factor_sign, and cb_bit_shifting are not present in the slice header. When not present, cb_enhancement_flag is inferred to be equal to 0.

cb_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cb. The value of cb_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cb_scaling_factor_abs_minus1 and cb_scaling_factor_sign together specify the value of variable CbScalingFactor as follows:

CbScalingFactor = (1 – 2 * cb_scaling_factor_sign) * (cb_scaling_factor_abs_minus1+1)

The value of cb_scaling_factor_abs_minus1 shall be in the range of 1 to 1023, inclusive.

cr_cross_color_flag equal to 1 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs, cr_scaling_factor_sign, and cr_bit_shifting are present in the slice header. cr_enhancement_flag equal to 0 specifies that cr_filter_coeff_ plus8 [i], cr_scaling_factor_abs, cr_scaling_factor_sign, and cr_bit_shifting are not present in the slice header. When not present, cr_enhancement_flag is inferred to be equal to 0.

cr_filter_coeff_ plus8[i] minus 8 specifies the i-th filter coefficient of the high pass filter for the chroma component Cr. The value of cr_filter_coeff_ plus8 [i] shall be in the range of 0 to 15, inclusive.

cr_scaling_factor_abs_minus1 and cr_scaling_factor_sign together specify the value of variable CrScalingFactor as follows:

CrScalingFactor = (1 – 2 * cr_scaling_factor_sign) * (cr_scaling_factor_abs_minus1+1)

The value of cr_scaling_factor_abs_minus1 shall be in the range of 1 to 1023, inclusive.

## Decoding Process

G.8.1.4 Derivation process for inter layer reference pictures

……

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if ScaledW is equal to PicWRL and ScaledH is equal to PicHRL

  - rsPicSample is set to rlPicSample,

  - If cb_cross_color_flag is equal to 1 or cr_enhancement_flag is equal to 1, the chroma enhancement filtering process as specified in subclause G.8.1.4.3 is invoked with the sample values of rsPicSample as input, and with the modifed chroma sample values of rsPicSample as output.

  - When sps_inter_layer_mfm_enable_flag is equal to 1, rsPicMotion is set to rlPicMotion.

–	otherwise, rsPic is derived as follows:

  - The picture sample resampling process as specified in subclause G.8.1.4.1 is invoked with the sample values of rlPicSample as input, and with the resampled sample values of  rsPicSample as output.

  - If cb_cross_color_flag is equal to 1 or cr_enhancement_flag is equal to 1, the chroma enhancement filtering process as specified in subclause G.8.1.4.3 is invoked with the sample values of rsPicSample as input, and with the modifed chroma sample values of rsPicSample as output.

  - When sps_inter_layer_mfm_enable_flag is equal to 1, the picture motion field resampling process as specified in subclause G.8.1.4.2 is invoked with the compressed motion field of rlPicMotion as input, and with the  resampled motion field of rsPicMotion as output.

……

G.8.1.4.3 Chroma enhancement filtering process for inter layer reference pictures

Input to this process is:

–  is the luma reference sample array rlPicSampleL.

–  a (ScaledW/2)x(ScaledH/2) array rsPicSampleCb of chroma samples of the component Cb

–  a (ScaledW/2)x(ScaledH/2) array rsPicSampleCr of chroma samples of the component Cr

Outputs of this process are:

–	a modified (ScaledW/2)x(ScaledH/2) array rsPicSampleCb of chroma samples of the component Cb,

–	a modified (ScaledW/2)x(ScaledH/2) array rsPicSampleCr of chroma samples of the component Cr.

If cb_enhancement_flag is equal to 1, the modified array rsPicSampleCb of the samples of the chroma component Cb is derived by invoking the enhancing process of chroma sample values specified in subclause G.8.1.4.3.1 with the (ScaledW)x(ScaledH) array rsPicSampleL, (ScaledW/2)x(ScaledH/2) array rsPicSampleCb, cb_filter_coeff_plus8, CbScalingFactor, and cb_bit_shifting given as inputs, and with the modified (ScaledW/2)x(ScaledH/2) array rsPicSampleCb as output. If cb_enhancement_flag is equal to 0, the array rsPicSampleCb of the samples of the chroma component Cb is not modified.

If cr_enhancement_flag is equal to 1, the modified array rsPicSampleCr of the samples of the chroma component Cr is derived by invoking the enhancing process of chroma sample values specified in subclause G.8.1.4.3.1 with the (ScaledW)x(ScaledH) array rsPicSampleL, (ScaledW/2)x(ScaledH/2) array rsPicSampleCr, cr_filter_coeff_ plus8, CrScalingFactor, and cr_bit_shifting given as inputs, and with the modified (ScaledW/2)x(ScaledH/2) array rsPicSampleCr as output. If cr_enhancement_flag is equal to 0, the array rsPicSampleCr of the samples of the chroma component Cr is not modified.

G.8.1.4.3.1 Enhancing process of chroma sample values

Inputs to this process are:

–	is the luma reference sample array rlPicSampleL.,

–	a (ScaledW/2)x(ScaledH/2) array rsPicSampleC,

–	a variable array filter_coeff_plus8 [i] ( i = 0…10 ),

–	a variable ScalingFactor,

Outputs of this process is:

–	a modified (ScaledW/2)x(ScaledH/2) array rsPicSampleC.

The modified chroma samples rsPicSampleC[ xPC ][ yPC ] with ( xPC = 0..ScaledW/2 – 1, yPC = 0..ScaledW/2 – 1) are derived by applying the following ordered steps:

- Temporal array filter_coeff [i] ( i = 0…11 ) values are specified as follows:

filter_coeff [i] = filter_coeff_plus8 [i] +8( i = 0…10 );

filter_coeff [11] =- filter_coeff [10]- filter_coeff [9]- filter_coeff [8]- filter_coeff [7]- filter_coeff [6]- filter_coeff [5]- filter_coeff [4]- filter_coeff [3]- filter_coeff [2] - filter_coeff [1]- filter_coeff [0];

- The variable Temp is derived as follows:

Temp=( filter_coeff [0] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL -1), Clip3(0, ScaledH-1, yPL -1)] )
+( filter_coeff [1] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL    ), Clip3(0, ScaledH-1, yPL -1)] )
+ ( filter_coeff [2] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL +1), Clip3(0, ScaledH-1, yPL -1)] )
+ ( filter_coeff [3] ) *( rlPicSampleL [ Clip3(0, ScaledW-1, xPL -1), Clip3(0, ScaledH-1, yPL     )] )
+ ( filter_coeff [4] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL     ), Clip3(0, ScaledH-1, yPL     )] )
+ ( filter_coeff [5] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL +1), Clip3(0, ScaledH-1, yPL    )] )
+ ( filter_coeff [6] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL -1), Clip3(0, ScaledH-1, yPL +1)] )
+ ( filter_coeff [7] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL), Clip3(0, ScaledH-1, yPL +1)] )
+ ( filter_coeff [8] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL +1), Clip3(0, ScaledH-1, yPL +1)] )
+ ( filter_coeff [9] ) * rlPicSampleL [ Clip3(0, ScaledW-1, xPL -1), Clip3(0, ScaledH-1, yPL +2)] )
+ ( filter_coeff [10] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL   ), Clip3(0, ScaledH-1, yPL +2)] )
+ ( filter_coeff [11] ) * (rlPicSampleL [ Clip3(0, ScaledW-1, xPL +1), Clip3(0, ScaledH-1, yPL +2)] )

here

xPL = (((2*xPC - offsetX )* scaleFactorX  + ( 1 << ( shiftX − 1 ) ) ) >> shiftX+8)>>4

yPL = (((2* yPC - offsetY)* scaleFactorY  + ( 1 << ( shiftY − 1 ) ) ) >> shiftY+8)>>4

- The variable ScaledTemp is derived as follows:

ScaledTemp=Sign(temp * ScalingFactor) * ((Abs(temp * ScalingFactor) +32768) >> 16)

- The modified rsPicSampleC[ xPC ][ yPC ] is derived as follows:

rsPicSampleC[ xPC ][ yPC ] = Clip3( 0, ( 1 << BitDepthC) – 1, rsPicSampleC[ xPC ][ yPC ]+ScaledTemp).