| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0153 |
| --- | --- |

| Title: | On adaptive re-sampling filter coefficients coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin | Email: | elena_a.alshina@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

In SCE1 and SCE3 adaptive re-sampling filter solutions for SHVC were studied. In this contribution coding method for content adaptive re-sampling filter coefficients is suggested. The benefit of proposed method is no overflow of 16-bits intermediate buffer is guaranteed.

# Introduction

In the current SHVC working draft [1], up-sampling for inter layer prediction is designed based on an assumption that the sampling grids of BL and EL pictures have no phase shift. Under this assumption for ×2 and ×1,5 spatial scalability support only 1 additional Luma and 3 Chroma interpolation filters were added into SHVC design (compare to HEVC motion compensation interpolation filters set). These filters are listed in Table 1, 2. Interpolation filters used in HEVC are marked “HEVC”. Interpolation filters added for ×2 and ×1,5 spatial scalability support are marked as “SHVC”.

More filters for fractional positions not used under ctc were tested in SCE1 (marked as “SCE1”) and related contributions.

In some SCE1 and SCE3 tests variable filter coefficients for better content adaptation were studied.

The goal of this contribution is to analyse dynamic range in interpolation process and formulate restriction on coefficients which would guarantee no overflow of 16-bits intermediate buffer.

The scheme of SHM2.0 re-sampling interpolation process is shown on Figure 1 (N=8).

Fig 1. SHM2.0 re-sampling process.

For input data is between InputMin = 0 and InputMax =2N-1. After horizontal interpolation (“FIR_hor”) data is in diapason

InputMin * SumPos + InputMax * SumNeg  ≤  FIR_hor  ≤  InputMin * SumNeg + InputMax * SumPos,       (1)

where SumNeg≤0 is sum of negative coefficients of FIR and SumPos≥0 is sum of positive coefficients of FIR. For fixed predefined FIR used in SHM2.0 in the worst case (1/2-pel interpolation) SumPos = 88 and SumNeg = -24. It means that result of FIR is signed integer:

-24*(2N-1) ≤  FIR_hor  ≤  88 *(2N-1);        (2)

which is for N=8

-(215-1)< -6120 ≤  FIR_hor  ≤  22440 < 215-1.  (3)

Table 1. SHM2.0 and additional Luma interpolation filters studied in SCE1.

fL[ 16-p, k ] = fL[ p, 7-k ] (0≤k≤7)

| phase p | Luma interpolation filter coefficients |  |  |  |  |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |  |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 | HEVC |
| 1 | 0 | 1 | -3 | 63 | 4 | -2 | 0 | 0 | SCE1 |
| 2 | 0 | 2 | -6 | 61 | 9 | -3 | 1 | 0 | SCE1 |
| 3 | -1 | 3 | -8 | 60 | 13 | -4 | 1 | 0 | SCE1 |
| 4 | -1 | 4 | -10 | 58 | 17 | -5 | 1 | 0 | HEVC |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 | SHM 2.0 |
| 6 | NA | NA | NA | NA | NA | NA | NA | NA | Not tested |
| 7 | -1 | 4 | -11 | 45 | 34 | -10 | 4 | -1 | SCE1 |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 | HEVC |

Table 2. SHM2.0 and additional Chroma interpolation filters studied in SCE1.

fC[ 16-p, k ] = fC[ p, 3-k ] (0≤k≤3)

| phase p | interpolation filter coefficients |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |  |
| 0 | 0 | 64 | 0 | 0 | HEVC |
| 1 | -2 | 62 | 4 | 0 | SHM 2.0 |
| 2 | -2 | 58 | 10 | -2 | HEVC |
| 3 | -4 | 56 | 14 | -2 | SCE1 |
| 4 | −4 | 54 | 16 | −2 | HEVC |
| 5 | −6 | 52 | 20 | −2 | SHM 2.0 |
| 6 | −6 | 46 | 28 | −4 | HEVC |
| 7 | -4 | 42 | 30 | -4 | SHM 2.0 |
| 8 | −4 | 36 | 36 | −4 | HEVC |

So with SHM2.0 filter coefficients no overflow of 16-bits temporal buffer is guaranteed.

Let’s assume that filter coefficients are flexible (for example):

{ -13,42,-111,400,400,-111,42,-13};             (4)

SHM2.0 filter coefficients are used as predictor and the signed difference {k} between SHM2.0 filter coefficients and content adaptive filter coefficients is encoded. In order to keep normalization the sum of filter coefficients should be 64, so 3=-0-1-2.

If |k|≤=2p-1, k=0,1,2 then |k| can be encoded with p bits fixed length code and one more bit is needed for sign is (|k|>0). In the worst case |3|≤3*. Using this assumption positive and negative coefficients  sums for filter (4) are

SumPos = 88+2+2  and SumNeg = –24 – 2– 6.

and (2) is transformed to

(–24 – 2– 6)*(2N-1) ≤  FIR_hor  ≤  (88+2+2  ) *(2N-1).

In order to keep result of FIR_hor within 16 bits following system of un-equalities should be satisfied:

(24 + 2+ 6)*(2N-1) ≤ 215-1

(88+2+2 )*(2N-1) ≤ 215-1

Which leads to

≤ 13 and ≤ 10.

So, if

|k|≤=7

then |k| can be encoded using 3 bits fixed length code or truncated unary with truncated value 7, and no overflow of 16-bit intermediate buffer is guaranteed.

In opposite if (for example) ≥ 11 the there is an example of filter (4) which will overflow 16 bits intermediate buffer.

# Conclusions

- If content adaptive re-sampling filter coefficients are encoded as difference between default re-sampling SHM2.0 filter and selected one and absolute value of this difference doesn’t exceed 7 then no overflow of 16-bits intermediate buffer is guaranteed.

- Usage of fixed length (3 bits) or truncated unary code for absolute value of filter coefficients difference encoding guarantees no overflow of 16-bits intermediate buffer

- No restriction for in absolute value of re-sampling filter coefficients leads to overflow of 16-bits intermediate buffer and should not be allowed.

# Reference

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, “SHVC Working Draft 2,” JCT-VC document, JCTVC-M1008, Incheon, South Korea, April 2013.

- JCTVC-M0267: “SCE4: Results of Test 4.2.2 on Adaptive Re-Sampling Filter” (Qualcomm)

# Patent rights declaration(s)

Samsung Electronics Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).