| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0155 |
| --- | --- |

| Title: | HLS: Thumbnail Support In HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ce Wang Charlie Wang Yi-jen Chiu Wenhao Zhang | Email: | ce.wang@intel.com charlie.l.wang@intel.com yi-jen.chiu@intel.com wenhao.zhang@intel.com |
| Source: | Intel Corporation |  |  |

_____________________________

# Abstract

This contribution proposes a modification to the syntax of current HEVC in order to embed thumbnail video into HEVC bit streams. A thumbnail video, which is a sequence of snapshots with smaller resolution, is required in many video applications to give a preview of the video content. To support this feature, three modifications are applied to the high level syntax of HEVC. Firstly, a new VCL NAL unit type is proposed to contain the coded thumbnail picture. Secondly, a new SEI message is added to transmit auxiliary information, e.g. the period of presentation of the thumbnails. Finally, a reserved bit in profile_tier_level() structure is changed into a flag to signal the on/off of the thumbnail video in a bit stream. These modifications do not violate the conformance requirement of current HEVC, and provide a flexible method to enclose the thumbnail information into regular HEVC bit stream.

# Introduction

Many video applications require thumbnails of the video content for preview. The video thumbnail is a sequence of snapshots of the video with smaller resolution. Usually the thumbnail resolution is around 100-200 pixels each dimension, comparing with 1080p of full version. With thumbnails, application may choose to display typical scenes from the movie in order to attract attention. And users may choose to play the full movie from that particular scene. The image below is an example for a NBA game. The smaller pictures at bottom are thumbnails of the game, and are also starting points of game segments. The upper image is a snapshot from the second sequence.

Figure 1: An example of thumbnail

Currently, all the popular video codecs carry only full resolution information of the content. In order to generate the thumbnail preview, a down-scaling processing is required after fully decoding the original video. This demands significant processing power and may hinder real-time application adoption at the client side. Some codecs such as H.264/SVC has capability of carrying layered multi-resolution video information. But it needs special (SVC compatible) decoder to retrieve all the layered information. A regular H.264/AVC decoder can only obtain the base-layer video (smallest resolution).

Another benefit of introducing thumbnail in HEVC elementary bitstream is about premium contents. As HEVC becomes popular, its adoption by major entertainment studios as carrier for premium contents is expected. However the premium contents are usually protected by various DRM schemes. It brings in extra headache for applications to generate preview. The proposed thumbnails embedded in original streams makes it possible that thumbnail sequences are put in clear, while original video still being protected. Application may use thumbnail sequences as preview without fully decrypting and decoding the accompanying contents. This is a much desired feature nowadays for premium content streaming industry.

In this proposal, a codec extension is described which is fully compatible with baseline decoder and carries both full video and thumbnail information.

# Proposed technology

To enclose thumbnail picture information in regular HEVC bit streams, the conformance of current HEVC specification (ISO/IEC 23008-2:201x(E)) does not need to be violated. Only three extensions will be added in the syntax.

## Adding a new VCL NAL unit type

The nal_unit_type 10 is renamed to TN_N from RSV_VCL_N10. This indicates a non-reference VCL unit. The NAL unit should contain an intra-coded portion of thumbnail picture. That means the slice segment header following the NAL unit header comes with slice_type equaling 2. The syntax elements as well as their semantics of the rest of the new NAL unit have no difference from a normal intra coded VCL unit with nal_unit_type among 0 to 9 or 16 to 21 inclusively. This makes a conformance HEVC decoder should be able to decode it.

The new NAL units should be inserted in bit streams before the associated regular NAL units with same scalable layer and temporal sublayer. To illustrate, the syntax of NAL unit header is attached in Table 1. The new NAL unit with nal_unit_type TN_N should be given the same nul_layer_id and nul_temporal_id_plus1 as the following regular NAL units. Thus they can also belong to multiple layers and sublayers.

Table 1: NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(6) |
| nuh_layer_id | u(6) |
| nuh_temporal_id_plus1 | u(3) |
| } |  |

The new NAL units contain encoded picture of thumbnail resolution. The thumbnail resolution is specified by associated PPS and SPS NAL units. The special SPS and PPS headers have regular syntax, but are only referenced by the thumbnail NAL units. These headers can specify all the normal HEVC coding tools such as Tiles and WPPs if application prefers.

The Thumbnail pictures are designed to be intra coded in order to facilitate easy random access. Because resolutions of thumbnail pictures are usually small (typically 150-200 pixels) comparing with those of original pictures (1920x1080 or 1080x720 etc.), plus they are distributed sparsely (one thumbnail usually represents a big group of pictures), coding in intra mode will only take a small percentage of whole video bit streams.

The thumbnail pictures are marked with non-reference NAL unit type. They will not hold positions in the DPB buffer. Therefore normal DPB operation is not influenced. This is important for bit streams with multiple layers and sub-layers, where the DPB sizes for lower layers/sub-layers are small and precious.

The thumbnail NALs are designed to take same layer and sub-layer IDs with their associated original picture groups. This design gives the flexibility that application may choose to decode and display thumbnail pictures of certain layers/sub-layers, not all.

Thumbnail stream can be used as an index to the original video sequence. To retrieve the original video segment, decoder can start to decode the original sequence (with regular NAL types) from the next available RAP picture following the thumbnail picture.

## Adding a new SEI message

A new Thumbnail Picture SEI message is defined to indicate presence of thumbnail picture NAL types in the bit stream as auxiliary information. The associated SEI parameters are:

- nal_unit_type = PREFIX_SET_NUT (39)

- payloadType=135

- payloadSize = variable

The syntax of the Thumbnail Picture SEI message is defined as:

Table 2: Thumbnail picture SEI message syntax

| thumbnail_pictures_info( payloadSize ) { | Descriptor |
| --- | --- |
| thumbnail_present_flag | u(1) |
| if (thumbnail_present_flag) { |  |
| thumbnail_uniform_spacing_flag | u(1) |
| if (thumbnail_uniform_spacing_flag) |  |
| thumbnail_spacing_minus1 | ue(v) |
| } |  |
| } |  |

If thumbnail pictures are carried in the bit stream, one NAL unit of this SEI message should also appear in the bit stream, and before any VCL NAL units. This SEI message may also present in the bit stream multiple times for error resilience purpose.

If thumbnail pictures are inserted in the bit stream periodically, auxiliary information may be provided in the SEI message to specify the picture distance between consecutive thumbnail pictures (thumbnail_spacing_minus1 + 1). Applications may rely on this syntax element to obtain presence frequency of thumbnail pictures without parsing whole bit streams.

## Indication in profile_tier_level() structure

In current conformance HEVC standard, general_reserved_zero_44bits should take value 0 for all the bits. It is proposed to allocate one bit from it to indicate the presence of thumbnail pictures in the bit stream. General_reserved_zero_44bits is converted into general_thumbnail_flag (1 bit) and general_reserved_zero_43bits (43 bits).

Table 3: updated Profile, tier and level syntax

| profile_tier_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| general_profile_space | u(2) |
| general_tier_flag | u(1) |
| general_profile_idc | u(5) |
| for( j = 0; j < 32; j++ ) |  |
| general_profile_compatibility_flag[ j ] | u(1) |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_thumbnail_flag | u(1) |
| general_reserved_zero_43bits | u(43) |
| general_level_idc | u(8) |
| … … |  |

The profile_tier_level() structure is included in both video parameter set (VPS) header structure and sequence parameter set (SPS) header structure. The proposed general_thumbnail_flag in the VPS is used to indicate the presence of thumbnail pictures in the whole bit stream. And general_thumbnail_flag in the SPS is used to indicate the presence of thumbnail pictures in the current GOP stream bounded by IDRs.

The location of bit general_thumbnail_flag is fixed at bit 44 of profile_tier_level() structure. And it is also at fixed locations in both VPS and SPS NAL units. Thus it benefits easy extraction from bit stream for application.

# Conclusion

In summary, three syntax extensions are introduced in the standard HEVC specification to support thumbnail pictures. These extensions have no impact on the conformed HEVC bit stream decoding. If a decoder does not recognize the new syntax, the thumbnail picture NALs, SEI messages and syntax element general_reserved_zero_44bits are simply ignored, and normal decoding is maintained. If a decoder recognizes the new syntax, it may use the SEI message and/or general_thumbnail_flag to decide if thumbnail pictures present in the bit stream and extract only the thumbnail pictures for preview/indexing without parsing or decoding the full bit stream.

The key point here is the new thumbnail NAL unit is introduced and it is coded as a regular intra predicted picture. Thus no new decoding logic is required to decode it. The change is limited in the high level syntax. The current software and hardware HEVC decoders are fully transparent to the change. At encoder side, the coding and insertion of thumbnail pictures can be performed in a separate process. No change on the encoder is required either. This point is important for the industry adoption.

Last, although the idea as well as syntax proposed here is related to video, it can be directly applied to still pictures without any change, because the HEVC Main Still Picture Profile shares the same syntax as Main Profile and Main 10 Profile. Furthermore, many experiments point out that HEVC intra picture encoding already surpass JPEG, JPEG2000, and JPEG_XP etc. popular still picture encoding formats in compression rate. It is possible that with HEVC becomes more popular, it may replace JPEG format in photography industry. If it really happens, the proposed thumbnail support would have much greater applications.

# Patent rights declaration(s)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).