| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0159 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0071 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Parallel Processing Indications for Tiles in HEVC Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Xiang Li Jianle Chen Wei Pu Ye-Kui Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | -858-658-2042 krapaka@qti.qualcomm.com lxiang@qti.qualcomm.com cjianle@qti.qualcomm.com wpu@qti.qualcomm.com yekuiw@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

HEVC supports tile based coding to enable parallel processing. In this contribution, some problems are discussed related to parallel processing of tiles across layers and two methods are proposed to address the problems and to enable more friendly parallel processing of tiles across layers. The first method proposes an indication of an encoder constraint on inter-layer prediction for the samples of the enhancement layer picture that lie across the tile boundaries. The second method proposes a tile based up-sampling. The third method proposes an indication if inter-layer prediction is used for a particular tile in the enhancement layer picture.

# Problem Description

Existing solutions for parallel processing in SHVC and MV-HEVC are associated with at least the following problems:

- The scheme for offset delay signaling proposed in JCTVC-M0463 was adopted in Incheon JCTVC meeting. The scheme is designed to aid in parallel decoding across layers by signaling offset delay syntax elements in SPS (VUI) conveying the minimum required decoding delay for each layer. The decoding of a particular layer can start as soon as the region in reference layer specified by the offset delay is decoded due constraining for inter-layer prediction from samples that follow the specified offset delay in decoding order . However, the current design of offset delay signaling for HEVC extensions is inefficient when only some samples across the tiles introduce delay as shown in the example

Figure 1: An example tile configuration across layers with inter-layer prediction dependencies for samples across the tile boundaries.

- Further with the current design, it is not possible to indicate delay such as tile ET1 depends on tile BT0 and it is assumed that tile BT0 is decoded and available for inter-layer prediction when decoding tile ET1. This may disallow synchronized processing across layers.

- It is beneficial for pipelining tiles of the current picture with respect to that of the reference layer pictures if it is known that inter-layer prediction is used for a particular tile or not. Currently it is not possible to know whether a particular tile uses inter-layer prediction without decoding the tile, unless it belongs to a picture of the base layer (in which case inter-layer prediction surely is not used).

# Proposal

In this proposal, the following three methods are proposed:

- Encoder constraint on inter-layer prediction

- Tile based upsampling filter

- Tile based inter-layer prediction signaling

## Encoder constraint on inter-layer prediction

In this method an encoder constraint is applied so that no CU/PU in an enhancement tile is inter-layer texture predicted or inter-layer syntax predicted from a collocated CU/PU that may have up-sampling across the tiles in the reference layer pictures. The indication of the application of this restriction is signalled in VUI.

The proposed syntax and semantics changes are as follows.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| tiles_fixed_structure_flag | u(1) |
| inter_layerpred_not_along_tile_boundary_flag | u(1) |
| … |  |
| } |  |
| } |  |

inter_layerpred_not_along_tile_boundary_flag equal to 1 indicates that tile boundaries of different layers are aligned and inter-layer texture prediction and inter-layer syntax prediction are not allowed along/across the tile boundaries such that any enhancement layer tile can be decoded without decoding any non-aligned base layer tile. The alignment of tile boundaries means that any two enhancement layer picture samples that lie within the same enhancement layer tile, the collocated base layer samples shall also lie within the same base layer tile. inter_layerpred_not_along_tile_boundary_flag equal to 0 indicates that there is no restriction on the tile configurations among layers. When inter_layerpred_not_along_tile_boundary_flag is not present in bitstream, it shall be inferred to be equal to 0.

## Tile based up-sampling and filtering

Base layer pixels are re-sampled and used as interlayer reference. If the base layer picture is coded with tiles, sub-pixel samples close to the border of the tiles are calculated using the integer samples from neighboring tile.

In this method, it is proposed to signal a flag in SPS extension for enhancement layer to indicate whether tile based up-sampling is enabled.

| if( sps_extension_flag ) { |  |
| --- | --- |
| ... |  |
| tile_based_resampling_filter_enable_flag | u(1) |
| ... |  |
| } |  |

tile_based_resampling_filter_enable_flag equal to 0 specifies that picture based up-sampling is enabled. tile_based_resampling_filter_enable_flag equal to 1 specifies that tile based up-sampling is enabled and tile boundaries of different layers are aligned. When tile_based_resampling_filter_enable_flag not present it is inferred to be equal to 0.

When tile based up-sampling is enabled, tile boundaries are treated in the same way as that of picture boundaries in up-sampling process. That is, when a pixel sample outside the current tile (may be inside or outside the current picture) is needed for up-sampling, the pixel is generated with pixel padding from those inside the current tile. The padding process is the same as that for defined for pixels outside the current picture.

## Tile based inter-layer prediction signalling

In the multi-loop decoding framework of SHVC and MV-HEVC, it is useful to indicate if inter-layer prediction is used for a particular tile or not. Such an indication can be used for pipelining tiles of current picture with respect to that of reference layer pictures. For example, if a particular tile of an enhancement layer picture does use inter-layer prediction, then the decoding of this tile can be scheduled in parallel to the decoding of reference layer tiles. Currently it is not possible to know whether a particular tile uses inter-layer prediction without decoding the tile, unless it belongs to a picture of the base layer (in which case inter-layer prediction surely is not used).

We thus propose to signal such indication in an SEI message with the following syntax and semantics.

| tile_interlayer_pred_info( payloadSize ) { | Descriptor |
| --- | --- |
| for( i = 0; i <= num_tile_columns_minus1; i++ ) |  |
| for( j = 0; j <= num_tile_rows_minus1; j++ ) |  |
| inter_layer_pred_tile_enabled_flag[ i ][ j ] | u(1) |
| } |  |

The tile inter-layer prediction information SEI message is a prefix SEI message and may be associated with each coded picture. If a tile inter-layer prediction information SEI message is a non-nested SEI message, the associated coded picture is the coded picture containing the VCL NAL unit that is the associated VCL NAL unit of the SEI NAL unit containing the tile inter-layer prediction information SEI message. Otherwise (the SEI message is a nested SEI message), the associated coded picture is specified by the containing scalable nesting SEI message.

inter_layer_pred_tile_enabled_flag[ i ][ j ] equal to 1 indicates that inter-layer prediction may be used in decoding the tile of the i-th tile column and j-th tile row. inter_layer_pred_tile_enabled_flag[ i ][ j ] equal to 0 indicates that inter-layer prediction is not used in decoding the tile of the i-th tile column and j-th tile row. When not present, the value of inter_layer_pred_tile_enabled_flag is inferred to be equal to 1.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).