| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 |  |  | Document: JCTVC-N0161 |
| --- | --- | --- | --- |
| Title: | Using decoded pictures from higher layer as references in SHVC |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Jianle Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Email: | krapaka@qti.qualcomm.com cjianle@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a design to enable using the decoded pictures from higher quality layer as reference for lower layer pictures for SNR scalability in SHVC. The document proposes

- Signaling a flag in VPS to specify that immediate higher layer picture is used as reference for lower layer

- The concept of key picture to reduce error drift for base layer decoding

When the SHM 2.0 anchor is modified to enable temporal scalability, the proposed method reports an average of -2.5%, -3.1% and -3.2% luma BD-rate reductions (EL+ BL) for RA-SNR, LD-P and LD-B SNR scalability cases respectively. It is also reported that over SHM 2.0 anchor an average of -2.3%, 0.2% and 1.4% luma BD-rate reductions (EL+ BL) is obtained for RA-SNR, LD-P and LD-B SNR scalability cases respectively.

# Introduction

Various concepts had been studied for trading off enhancement layer coding efficiency and error drift for packet-based quality scalable coding. Figure 1 below shows two such concepts as an example

Figure 1: Example concepts for trading off enhancement layer coding efficiency and drift for packet-based quality scalable coding

Figure 1 (a) employs two motion compensation loops similar to SHVC. The base layer is not influenced by packet losses in the enhancement layer.

Figure1 (b) allows the flexibility to use higher layer decoded pictures (with high quality) as reference for lower layers pictures. This mechanism improves the coding efficiency compared to the method in Figure 1 (a), however the base layer is influenced by packet losses in the enhancement layer and introduces drift. The drift describes the effect that the motion-compensated prediction loops at encoder and decoder are not synchronized, e.g., because quality refinement packets are discarded from a bit stream. To reduce the drift, “key picture” (marked by the black-framed boxes) are used which don’t use higher layer decoded picture (with high quality) as reference.

Currently, SHVC does not allow the use of higher layer decoded picture (with high quality) as reference for lower layers pictures. This contribution proposes to enable such scheme to achieve better coding efficiency for SNR scalability. The concept of key picture is applied in this proposal to achieve the good trade-off between enhancement layer coding efficiency and base layer error drift.

# Proposal

This contribution proposes to allow use of higher layer decoded pictures as reference pictures for lower layer pictures. To allow this flexibility and to keep the design simple, following are proposed

- Signaling a flag in VPS to specify that immediate higher layer picture, when available, is used as reference for lower layer picture.

- The concept of key picture is introduced to reduce error drift for base layer decoding. The key pictures only use the decoded key pictures of the same layer as reference. For simplicity, the pictures with temporal ID equal to 0 are inferred to be a key picture in the current design.

- Higher layer reference pictures for lower layer pictures is allowed only for SNR scalability

Since the proposed method is only applied to SNR scalability, it does not introduce any low level decoding process modification as well as other noticeable complexity. The reference to the higher layer decoded picture can be easily fulfilled by changing the entry of reference picture set.

# Experimental results

In the proposed method, the picture with temporal ID equal to 0 is inferred to be a key picture, which only uses the decoded key picture of the same layer as reference for inter prediction. Therefore, the proposed method requires to be enabled together with temporal scalability so that the pictures with temporal ID larger than 0 can use the higher layer picture as the references for inter layer prediction.

In this section, the proposed method is experimentally verified for two anchor configurations as summarized below.

Anchor configuration 1: SHM 2.0 CTC

Anchor configuration 2: SHM 2.0 CTC with Temporal scalability enabled as in JCTVC-L0322

Proposed method: SHM 2.0 CTC with Temporal scalability enabled as in JCTVC-L0322 and the proposed method.

Table 1:  Experimental results of Configuration 2 (Anchor) vs. Proposed Method

Table 2:  Experimental results of Configuration 1 (Anchor) vs. Proposed Method

Table 3: Experimental results of Configuration 1 (Anchor) vs. Configuration 2

# Specification modification

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| ……. |  |
| for( i = 0; i <= vps_max_layers_minus1 - 1; i++ ) |  |
| enable_higher_layer_ref_pic_pred[ i ] | u(1) |
| …… |  |
| } |  |

enable_higher_layer_ref_pic_pred[ i ] equal to 0 specifies that within the CVS, the decoded pictures with nuh_layer_id greater than layer_id_in_nuh[ i ], are not used as reference for pictures with nuh_layer_id equal to layer_id_in_nuh[ i ]. enable_higher_layer_ref_pic_pred[ i ] equal to 1 specifies that within the CVS, the decoded pictures with nuh_layer_id equal to layer_id_in_nuh[ i + 1], when available, shall be used as a reference for pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than 0. When not present, enable_higher_layer_ref_pic_pred[ i ] is inferred to be equal to 0.

It is a requirement of bitstream conformance that enable_higher_layer_ref_pic_pred[ i ] shall be equal to 0 for non-SNR scalability cases.

In addition to the VPS syntax and semantics change, the derivation of RPS and marking process are modified to include immediate higher layer reference picture into the current layer RPS and follow marking process as current layer. Also, when output and removal of pictures from the DPB before the decoding of the current picture, RPS is invoked to mark only the pictures with the same value of nuh_layer_id. The detailed modifications to the spec are highlighted in the specification text along with this proposal.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).