| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0162 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 5th Meeting: Vienna, AT, 27 July – 2 Aug. 2013 | Document: JCT3V-E0068 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Inter-layer scaling list inheritance for HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Thomas Rusert Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 76 8428380 martin.m.pettersson@ericsson.com +46 76 7602660 thomas.rusert@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This proposal is a follow-up of the JCTVC-M0134 proposal. The contribution addresses a redundancy problem that is asserted to be present when two or more different SPSs or PPSs containing identical scaling list (quantization matrix) data are present for different layers in multi-layer HEVC extensions. If scaling list data is present in SPS or PPS it typically takes up a major part of the amount of bits in the particular SPS or PPS. The scaling list data would in the current version of the draft be duplicated, causing bit overhead for parameter set transmission and storage.

It is proposed for HEVC extensions to introduce inter-layer scaling list inheritance to reduce bit redundancy. This implies extending the definition of SPS and PPS in the extension specifications. In the proposed SPS/PPS inheritance scheme, scaling list data is inherited from a reference SPS/PPS, determined as the active SPS/PPS of the reference layer, which is signaled in the referencing SPS/PPS.

# Introduction

The scope of this contribution is to reduce SPS and PPS related bit overhead in HEVC extensions by inheriting redundant scaling list data from a reference layer. The proposed solution is applicable for both multiview video and scalability extensions of HEVC. For multiview coding, where the views have high correlation it is intuitive to use the same scaling lists across views. Likewise, for spatial scalability or SNR scalability or potentially other forms of scalability where the correlation is high between layers, it would typically be beneficial to reuse the scaling lists from the lower layers.

One approach for reducing SPS/PPS overhead is to re-use SPSs/PPSs across layers, i.e. to activate the same SPS/PPS for more than one layer. The effectiveness of that approach is limited since even though many SPS/PPS parameters typically have identical values across layers, some SPS/PPS parameters still have different values.

This contribution proposes inheritance of scaling list data in SPS and PPS across layers by using a reference mechanism. The syntax elements for a current SPS or PPS associated with a current layer are conditionally inferred from the active SPS or PPS of a referenced layer below the current layer. The presence of such layer references in the SPS or PPS is indicated by flags that are present for layers above the base layer (nuh_layer_id greater than 0).

# Proposed scheme

Proposed syntax and semantics are provided further below. The definitions are based on JCTVC-M1008 and HEVC version 1. All changes are highlighted in yellow.

Scaling list data inheritance for SPS

The proposed layer referencing mechanism is conditionally present in SPSs with scaling_list_enabled_flag equal to 1, sps_scaling_list_data_present_flag equal to 1 and nuh_layer_id greater than 0. The presence of SPS scaling list data inheritance is indicated using sps_scaling_list_data_inheritance_flag. If present, a reference to another layer is signaled using sps_delta_ref_layer_id_minus1. A reference may not be made to a layer for which its active SPS itself has sps_scaling_list_data_inheritance_flag set to 1.

Scaling list data inheritance for PPS

The proposed layer referencing mechanism is conditionally present in PPSs pps_scaling_list_data_present_flag equal to 1 and nuh_layer_id greater than 0. The presence of PPS scaling list data inheritance is indicated using pps_scaling_list_data_inheritance_flag. If present, a reference to another layer is signaled using pps_delta_ref_layer_id_minus1. A reference may not be made to a layer for which its active PPS itself has pps_scaling_list_data_inheritance_flag set to 1.

## Syntax

The following replaces the contents of subclause F.7.3.2.2.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) { |  |
| if( nuh_layer_id > 0 ) |  |
| sps_scaling_list_data_inheritance_flag | u(1) |
| if( sps_scaling_list_data_inheritance_flag ) |  |
| sps_delta_ref_layer_id_minus1 | ue(v) |
| else |  |
| scaling_list_data( ) |  |
| } |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The following replaces the contents of subclause F.7.3.2.3.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| dependent_slice_segments_enabled_flag | u(1) |
| output_flag_present_flag | u(1) |
| num_extra_slice_header_bits | u(3) |
| sign_data_hiding_enabled_flag | u(1) |
| cabac_init_present_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| init_qp_minus26 | se(v) |
| constrained_intra_pred_flag | u(1) |
| transform_skip_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| if( cu_qp_delta_enabled_flag ) |  |
| diff_cu_qp_delta_depth | ue(v) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| weighted_pred_flag | u(1) |
| weighted_bipred_flag | u(1) |
| transquant_bypass_enabled_flag | u(1) |
| tiles_enabled_flag | u(1) |
| entropy_coding_sync_enabled_flag | u(1) |
| if( tiles_enabled_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_spacing_flag | u(1) |
| if( !uniform_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| pps_loop_filter_across_slices_enabled_flag | u(1) |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| pps_scaling_list_data_present_flag | u(1) |
| if( pps_scaling_list_data_present_flag ) { |  |
| if( nuh_layer_id > 0 ) |  |
| pps_scaling_list_data_inheritance_flag | u(1) |
| if( pps_scaling_list_data_inheritance_flag ) |  |
| pps_delta_ref_layer_id_minus1 | ue(v) |
| else |  |
| scaling_list_data( ) |  |
| } |  |
| lists_modification_present_flag | u(1) |
| log2_parallel_merge_level_minus2 | ue(v) |
| slice_segment_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

The specifications in subclause F.7.4.3.2 apply with the following additions.

sps_scaling_list_data_inheritance_flag equal to 1 indicates that the scaling list data for the current SPS with nuh_layer_id equal to nuhLayerIdA is inherited from the SPS active for nuh_layer_id equal to nuhLayerIdA - sps_delta_ref_layer_id_minus1 - 1. When sps_scaling_list_data_inheritance_flag is equal to 1, it is a requirement of bitstream conformance that the SPS active for nuh_layer_id equal to nuhLayerIdA - sps_delta_ref_layer_id_minus1 - 1 has sps_scaling_list_data_inheritance_flag equal to 0. When sps_scaling_list_data_inheritance_flag is not present, it is inferred to be equal to 0.

sps_delta_ref_layer_id_minus1 plus 1 specifies the difference of the value of nuh_layer_id and the value of nuh_layer_id for the reference layer used for sequence parameter set scaling list data inheritance. The value of sps_delta_view_ref_layer_id_minus1 shall be in the range of 0 to nuh_layer_id - 1, inclusive.

The specifications in subclause F.7.4.3.3 apply with the following additions.

pps_scaling_list_data_inheritance_flag equal to 1 indicates that the scaling list data for the current PPS with nuh_layer_id equal to nuhLayerIdA is inherited from the PPS active for nuh_layer_id equal to nuhLayerIdA - pps_delta_ref_layer_id_minus1 - 1. When pps_scaling_list_data_inheritance_flag is equal to 1, it is a requirement of bitstream conformance that the PPS active for nuh_layer_id equal to nuhLayerIdA - pps_delta_ref_layer_id_minus1 - 1 has pps_scaling_list_data_inheritance_flag equal to 0. When pps_scaling_list_data_inheritance_flag is not present, it is inferred to be equal to 0.

pps_delta_ref_layer_id_minus1 plus 1 specifies the difference of the value of nuh_layer_id and the value of nuh_layer_id for the reference layer used for picture parameter set scaling list data inheritance. The value of pps_delta_view_ref_layer_id_minus1 shall be in the range of 0 to nuh_layer_id - 1, inclusive.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).