| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 02 Aug. 2013 | Document: JCTVC-N0164 |
| --- | --- |

| Title: | RCE2: Cross-check results for Test C.4 (JCTVC-N0053) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Eugen Wige, André Kaup Universität Erlangen-Nürnberg, Germany Peter Amon, Andreas Hutter Siemens Corporate Technology, Germany | Email: Email: | wige@LNT.de kaup@LNT.de p.amon@siemens.com andreas.hutter@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

In this contribution the results of the cross-check performed by University of Erlangen-Nuremberg / Siemens Corporate Technology for Test C.4 of the HEVC Range Extensions Core Experiment 2 (RCE2) are reported. In test C.4 bitrate savings are achieved by replacing the regular DC mode of the HEVC standard by the Method 2 of JCTVC-M0291 [1] and further described in JCTVC-N0053 [3]. It is reported that the lossy as well as lossless coding tests provided by the proponents of test C.4 match the results of the cross-checker.

# Complexity considerations

The proposed method can be performed fully parallel at the encoder needing one addition and one subtraction per sample. The method is also parallelizable at the decoder as presented in [1]. However, than the complexity per sample would become higher.

# Simulations

## Simulation environment

The simulation platform is a homogenous cluster (consisting of Intel(R) Core(TM)2 Quad CPU Q9400 @ 2.66GHz CPUs). For lossless encoding two simulations were performed simultaneously per cluster node whereas for lossy encoding three simulations have been performed simultaneously. The decoding simulations have been performed in a serial way, i.e., only one processor out of four has been used.

## Simulation conditions

Test sequences and simulation conditions as provided in [2] were used for the tests.

## Lossless simulation results

The simulation results for lossless coding are reported in Table 1, Table 2, and Table 3.

Table 1- Lossless coding results for All Intra Main configuration settings

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 5.2 | 5.4 | -1.7% |
| Class B | 2.2 | 2.3 | -1.3% |
| SC RGB 444 | 10.1 | 10.4 | -3.1% |
| SC YUV 444 | 11.4 | 11.7 | -2.6% |
| RangeExt | 2.4 | 2.4 | -0.3% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 108% |  |  |

Table 2- Lossless coding results for Random Access Main configuration settings

|  | Random Access Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 31.7 | 32.0 | -1.0% |
| Class B | 2.6 | 2.6 | -0.2% |
| SC RGB 444 | 100.4 | 103.6 | -2.2% |
| SC YUV 444 | 128.9 | 132.1 | -1.7% |
| RangeExt | 2.5 | 2.5 | 0.1% |
| Enc Time[%] | 117% |  |  |
| Dec Time[%] | 117% |  |  |

Table 3- Lossless coding results for Low delay B Main configuration settings

|  | Low delay B Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class F | 49.8 | 50.1 | -0.9% |
| Class B | 2.6 | 2.6 | -0.2% |
| SC RGB 444 | 381.6 | 395.0 | -2.1% |
| SC YUV 444 | 325.6 | 335.9 | -1.5% |
| RangeExt | 2.5 | 2.5 | 0.1% |
| Enc Time[%] | 117% |  |  |
| Dec Time[%] | 108% |  |  |

## Lossy simulations results

The simulation results for lossy coding experiments are depicted in Table 4, Table 5, and Table 6.

Table 4 – Lossy coding results for All Intra HE Main configuration settings

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.8% | -1.8% | -2.0% | -1.8% | -1.9% | -2.0% | -1.7% | -1.8% | -1.8% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | -4.4% | -4.2% | -4.1% | -4.5% | -4.3% | -4.2% | -4.2% | -4.0% | -4.0% |
| SC YUV 444 | -3.2% | -3.0% | -2.9% | -3.4% | -3.6% | -3.5% | -3.5% | -3.7% | -3.5% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |

Table 5 – Lossy coding results for Random Access HE Main configuration settings

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -1.1% | -1.1% | -1.3% | -1.0% | -1.0% | -1.1% |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| SC RGB 444 | -3.1% | -2.9% | -2.9% | -3.1% | -3.0% | -2.9% |
| SC YUV 444 | -2.3% | -2.2% | -2.2% | -2.4% | -2.6% | -2.4% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6 – Lossy coding results for Low delay B HE Main configuration settings

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.6% | -1.0% | -0.8% | -0.5% | -0.6% | -0.7% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.9% | -1.7% | -1.7% | -2.1% | -1.9% | -1.8% |
| SC YUV 444 | -0.9% | -0.6% | -0.7% | -1.3% | -1.2% | -1.3% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

The cross-checks confirm the results provided by Qualcomm for Test C.4 in document JCTVC-N0053 [3].

# References

- R. Joshi, J. Sole, M. Karczewicz, “Non-RCE2: Modification to the DC intra prediction mode for lossless coding”,  JCTVC-M0291, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013.

- R. Joshi, P. Amon, R. Cohen, S. Lee, M. Naccari, “HEVC Range Extensions Core Experiment 2 (RCE2): Prediction and coding techniques for transform-skip and transform-bypass blocks”, JCTVC-M1122, 13th JCT-VC Meeting, Incheon, KR, Apr. 2013.

- R. Joshi, J. Sole, M. Karczewicz, “RCE2 subtest C.4: Replacement of DC mode for transform-skip and transform-bypass blocks”, ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 WP3, document JCTVC-N0053, 14th JCT-VC Meeting, Vienna, AT, July 2013.