| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013 | Document: JCTVC-N0168-r1 |
| --- | --- |

| Title: | AHG14: Color Gamut Scalable Video Coding using 3D LUT: New Results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Pierre Andrivon, Patrick Lopez, Franck Hiron 975 avenue des champs blancs CS 17616, 35576, Cesson-Sévigné Cedex, France | Tel: Email: | +33-2-99-27-32-42 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports new results in the area of color gamut scalable video coding. In the last meeting, JCTVC-M0197 proposed an inter-layer prediction model for color gamut scalable video coding based on 3D color Look-Up Tables (LUT). It was asserted the application requirements in term of color gamut scalable video coding is not limited to simple transformation between Base layer color space (ex: Rec.709) and Enhancement layer color space (ex: Rec.2020), but also the cases the Base layer and the Enhancement layer has been color graded differently.

It is asserted the size of the 3D LUT can be chosen to meet both the application complexity requirements and the BD-rate distortion trade-off: small size for describing simple color mapping transformation between Base layer and Enhancement layer, and larger size for representing more complex color differences in-between the layers.

Results are reported in 8 bits with the 4 Color graded sequences proposed in AHG-14, between Base layer using Rec.709 color space and Enhancement layer using DCI-P3 color space. It is reported the BD-rate gains of SHM2.0-CGS compared to the simulcast scenario (SHM2.0) is between 4.8% to 32% for luma in average, the BD-rate gains of SHVC (SHM2.0-CGS vs SHM2.0) is between 5.5% to 6.8% for luma in average (between 6.2% to 16.7% for EL).

# Introduction / Problem Statement / …

Color Gamut Scalability has been identified as one requirement of Scalable Coding Extension of HEVC Error: Reference source not found. It allows addressing the cases the original Enhancement Layer uses a different color gamut than the Base Layer. This can be useful for instance in case of deployment of UHD services compatible with legacy HD devices: HD is using the Rec.709 [2], while UHD is likely to use some of the parameters defined in the Rec.2020 [3].

The general diagram of a scalable video encoder including a prediction tool for color differences between the base layer (BL) and enhancement layer (EL) has been proposed in [4][5] and is shown in Figure 1.

Figure 1: Color Space Scalable Encoder (courtesy from Sharp).

Basically, the role of the Color Difference Predictor module is to predict the EL color samples from the collocated BL color samples. However, for a given pair of BL and EL video sequence, the determination of this color transfer function is not straightforward because the content creation workflows may include deterministic processing (Color Space 1 vs Color Space 2 conversion) but also non-deterministic operations, for the reasons explained below:

- The last digital cameras used in Digital Cinema (DC) allow to capture video signal with wide color and luminance range. The output raw data are Wide Color Gamut (WCG) that may be beyond DCI-P3-gamut, with potentially extended dynamic range (more than 14 f-stops). The floating point or 16 bits raw data  represent much more information than what will be distributed and displayed finally.

This trends will probably increase in the coming years, the new captors being able to capture several exposure directly for instance.

- Bit-depth scaling / tone mapping: the choice of the luminance range mapping versus the Reference Output Display characteristics (ODT) (ex: 16 bits vs 8 or 10 bits) is made by human operator depending on artistic intents.

- Color grading (artistic intent): the color balancing is of key importance in content creation, because it traduces the artistic intent of the film director and has major impact on the final rendering. It is performed by high-skilled graphic operators (colorists), using a reference display. Then, if two targeted displays with different characteristics are used (e.g. DCI projectors for DC and Rec.709 TV for HDTV), the artistic intent may be different and the color grading may be different too. These graphic designers use special color authoring tools using 3D LUTs to represent/output their color processing operations.

- Color Space conversion: currently and pragmatically Rec.709-Rec.2020 has been identified as a probable color space conversion use case (e.g. scalable HDTV and UHDTV). This conversion is basically not linear. Besides, new video signal definitions may appear in the coming decade, fueled by the increasing display technology capabilities (e.g. OLED displays…). Some applications should require to adapt content to the end-device rendering characteristics/capabilities consequently.

Figure 2: Hypothetical scalable HD/UHD processing workflow, inspired from (simplified)  Digital Cinema workflow.

An hypothetical scalable HD/UHD processing workflow is depicted in Figure 2. A more precise definition of a digital motion picture workflow is proposed by the Academy Color Encoding System (ACES). It may be used to create content for movie theater or for physical media distribution such as DVD or Blu-ray disc typically.

Consequently, the “Color Difference Predictor” function model is highly unpredictable and may have very various shape. This justifies to describe it based on a generic and flexible model.

# Proposed Color Difference predictor

In order to be able to address a wide range of Color Gamut Scalability applications, without any a-priori on the “Color Difference Predictor” model, we propose to use a 3D Look-Up Table (LUT) to convert/predict the tri-chromatic samples from the color space 1 (BL) to the color space 2 (EL). This 3D LUT approach is the one chosen in post-production facilities to process content from raw to targeted device color spaces. The principle of the 3D LUT is depicted in Figure 3: the 3D LUT can be considered as a sub-sampling of the 3D color space 1, where each vertex is associated with a color triplet corresponding to the color space 2 (predicted) values. For a given BL color sample (color space 1), the computation of its prediction in (EL) color space 2 is made using tri-linear interpolation as depicted in Figure 4 and equation (1).

Figure 3: Principle of the 3D Color Look-Up Table (LUT).

Figure 4: Tri-linear interpolation with 3D LUT (courtesy of Nvidia).

(1)

Where:

S0(y) = y1-y and S1(y) = y-y0

y0 is the index of the nearest sub-sampled vertex inferior to y,

y1 is the index of the nearest sub-sampled vertex superior to y.

In order to encode the 3D LUT data efficiently, each color component of a vertex is encoded with previously encoded color components of neighboring vertices. We propose also to use an octree based description of the 3D LUT in such a way the unused (or less used) 3D color space regions are encoded with coarsely lattice size as depicted in Figure 5.

At the decoder side, all the not coded vertices inside one octant are interpolated to reconstruct the full definition 3D LUT.

Figure 5: Octree based 3D LUT: each octant is encoded with 8 vertices at most.

This approach has several advantages:

- Many Color processing tools uses 3D LUTs to represent and save their intermediate and final color grading operations. In these cases, the 3D LUT information can be made available to the encoder easily.

- We propose that the size of the 3D LUT (number of vertices in one direction), is a parameter read in the bit-stream. In that way, the encoder may choose the best trade-off between “Color Difference Predictor” module accuracy and encoding cost.

- The 3D LUT using tri-linear interpolation has the following property: if its size is 1, ie one single octant with 8 vertices, and if the color components of the vertices are set appropriately (see Error: Reference source not found-Annex), the 3D LUT is equivalent to the Gain-Offset model previously proposed in [4].

- At last, 3D color LUT interpolation module for color conversion is implemented in many STBs and display devices (graphics card).

# Results

We implemented the 3D LUT Color Difference Prediction model (SHM2.0-CGS) in the SHM2.0. The LUTs are loaded by the encoder at the initialization stage and encoded in PPS as described above. We used the common SHM test conditions and software reference configuration parameters [6] and the Color graded sequences (class B) provided in AHG14 (Figure 6).

Figure 6: The 4 color graded sequences (BT.709) used for the tests in AHG14.

## Use of Color graded sequences in 8 bits

In Table 1 and Table 2 we present the results obtained with AI and RA configurations respectively, using the 8-bit color graded sequences:

Overall (BL+EL): Average BD-rate of with BL + EL streams (SHM2.0-CGS) vs BL + EL streams (SHM2.0).

Overall (EL): Average BD-rate of with EL stream (SHM2.0-CGS) vs EL stream (SHM2.0).

Overall (Test EL+BL vs simulcast): Average BD-rate of with BL + EL streams (SHM2.0-CGS) vs two single streams (EL and BL) (SHM2.0-CGS).

Anchor (EL+BL vs simulcast): Average BD-rate of with BL + EL streams (SHM2.0) vs two single streams (EL and BL) (SHM2.0).

The results show the BD-rate gains of SHM2.0-CGS improve the simulcast scenario (SHM2.0) by 11.9% to 30% for luma in average. The BD-rate gains of SHVC BL+EL (SHM2.0-CGS vs SHM2.0) is between 5.5% to 6.8% for luma in average (between 6.2% to 16.7% for EL).

The encoding complexity is decreased up to 3% while the decoding time is increased between 6% to 58% depending on the configurations. Note the time increase is due to the tri-linear interpolation of the 3D color LUT while many STBs and display devices implement this functionality in their graphics processing unit.

Table 1: BD rate performance gain of SHM2.0-CGS vs SHM2.0 (AI configurations, 8 bits).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class B | Y | U | V | Y | U | V |
| BL+EL | -5.5% | -3.9% | -5.7% | -5.8% | -4.1% | -6.4% |
| EL | -9.4% | -7.7% | -9.5% | -16.7% | -15.1% | -17.8% |
| Test EL+BL vs simulcast | -20.0% | -18.8% | -20.8% | -30.0% | -29.7% | -32.0% |
| Anchor EL+BL vs simulcast | -15.4% | -15.3% | -16.0% | -25.6% | -26.3% | -27.3% |
| Enc Time[%] | 97.8% |  |  | 97.3% |  |  |
| Dec Time[%] | 106.7% |  |  | 110.8% |  |  |

Table 2: BD rate performance gain of SHM2.0-CGS vs SHM2.0 (RA configurations, 8 bits).

## Use of Color graded sequences in 10 bits

We modified the SHM2.0 in order to allow testing SHVC with 10-bit sequences (see macro FIX_UPSAMPLING_10BITS in the provided code).

In Table 3 and Table 4 we present the results obtained with AI and RA configurations respectively, using the 10-bit color graded sequences:

Overall (BL+EL): Average BD-rate of with BL + EL streams (SHM2.0-CGS) vs BL + EL streams (SHM2.0).

Overall (EL): Average BD-rate of with EL stream (SHM2.0-CGS) vs EL stream (SHM2.0).

Overall (Test EL+BL vs simulcast): Average BD-rate of with BL + EL streams (SHM2.0-CGS) vs two single streams (EL and BL) (SHM2.0-CGS).

Anchor (EL+BL vs simulcast): Average BD-rate of with BL + EL streams (SHM2.0) vs two single streams (EL and BL) (SHM2.0).

The results show the BD-rate gains of SHM2.0-CGS improve the simulcast scenario (SHM2.0) by 12.7% to 31.8% for luma in average. The BD-rate gains of SHVC BL+EL (SHM2.0-CGS vs SHM2.0) is between 4.4% to 8.4% for luma in average (between 7.3% to 20.5% for EL).

The encoding complexity is decreased up to 3% while the decoding time is increased between 2% to 53% depending on the configurations. Note the time increase is due to the tri-linear interpolation of the 3D color LUT while many STBs and display devices implement this functionality in their graphics processing unit.

Table 3: BD rate performance gain of SHM2.0-CGS vs SHM2.0 (AI configurations, 10 bits).

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class B | Y | U | V | Y | U | V |
| BL+EL | -6.6% | -4.3% | -6.9% | -8.4% | -6.6% | -9.9% |
| EL | -11.2% | -8.9% | -11.4% | -20.5% | -18.4% | -21.8% |
| Test EL+BL vs simulcast | -21.4% | -19.5% | -21.9% | -31.8% | -30.8% | -33.5% |
| Anchor EL+BL vs simulcast | -15.9% | -15.7% | -16.1% | -25.52% | -25.65% | -26.27% |
| Enc Time[%] | 97.6% |  |  | 97.6% |  |  |
| Dec Time[%] | 102.5% |  |  | 108.2% |  |  |

Table 4: BD rate performance gain of SHM2.0-CGS vs SHM2.0 (RA configurations, 10 bits).

# Proposed syntax

It is proposed to encode the 3D Color LUT data into the PPS or the VPS.

As in [4], it is proposed to signal the Color Gamut prediction mode by adding an additional syntax elements to PPS when pps_extension_flag is set.  The syntax element, use_color_prediction, is used to indicate the use of color prediction in the decoding of coded pictures as shown in Table 5.

Table 5: signaling of the prediction parameters.

pps_extension_flag

if( pps_extension_flag )

use_color_prediction

if(use_color_prediction)

3D_ LUT_ color_data ( )

rbsp_trailing_bits( )

If the use_color_prediction flag is equal to ‘1’, the 3D_ LUT_ color_data function is called to signal 3D color LUT data as shown inTable 6.

Table 6: coding of the 3D LUT color data.

| 3D_ LUT_ color_data ( ) { | Descriptor |
| --- | --- |
| nbp_code | u(3) |
| coding_octant(0, 0, 0, 0) |  |
| } |  |

nbp_code indicates the 3D LUT size as listed in Table 7 for the given value of nbp_code.

Table 7: Interpretation of nbp_code.

| nbp_code | 3D LUT size |
| --- | --- |
| 0 | 2 |
| 1 | 3 |
| 2 | 5 |
| 3 | 9 |
| 4 | 17 |
| 5 | 33 |

The decoding of the octant( layer, y,u,v) is a recursive function. Each octant is composed of 8 vertices associated with a flag (encoded_vertex_flag[i]) indicating whether the residual components values are encoded or all inferred to be zero. The component values are reconstructed by adding the residuals to the prediction of the components values. The prediction of the components values is computed using tri-linear interpolation of the 8 neighboring vertices of layer-1.

Table 8: syntax elements for coding_octant().

| coding_octant ( layer, y,u,v) { | Descriptor |
| --- | --- |
| for( i = 0; i < 8 ; i++ ) { |  |
| encoded_vertex_flag[i] | u(1) |
| if ( encoded_vertex_flag[i] ) { |  |
| resY[j] | ue(v) |
| resU[j] | ue(v) |
| resV[j] | ue(v) |
| } |  |
| } |  |
| split_octant_flag | u(1) |
| if ( split_octant_flag ) { |  |
| for( i = 0; i < 8 ; i++ ) { |  |
| coding_octant ( layer+1, y+dy[i],u+du[i],v+dv[i]) |  |
| } |  |
| } |  |
| } |  |

Where (y+dy[i]), (u+du[i]) and (v+dv[i]) are the 8 children octants coordinates (coordinates of the first 3D color vertex) of the current octant (having (y,u,v) as first vertex coordinates).

## 3D color Lut decoding

The ouput of the 3D LUT decoding is a 3 dimension array lut of size nbp x nbp x nbp. Each lut array element is here below called a vertex and is associated with 3 reconstructed sample values (recSamplesY= lutY[i][j][k], recSamplesU=lutU[i][j][k], recSamplesV=lutV[i][j][k]) of bit depth equal to (lut_bit_depth_minus8+8). A vertex (i,j,k) is said to belonging to layer layer_id if the values of i%(nbp>>layer_id), j%(nbp>>layer_id), k%(nbp>>layer_id) are all equal to zero. One vertex may belong to several layers. An octant of layer layer_id is composed of 8 neighboring vertices belonging to layer_id (Error: Reference source not found).

Figure 7: Relative positions of the 8 vertices in one octant of layer_id=layer (lut_bit_depth_minus8=0).

The decoding of the octant( layer_id, y,u,v) is a recursive function: split_octant_flag specifies whether an octant is split into octants with half horizontal and vertical size. The values (y,u,v) specify the location of the first vertex (Error: Reference source not found) in the 3D LUT.

Each octant is composed of 8 vertices (i=0,…7) associated with a flag (encoded_vertex_flag[i]) indicating whether the residual components values (resY[i],resU[i], resV[i]) are encoded or all inferred to be zero. The component values are reconstructed by adding the residuals to the prediction of the components values. The prediction of the components values is computed using tri-linear interpolation of the 8 neighboring vertices of layer_id-1, except the first layer (layer_id=0) using prediction values shown in Table 9. Once reconstructed a vertex is marked as reconstructed.

Table 9: prediction values used for the 3 components of the 8 vertices belonging to the first layer (layer_id=0).

| i | predSamplesY[i] | predSamplesU[i] | predSamplesV[i] |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 255 | 0 | 255 |
| 2 | 0 | 255 | 0 |
| 3 | 0 | 255 | 255 |
| 4 | 255 | 0 | 0 |
| 5 | 255 | 0 | 255 |
| 6 | 255 | 255 | 0 |
| 7 | 255 | 255 | 255 |

If (y+dy[i]), (u+du[i]) and (v+dv[i]) are the 8 children octants coordinates (coordinates of the first 3D color vertex) of the current octant (having (y,u,v) as first vertex (i=0) coordinates), the values dy[i],du[i] and dv[i] for a given layer are depicted in Error: Reference source not found:

Table 10: values dy[i],du[i] and dv[i] in function of index I, for vertices belonging to layer = layer_id.

| i | dy[i] | du[i] | dv[i] |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | nbp >> layer_id |
| 2 | 0 | nbp >> layer_id | 0 |
| 3 | 0 | nbp >> layer_id | nbp >> layer_id |
| 4 | nbp >> layer_id | 0 | 0 |
| 5 | nbp >> layer_id | 0 | nbp >> layer_id |
| 6 | nbp >> layer_id | nbp >> layer_id | 0 |
| 7 | nbp >> layer_id | nbp >> layer_id | nbp >> layer_id |

The reconstructed 3D color LUT samples (recY=recSamplesY[i], recU=recSamplesU[i], recV=recSamplesV[i]) for the vertice ( (y+dy[i]), (u+du[i]), (v+dv[i]) ) belonging to an octant of the layer=layer_id is given by:

recSamplesY[i] = resY[i] + predSamplesY[i]				(1)

where the value of (predSampleY[i], predSampleU[i], predSampleV[i]) is derived using tri-linear interpolation interpol(recY, recU, recV, LUTY, LUTU, LUTV) with the vertices of the octant of layer = layer_id-1 that contains the current octant (eq. (2),(3),(4)):

predSamplesY[i] = ( A0,0,0 + A0,0,1 + A0,1,0 + A0,1,1 + A1,0,0 + A1,0,1 + A1,1,0 + A1,1,1 + (1<<(shift3Y-1)) )  >>  shift3Y    (2)

predSamplesU[i] = ( B0,0,0 + B0,0,1 + B0,1,0 + B0,1,1 + B1,0,0 + B1,0,1 + B1,1,0 + B1,1,1 + (1<<(shift3C-1)) )  >>  shift3C      (3)

predSamplesV[i] = ( C0,0,0 + C0,0,1 + C0,1,0 + C0,1,1 + C1,0,0 + C1,0,1 + C1,1,0 + C1,1,1 + (1<<(shift3C-1)) )  >>  shift3C      (4)

where the values of (Al,m,n , Bl,m,n , Cl,m,n )l=0,1 m=0,1 n=0,1 and shift3Y, shift3C are derived as follows:

Al,m,n = wl,m,n * LUTY[yi+l][ui+m][vi+n]

Bl,m,n = wl,m,n * LUTU[yi+l][ui+m][vi+n]

Cl,m,n = wl,m,n * LUTV[yi+l][ui+m][vi+n]

wl,m,n = sl(recY)* sm(recU)* sn(recV)

shift1Y = (8 - nbp_code + BitDepthY-8)

shift1C = (8 - nbp_code + BitDepthC-8)

shift3Y = shift1Y + 2* shift1C – (BitDepthY – 8)

shift3C = shift1C + 2* shift1C – (BitDepthC – 8)

yi = Floor( recY >> shift1Y)

s0(recY) = recY – ( yi << shift1Y )

sl(recY) = yi+1- recY

ui = Floor(recU >> shift1C )

s0(recU) = recU – (ui << shift1C)

sl(recU) = ui+1- recU

vi = Floor(recV >> shift1C )

s0(recV) = recV – (ui << shift1C )

sl(recV) = ui+1- recV

## Inter-layer sample prediction process

The inter-layer color prediction process is invoked when use_color_prediction is equal to 1, before the up-sampling process. It uses same interpolation function interpol() as described in the previous section.

Inputs to this process are:

- The reconstructed samples (recY, recU, recV) of the layer_id used as prediction,

- The 3D color LUTs: LUTY, LUTU, LUTV

The color predicted samples (predY, predU, predV) are derived as follows (eq. (2),(3),(4)):

predY = ( A0,0,0 + A0,0,1 + A0,1,0 + A0,1,1 + A1,0,0 + A1,0,1 + A1,1,0 + A1,1,1 + (1<<(shift3Y-1)) )  >>  shift3Y    (2)

predU = ( B0,0,0 + B0,0,1 + B0,1,0 + B0,1,1 + B1,0,0 + B1,0,1 + B1,1,0 + B1,1,1 + (1<<(shift3C-1)) )  >>  shift3C      (3)

predV = ( C0,0,0 + C0,0,1 + C0,1,0 + C0,1,1 + C1,0,0 + C1,0,1 + C1,1,0 + C1,1,1 + (1<<(shift3C-1)) )  >>  shift3C      (4)

where the values of (Al,m,n , Bl,m,n , Cl,m,n )l=0,1 m=0,1 n=0,1 and shift3Y, shift3C are derived identically as described in the previous section.

# Conclusion

This contribution reports new results in the area of color gamut scalable video coding, using prediction model based on 3D color Look-Up Tables (LUT). The experiments have been conducted using the 4 Color graded sequences proposed in AHG-14, in 8-bits and in 10-bits, between Base layer using Rec.709 color space and Enhancement layer using DCI-P3 color space.

The results in 8-bits show the BD-rate gains of SHM2.0-CGS improve the simulcast scenario (SHM2.0) by 4.8% to 32% for luma in average. The BD-rate gains of SHVC (SHM2.0-CGS vs SHM2.0) is between 5.5% to 6.8% for luma in average (between 6.2% to 16.7% for EL).

We got similar gains in 10-bits.

It is asserted the use of 3D color Look-Up Tables (LUT) allows to cover the Color Gamut Scalability MPEG requirements without any a-priori on the color difference prediction model.

We propose to include this color prediction tool in the scalable HEVC design.

# References

- Ajay Luthra, Jens-Rainer Ohm, Jörn Ostermann, “ Requirements of the scalable enhancement of HEVC,” WG11 Requirements and Video, ISO/IEC JTC1/SC29/WG11 N12783, May 2012, Geneva, Switzerland.

- ITU-R Recommendation BT.709 “Parameter values for the HDTV standards for production and international programme exchange” Dec. 2010.

- ITU-R Recommendation BT.2020  “Parameter values for UHDTV systems for production and international programme exchange” April 2012.

- L.Kerofsky, A.Segall, SH.Kim, K.Misra, “Color Gamut Scalable Video Coding: New Results,” JCTVC-L0334, 12th JCTVC Meeting, Geneva, CH, 14–23 Jan. 2013.

- P.Bordes, P.Andrivon, R. Zakizadeh, “AHG14: Color Gamut Scalable Video Coding using 3D LUT,” JCTVC-M0197, 13th JCTVC Meeting, Incheon, KR, 18–26 Apr. 2013.

- X.Li, J.Boyce, P.Onno, Y.Ye, “Common SHM test conditions and software reference configurations,” JCTVC-M1009, 13th Meeting: Incheon, KR, 18–26 Apr. 2013.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).